# SECTION 229 – ALUMINUM ALLOY

## 229.01 – Description

These specifications cover aluminum alloy products designed in shapes and compositions to serve a specific purpose, such as a sign panel, post, or conduit, including necessary fasteners.

## 229.02 – Detail Requirements

- Sheets and plates shall conform to ASTM B209, alloy 6061 T6, 6061 T651, 5052-H32, 5052-H34, 3003-H14, or 5086-H116/H32. Aluminum sign panels shall be alloy 5052-H32, 5052-H34, 5052-H38, or 6061-T6.

- Bars, rods, and wire shall conform to ASTM B211, alloy 6061-T6 or 6061-T651.

- Extruded bars, rods, shapes, and tubes shall conform to ASTM-B221, alloy 6061-T6 or 6063-T6.

Aluminum alloy extrusions, extruded tubes, drawn tubes, or pipes that are to be bent on a radius of less than 3 feet may be made from alloy having a temper condition of 0.

- Drawn tubes shall conform to ASTM B210, alloy 6061-T6.

- Pipe shall conform to ASTM B429 or ASTM B241, alloy 6061-T6 or 6063-T6.

- Bolts, studs, nuts, set screws, washers, and rivets shall be furnished as commercial items suitable for the application.

- Permanent-mold castings for items other than rail posts shall conform to ASTM B108, alloy 356.0-T6. Cast aluminum alloy rail post shall conform to ASTM B108, alloy A444.0.

- Sand castings shall conform to ASTM B26, alloys 319-F, 319.0-T6, 356.0-F, 356.0-T6, or 535.0-F.

- Shims shall be made from a sheet or plate conforming to ASTM B209, alloy 1100-O.

- Aluminum filler metal for welding shall conform to AWS 1.2.

- Rolled or extruded structural shapes shall conform to ASTM B308, alloy 6061-T6.

- Breakaway support couplings for light poles and sign posts shall conform to ASTM B209 or ASTM B221, alloy 6061-T6.

- Frangible bases for light and signal poles shall conform to ASTM B26 or ASTM B108, alloy 319 or 356.0-T6.

- Aluminum alloy for controller, control center, and flasher cabinets shall conform to ASTM B209, alloy 5052-H32.

- Aluminum alloy for lighting and pedestal poles shall conform to ASTM B221, ASTM B241, or B429, alloy 6063-T6.