# SECTION 423 – TOOTH EXPANSION JOINT ASSEMBLY

## 423.01 – Description

This work shall consist of furnishing and installing tooth expansion joint assemblies, including associated neoprene troughs and connections to outlet devices, in accordance with these specifications and in conformity with the lines, grades and locations shown on the plans or established by the Engineer.

## 423.02 – Materials

Materials shall conform to Sections 212 and 226.

Neoprene troughs shall be made of continuous material. Field splices shall not be permitted unless approved by the Engineer.

Structural steel shall be hot-dipped galvanized in accordance with Section 233.

## 423.03 – Procedures

Working drawings showing complete details and dimensions of the tooth expansion joint and other pertinent information shall be submitted to the Engineer for review in accordance with Section 105.10.

Where indicated on the plans, the Contractor shall remove and replace portions of the existing deck and backwall to facilitate anchorage and installation. Such removal and replacement shall be performed and measured and paid for in accordance with Section 412.

Fabrication shall conform to Sections 407 and 212. Tooth thickness shall be specified on the plans.

All dimensions of the tooth expansion joint assemblies shall be checked after they are galvanized and before they are delivered to the field. The Contractor shall be responsible for misfits and errors and shall make necessary corrections or replacements at the Contractor’s expense.

Tooth expansion joint assemblies shall seal the deck surface, gutters, curbs, sidewalks, parapets, and raised medians to prevent water and other contaminants from seeping onto the superstructure and substructure members and shall include provision for connection to drainage outlets to prevent leakage of collected rainwater at any point along the joint or outlet.

Final sealing of the finished joint shall be completed as soon as practicable after installation.

Surfaces exposed to roadway traffic shall have antiskid provisions as indicated on the plans.

## 423.04 – Measurement and Payment

Tooth expansion joint assembly will be measured in linear feet measured along the pavement surface from out to out of the deck slab for the tooth thickness specified and will be paid for at the contract unit price per foot. This price shall include furnishing and installing tooth expansion joint assembly, all necessary components and anchoring devices.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Tooth Expansion Joint Assembly (Tooth thickness) | Linear foot |