# SECTION 510 – RELOCATING OR MODIFYING EXISTING MISCELLANEOUS ITEMS

## 510.01 – Description

This work shall consist of removing, salvaging, resetting, relaying, adjusting, installing, modifying, reconstructing, or relocating existing items or items furnished by the Department or others, including, but not limited to, right-of-way monuments, guardrail, riprap, drainage structures, traffic control devices, water or sanitary sewer facilities, or other items designated on the Plans or elsewhere in the Contract.

## 510.02 – Materials

The principal materials to be used in this work shall be salvaged items or materials from current or previous construction.

Items shall be constructed, adjusted, modified, or reconstructed with the same type of material as was used in the original construction.

The suitability of existing material for salvage, modification, or reuse will be as designated in the Contract or determined by the Engineer.

New, salvaged, or refurbished materials necessary for resetting, relaying, adjusting, modifying, or relocating the item specified shall conform to the applicable specifications for items of the same character and type. Salvaged or refurbished materials shall be in good working and useable condition. Preparation for use shall include cleaning, repainting, and refinishing to the approximate original condition.

## 510.03 – Procedures

Materials designated for salvage shall be carefully removed, dismantled, cleaned, and stockpiled in areas where they will not be damaged or shall be delivered to a storage area designated in the contract or by the Engineer. Material that is not designated for salvage shall be disposed of in an approved disposal area. Items designated for relocation, relaying, adjustment, modification, or installation shall be installed in accordance with the applicable specifications or as directed by the Engineer.

## 510.04 – Measurement and Payment

Removing, resetting, relaying, adjusting, installing, modifying, reconstructing, or relocating designated items will be measured by the unit specified in the Contract in accordance with the plans and the applicable sections of these specifications and will be paid for at the contract unit price for the specified item and action designated. This price shall include salvaging by dismantling, loading, transporting, and unloading furnished materials; cleaning, repainting, and refinishing or refurbishing salvaged items; removing, resetting, relaying, adjusting, installing, modifying, reconstructing, or relocating designated items; salvaging or disposing of surplus and unsuitable material; excavating; trenching; backfilling; preparing foundation; reconnecting components for electrical and electronic items; revising wiring diagrams or schematics; and restoring disturbed areas.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Remove, reset, relay, adjust, install, modify, reconstruct, relocate existing (Item or standard) | Each, Linear foot, Square yard, Cubic yard, or Lump sum |