# SECTION 607 – HERBICIDE SPRAYING

## 607.01 – Description

This work shall consist of applying an approved herbicide for the control of weeds in turfed areas as indicated on the Plans or as designated by the Engineer.

## 607.02 – Materials

Herbicide shall conform to Section 244.02.

## 607.03 – Procedures

Herbicide shall be applied in accordance with the manufacturer’s recommendations by a Virginia Department of Agriculture and Consumer Services (VDACS) certified applicator or registered technician. Herbicide shall not be applied when the ambient temperature is above 85°F or below 60°F. The spray pressure shall be at least 20 but not more than 30 pounds per square inch. Spraying shall not be performed when the vegetation is wet, when it appears that rain is imminent within 6 hours, or when the wind is blowing enough to scatter paper or trash or when drifting of spray onto non designated areas seems likely.

## 607.04 – Measurement and Payment

Herbicide spraying will be measured in units of 1,000 gallons of mixture and will be paid for at the contract price per unit. This price shall include furnishing and applying herbicide.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Herbicide spraying | Unit (1,000 gallons) |