# SECTION 704 – PAVEMENT MARKINGS AND MARKERS

## 704.01 – Description

This work shall consist of establishing the location of retroreflective pavement markings and installing pavement markings and pavement markers in accordance with the MUTCD, the Contract, and as directed by the Engineer.

## 704.02 – Materials

- Pavement Markings shall conform to Section 246.

- Glass Beads and retroreflective optics materials shall conform to Section 234.

- Pavement Markers shall conform to Section 235.

- Contrast Pavement Markings shall conform to Section 246.

All pavement marking materials shall be selected from the Materials Division’s Approved Products List 17. For Type B, Class VI pavement marking materials that are to be applied to latex emulsion or slurry seal surfaces. The selected Type B, Class VI manufacturer shall be a manufacturer that approves and warranties their product for application on that type of surface.

The Contractor shall use a Department approved inventory tracking system for all materials received from the manufacturer. Shipment of materials from such inventory shall be accompanied by a signed Form C-85 containing the following certification statement:

Material shipped under this certification has been tested and approved by VDOT as indicated by laboratory test numbers (MS#) listed hereon.

## 704.03 – Procedures

Once received by the Contractor, the Contractor shall store all materials in accordance with the manufacturer’s instructions until the day of installation, unless the Engineer otherwise authorizes. Pavement marking material shall not be installed if the material has exceeded its shelf life, has been improperly stored, has deteriorated or is otherwise damaged. The Contractor shall furnish a copy of the manufacturer’s installation instructions to the Engineer prior to installation.

The Contractor shall have a certified Pavement Marking Technician present during all temporary pavement marking, permanent pavement marking, and pavement marker operations.

All pavement markings and markers shall be installed on new roadways prior to opening the roadways to traffic. Temporary or permanent pavement marking installation, including edge lines and message markings, shall be completed within the time limits stated herein on existing roadways where the pavement markings have been removed or obscured and the roadway is to be reopened to traffic, unless otherwise directed by the Engineer. The Engineer will only authorize exceptions to these time limits for weather conditions preventing installation, when pavement surface conditions required by the manufacturer or these specifications such as cooling, curing, or aging limit application prior to the installation of permanent markings, or when Type B, Class VI marking material is to be inlaid into the freshly paved asphalt concrete roadway surface. Epoxy resin and polyurea pavement markings shall be installed on new pavement in accordance with the manufacturers’ instructions. Pavement markings shall not be installed on new asphalt concrete pavement until the pavement mat has cooled sufficiently to support the pavement marking equipment without deformation. Time limits for installation are as follow:

- Pavement marking installation on interstates, other freeways (fully limited-access, divided roadway with two or more lanes per direction) with posted speed limit of 55 mph or greater, and interstate/freeway ramps, where the corresponding pavement markings were removed, eradicated, or obscured shall be completed prior to opening the lane to traffic.

- Pavement marking installation on non-freeway roads having traffic volumes of 10,000 ADT or more shall be completed within 24 hours after the end of the workday when the corresponding existing pavement markings were removed, eradicated, or obscured.

- Pavement marking installation on non-freeway roads having traffic volumes between 3,000 and 10,000 ADT shall be completed within 48 hours after the end of the workday when the corresponding existing pavement markings were removed, eradicated, or obscured.

- Pavement marking installation on non-freeway roads having traffic volumes of less than 3,000 ADT shall be completed within 72 hours after the end of the workday when the corresponding existing pavement markings were removed or obscured.

If an approach to a signalized intersection has (a) two or more approach through lanes, (b) 45 mph or greater speed limit, (c) greater than 3000 ADT, and (d) all markings on that approach are obliterated, then all lane lines and centerlines within 250 feet of the stop line location shall be temporarily or permanently marked within 24 hours of opening the approach to traffic, unless a time extension is approved by the Engineer and “Unmarked Pavement Ahead” or “No Center Line” warning signs were properly installed when the unmarked approach was first opened to traffic as per the VWAPM.

- Temporary symbol/message markings and temporary edge lines, if required by the Contract, shall be marked within 72 hours after the end of the work day when the corresponding existing markings were obscured, removed, or eradicated on roads with 10,000 or greater ADT, and shall be marked within 96 hours after the end of the work day when the corresponding existing markings were obscured, removed, or eradicated on roads with less than 10,000 ADT.

If the Contractor cannot have permanent pavement markings installed within the time limits specified, the Contractor shall install temporary pavement markings within the same time limits and maintain such markings until the permanent pavement markings can be installed. The cost of installing, maintaining, and removing/eradicating temporary pavement markings shall be at no additional cost to the Department unless otherwise indicated in the Contract or directed by the Engineer.

The Contractor may mark the locations of proposed permanent markings on the roadway by installing premarking materials. Premarkings may be accomplished by installing Type D (removable, any class) tape, chalk, or lumber crayons, except pavement markings such as stop lines, crosswalks, messages, hatching, etc., shall be premarked using chalk or lumber crayons. Premarkings shall be of the same general color as the pavement markings being premarked.

When tape is used as a premarking material, premarking shall consist of 4- inch by 4-inch-maximum squares or 4-inch-maximum diameter circles spaced at 100-foot minimum intervals in tangent sections and 50-foot minimum intervals in curved sections. At locations where the pavement marking will switch colors, e.g., gore marking, the ends of the markings may be premarked regardless of the spacing.

When the Contractor uses chalk or lumber crayon as a premarking, the entire length of the proposed pavement marking may be premarked.

Premarkings shall be installed so their installation will not affect the adhesion of the permanent pavement markings. When Type D tape is used as the premarking material and the lateral location of such premarkings to location of the final pavement markings exceeds 6 inches, the Type D premarkings shall be removed at no additional cost to the Department.

Permanent pavement markings shall not be installed over temporary paint if the paint exceeds the maximum allowable specified thickness in Table VII-3 or is not fully dry. If the temporary paint is not located directly underneath the location where the permanent markings are to be installed, it shall be 100% eradicated prior to installation of the permanent markings at no additional cost to the Department.

The Contractor shall exercise caution and protect the public from damage while performing pavement marking operations. The Contractor shall be responsible for the complete preparation of the pavement surface, including, but not limited to, removing dust, dirt, loose particles, oily residues, curing compounds, concrete laitance, residues from eradication, and other foreign matter immediately prior to installing pavement markings. The pavement surface shall be clean and dry at the time of pavement marking installation and shall be tested in accordance with VTM-94 prior to permanent installation. The Contractor shall provide the apparatus indicated in VTM-94 that are needed to perform the moisture test prior to application.

Liquid markings shall be applied so as to prevent splattering and overspray and shall be protected from traffic until track free by the use of traffic control guarding or warning devices as necessary. If a vehicle crosses a pavement marking and tracks it or if splattering or overspray occurs, the affected marking and resultant tracking, overspray, or splattering shall be completely removed and new markings applied at the Contractor’s expense.

Truck-mounted equipment for application of liquid long line Type B markings shall be capable of hot applying liquid and/or plastic markings and broadcasting glass beads uniformly over the entire surface of the marking. Truck mounted equipment tanks shall be equipped with a mechanical agitator to keep the pavement marking materials thoroughly mixed at all times. Materials shall be blended, heated, and applied in accordance with the manufacturer’s installation instructions. Markings shall be applied in widths of 4 through 8 inches in accordance with the the Plans and Specifications. Equipment shall be capable of applying two pavement lines, either solid or skip, at the same time when double line markings are required.

Non-truck mounted equipment shall be self-propelled and regulated to allow for calibration of the amount and type of material applied.

The Contractor shall be responsible for ensuring that equipment is thoroughly cleaned between changes in colors or types of materials.

Markings shall not be installed directly over longitudinal pavement joints, except to cross the joint perpendicularly or at an angle.

Pavement markings shall have clean and well-defined edges without running, bleeding or deformation. Markings shall be uniform in appearance, free of waviness; (waviness is defined as the edge of the marking shall not vary from a straight line more than 1/4 inch in three feet or more than one inch in fifty feet for a maximum distance of 500 feet); shall be straight on tangent alignment; and shall be on a true arc on curved alignment.

The widths of pavement markings shall not deviate more than 1/4 inch on tangent nor more than 1/2 inch on curves from the required width. The length of the gap and the length of the individual stripes that form skip lines shall not deviate more than two inches from their required lengths. The length of the gap and individual skip line shall be of such uniformity throughout the entire length of each that a normal striping machine shall be able to repeat the pattern and superimpose additional striping upon the existing marking.

Glass beads and retroreflective optics shall be applied at the rate specified herein or as specified in the Materials Division’s Approved Products List for the specific pavement marking product. Beads and optics shall be evenly distributed over the entire surface of the marking. The Contractor shall apply beads to the surface of liquid markings with a bead dispenser attached to the applicator that shall uniformly dispense beads simultaneously on and into the just-applied marking. The bead dispenser shall be equipped with a cut-off control synchronized with the applied marking material cut off control so that the beads are applied totally on the marking. Beads shall be applied while the liquid marking is still fluid. Approximately seventy (70) percent of beads shall be buried in the marking, and the remaining 30 percent shall be 50 to 60 percent embedded in the marking’s surface unless otherwise specified by the pavement marking manufacturer. Beads installed on crosswalks and stop lines on roadways with curbs only (no gutter) may be hand applied for two feet at the end of each line next to the curb with 100 percent of the beads embedded 50 to 60 percent into the marking’s surface.

The Engineer will make a visual evaluation of the pavement marking material to assess the condition, retroreflectivity, and color after its installation and again prior to final acceptance. The Department, the Contractor, and the marking manufacturer’s representative will make a further inspection if problem areas are suspected to identify specific areas of concern. If required by the Engineer, the suspect areas shall be tested by the Contractor in the presence of the Engineer in accordance with VTM-125 to define the evaluation sections and the number of measurements needed. Acceptable test results shall meet the retroreflectivity and color requirements specified in Section 246. Markings that do not meet the requirements for retroreflectivity and day and nighttime color specified in Section 246 shall be eradicated and replaced by the Contractor at no cost to the Department. Pavement markings that exhibit signs of significant tearing, deformation, shrinkage, roll back, lifting, or other signs of poor adhesion shall also be replaced by the Contractor at no cost to the Department.

All costs associated with testing the marking material for retroreflectivity, color, and adhesion shall be borne by the Contractor. The Contractor will be paid for maintenance of traffic during this testing at the contract unit price for the maintenance of traffic items used.

Pavement marking manufacturer’s material guarantees shall be obtained by the Contractor and assigned to the Department in writing prior to final acceptance.

- Pavement Line Markings: Pavement markings shall be white or yellow markings (unless another color is specified in the Contract) as required by the MUTCD and plans for the specific location or as specified by the Engineer. Line markings shall be installed in accordance with Table VII-3 unless otherwise recommended by the manufacturer and approved by the Engineer. The Contractor shall furnish a copy of the manufacturer’s installation instructions for the specific marking to the Engineer prior to installation.

The Contractor shall perform quality control testing for application thickness and glass bead rate in accordance with VTM-94 at the beginning of each workday and every 3 hours thereafter. The Contractor shall provide the apparatuses needed to perform the quality control testing in accordance with VTM-94. Compliance testing using VTM-94 shall be performed in the presence of the Engineer and shall be documented on the Pavement Marking, Contractor’s Daily Log and Quality Control Report, Form C-85, immediately after testing is completed. If requested by the Engineer, the Contractor shall provide a quality control (QC) test plate and the provision of the test plate shall be documented on the Form C-85. The Contractor shall also provide a printed or electronic copy of the signed Form C-85 to the Materials Division Quality Assurance Technician for materials notebook evaluation.

The Contractor shall maintain a daily log, Form C-85, for both temporary and permanent pavement markings and markers. The C-85 form shall not be modified; all log entries shall be made in ink; and shall be legible. The log shall be signed by the Contractor and delivered to the Engineer by the end of each workday. If the C-85 is in electronic format, it shall be kept current with VTM-94 testing throughout the day and a printed copy, signed by the Contractor, shall be delivered to the Engineer at the end of each workday.

Pavement line markings shall consist of solid and skip lines, including but not limited to, lane division lines, edgelines, channelizing, outlining and marking safety zones around objects, and forming islands and parking lot stalls.

  - Type A markings shall be applied to asphalt concrete and hydraulic cement concrete pavements in accordance with the manufacturer’s installation instructions. Paint shall not be applied over existing pavement markings of other materials unless the existing marking is 90 percent worn away or eradicated. Paint may be applied over existing painted markings if the existing paint is clean and well adhered.

Paint shall be thoroughly mixed and heated such that it will not track within 60 seconds after its application.

Glass beads and retroreflective optics shall be applied to the entire surface of the marking at the minimum rate of 6 pounds per gallon of paint, unless specified otherwise in the Materials Division’s Approved Products List 20 for the selected pavement marking product.

  - Type B markings shall be applied in accordance with the manufacturers’ installation instructions.

The Contractor shall furnish a properly calibrated infrared instrument to measure the actual temperature of molten thermoplastic material. Multi-component material shall be applied using internally injected guns for the proper mixing of components.

Non–truck mounted equipment for application of thermoplastic material shall be of the screed extrude type with a screw drive.

    - Thermoplastic (Class I) material shall be applied by screed extrusion, ribbon gun, or spray equipment in accordance with the manufacturer’s installation instructions. A primer/adhesive manufactured or recommended by the thermoplastic marking manufacturer shall be applied to hydraulic cement concrete surfaces and to asphalt concrete surfaces in accordance with the manufacturer’s installation instructions.

Alkyd thermoplastic may be applied directly after the paving operations if the paved surface can support the equipment. Hydrocarbon thermoplastic shall not be applied to asphalt surfaces less than 30 days after paving operations are complete, hydrocarbon thermoplastic may be applied to hydraulic cement concrete surfaces as soon as permitted by the manufacturer’s instructions.

Alkyd and hydrocarbon materials shall not be mixed together.

Thermoplastic shall not be applied over existing pavement markings of other materials unless the existing marking is 90 percent percent worn away or eradicated; or over previously applied temporary paint that is fully dry and is at a thickness of 10 mils or less. Thermoplastic shall only be applied over existing thermoplastic markings, if the existing thermoplastic markings are clean, chalk free (not powdery), and well adhered.

Thermoplastic marking material shall be applied at thickness of 90 mils (± 5 mils) above the riding surface, whether dense or open graded surface.

Glass beads and retroreflective optics shall be surface applied at the rate of 7 pounds per 100 square feet unless specified otherwise on the Materials Division’s Approved Products List 43 for the specific thermoplastic product.

    - Preformed thermoplastic (Class II) material shall be installed as lines, stop bars, message markings, legends, and symbols in accordance with the manufacturer’s installation instructions. A primer/sealer manufactured by or recommended by the preformed thermoplastic manufacturer shall be applied to all hydraulic cement concrete surfaces and to asphalt concrete surfaces in accordance with the manufacturer’s installation instructions.

Preformed thermoplastic shall not be applied over existing pavement markings of other materials unless the existing marking is 90 percent worn away or eradicated; or over previously applied temporary paint that is fully dry and is at a thickness of 10 mils or less. Preformed thermoplastic may be applied over existing thermoplastic markings, provided the the existing thermoplastic marking is clean, chalk free (not powdery), and well adhered.

Preformed thermoplastic shall be 125 mils thick (± 5 mils) unless otherwise approved by the Engineer.

Additional glass beads and retroreflective optics shall be evenly applied at a rate of 7 pounds per 100 square feet (unless another rate is specified in the Materials Division’s Approved Products List 73 for the selected pavement marking product) to flood the entire surface immediately after installation while the material is molten.

    - Epoxy resin (Class III) material shall be applied in accordance with the manufacturer’s installation instructions and shall not be applied over existing pavement markings unless the existing marking is 90 percent worn away or eradicated. Epoxy marking material shall be applied at a wet film thickness of 20 mils (± 1 mil).

Glass beads and retroreflective optics shall be applied to the surface of the marking at the rate of 25 pounds per gallon of material, unless otherwise specified in the Materials Division’s Approved Products List 75 for the specific epoxy resin product.

    - Plastic-backed preformed tape (Class IV) shall be installed in accordance with the manufacturer’s installation instructions. Tape may be applied to asphalt concrete and hydraulic cement concrete pavements. Tape may be installed immediately following the final rolling of new asphalt concrete surface provided installation is done is strict conformance with the preformed tape manufacturer’s instructions for this type of application. Tape shall not be applied over existing pavement markings of other materials unless the existing marking is 90 percent worn away or eradicated.

Primer/adhesive shall be used to enhance adhesion in accordance with the manufacturers’ installation instructions, except when tape is inlaid immediately following the final rolling of the new asphalt concrete surface.

Tape for pavement line markings shall be applied by an application cart as recommended by the manufacturer. Tape shall be tamped into place with a tamper cart with a weight as recommended by the manufacturer. Vehicle wheels may be used to tamp line markings if allowed by the manufacturer’s installation instructions. If vehicle wheels are used to tamp the markings, the Contractor shall ensure that the vehicle tires ride true down the length of the tape marking.

    - Patterned preformed tape (Class VI) shall be installed either under the guidance of the manufacturer’s representative or by a manufacturer’s certified technician.

Type B, Class VI markings applied to new plant mix asphalt surfaces shall be installed as per manufacturer’s installation instructions, except that non-embedded (adhesive) surface application will not be permitted; the markings shall be inlaid in the freshly installed asphalt surface before the pavement mat has cooled. The temperature of the asphalt mat shall be between 100 and 180 degrees. The Type B, Class VI markings shall be inlaid with a roller (minimum 2 tons) operating in a non-vibratory mode when the asphalt mat is between 100 and 180 degrees.

The Contractor shall ensure that markings are not degraded by subsequent operations. Markings that are improperly inlaid during the pavement operations shall be completely eradicated and reapplied via non-embedded surface application at the Contractor’s expense.

Surface-applied Type B Class VI markings shall not be installed directly over existing markings, except that Type B Class VI markings may be installed over Type A markings that are fully dry and are at a thickness of 10 mils or less.

    - Polyurea (Class VII) shall be applied in accordance with the manufacturer’s installation instructions. Polyurea marking material shall not be applied over existing pavement markings unless the existing marking is 90 percent worn away or eradicated; or over Type A markings that are fully dry and are at a thickness of 10 mils or less.

Polyurea marking material shall be applied at a wet film thickness of 20 mils (± 1 mil). Glass beads and retroreflective optics shall be applied at the rate specified in the VDOT Materials Division’s Approved Products List 74 for the specific polyurea product.

  - Type D and E temporary pavement markings shall be installed in accordance with the manufacturers’ installation instructions and will be paid for in accordance with Section 512.

- Pavement message and symbols markings shall be the color required by the MUTCD or the plans for the specific location or as specified by the Engineer. The Contractor shall install message and symbols markings in accordance with Table VII-3, unless otherwise recommended in the manufacturers’ installation instructions and approved by the Engineer.

Message and symbol markings include, but shall not be limited to, those detailed in Standard Drawing PM-10.

| TABLE VII-3 Pavement Markings |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Class | Name | Surface Temp. at Time of Application | Film Thickness (mils) | Pavement Surface | Application Limitations | Appr. List No. |
| A |  | Traffic paint | (Note 5) | 15 1 when wet | AC HCC | May be applied directly after paving operations | 20 |
| B | I | Thermoplastic Alkyd | (Note 5) | 90 5 | AC HCC | May be applied directly after paving operations | 43 |
|  | I | Thermoplastic Hydrocarbon | (Note 5) | 90 5 when dry | AC HCC | Do not apply less than 30 days after paving operations | 43 |
|  | II | Preformed Thermoplastic | (Note 5) | 120-130 | AC HCC | Manufacturers installation instructions | 73 |
|  | III | Epoxy resin | (Note 5) | 20 1 when wet | AC HCC | Manufacturers installation instructions | 75 |
|  | IV | Plastic-backed preformed Tape | (Note 5) | 60 - 120 | AC HCC | Manufacturer’s installation instructions | 17 |
|  | VI | Patterned preformed Tape | (Note 5) | 20 min (Note 1) 65 min (Note 2) | AC HCC | (Note 4) | 17 |
|  | VII | Polyurea | (Note 5) | 20 1 | AC HCC | Manufacturer’s installation instructions | 74 |
| D | II | Removable tape | (Note 5) | (Note 3) | AC HCC | Temporary pavement marking | 17 |
|  | III | Wet Reflective Removable tape | (Note 5) | (Note 3) | AC HCC | Temporary pavement marking | 17 |
| E |  | Removable black tape (Non-Reflective) | (Note 5) | (Note 3) | AC | Temporary pavement marking for covering existing markings | 17 |
| Note 1: Thinnest portion of the tape’s cross section. Note 2: Thickest portion of the tape’s cross section. Note 3: In accordance with manufacturer's installation instructions. Note 4: In accordance with the manufacturer’s installation instructions, except that Type B, Class VI markings on new plant mix asphalt surfaces shall be inlaid into the freshly installed asphalt surface and not surface-applied. Note 5: Surface temperature at time of application shall be in accordance with manufacturer’s installation instructions. If the installation instructions do not specify minimum surface temperature, then the markings shall not be installed unless the surface temperature at time of application is 50 degrees F or higher. |  |  |  |  |  |  |  |

Pavement messages and symbols markings shall be installed at locations shown on the plans or at locations designated by the Engineer. The pavement message or symbol shall have clean and well-defined edges without running, bleeding or deformation; shall be uniform in appearance throughout, and free of overlaps.

Type B, Class I hydrocarbon thermoplastic material shall not be used for messages or symbols.

- Eradication: Eradication of existing pavement markings shall be in accordance with Section 512 except only 90 percent removal is required where the new markings will completely cover existing markings.

- Pavement markers: Retroreflectors for pavement markers shall conform to Section 235 and be the same color as the adjacent pavement marking except the backsides of the markers shall be as follows:

- One-way markers: The backside shall be red and the front side white.

- Two-way markers: The backside shall match the adjacent pavement marking.

  - Snow-plowable raised pavement markers shall be installed by cutting two parallel grooves into the pavement at the depth and dimensions recommended by the marker manufacturer. Grooves shall be parallel to the adjacent pavement marking. Grooves shall be cut with saw blades having a diameter to match the curvature of the steel casting bottom and keels. Keel surfaces and the cut grooves shall be free from moisture, scale, dirt, oil, grease, debris, or any other contaminant that might reduce bonding. Snow-plowable raised pavement markers shall not be installed on existing or new bridge decks. When pavement markers are required on concrete bridge decks, raised pavement markers shall be used and bonded to the surface using an adhesive in accordance with the manufacturer’s recommendations.

Snowplowable raised pavement markers shall be installed at least 2 inches from any joint, crack, or seam.

Casting keels shall be bonded in the saw-cut grooves in accordance with the manufacturers’ installation instructions. The bonding material shall be from the Materials Division’s Approved Products List 22 for the specific marker. Front and rear keel tips of the casting shall be installed flush with or below the pavement surface. The installed height of the raised pavement marker shall be approximately 1/4 inch to 1/2 inch above the pavement surface. The ambient air temperature, ambient moisture condition, and pavement surface condition shall be in accordance with the manufacturer’s installation instructions at the time of installation.

The top of reflectors shall be mounted flush with or below the top of the casting.

If the snowplowable raised pavement marker retroreflector becomes soiled or damaged during installation (including dirtying from duct tape or similar adhesive residue), the retroreflector shall be replaced by the Contractor at no additional cost to the Department.

  - Raised pavement markers shall be bonded to the pavement surface in accordance with the manufacturer’s installation instructions. The bonding material shall be from the Materials Division’s Approved Products List 22 for the specific marker.

## 704.04 – Measurement and Payment

Pavement line markings will be measured in linear feet and will be paid for at the contract unit price per linear foot for the type and/or class and width specified. This price shall include surface preparation, premarking, furnishing, installing, quality control tests, daily log, guarding devices, primer/adhesive, glass beads, reflective optics materials when required, and warranty.

Contrast Pavement Line Marking will be measured in linear feet and will be paid for at the Contract unit price per linear foot for the type or class and width specified. This price shall include surface preparation, premarking, furnishing, installing, quality control tests, daily log, guarding devices, primer or adhesive, glass beads, reflective optics materials when required, and warranty.

Pavement message markings will be measured in units of each per location or in linear feet as applicable and will be paid for at the Contract unit price per each or linear foot. This price shall include surface preparation, premarking, furnishing, installing, quality control tests, daily log, guarding devices, primer or adhesive, glass beads, reflective optics materials when required, and warranty.

Pavement symbol markings will be measured in units of each per location for the symbol and type material specified and will be paid for at the Contract unit price per each.  This price shall include surface preparation, premarking, furnishing, installing, quality control tests, daily log, guarding devices, primer or adhesive, glass beads, reflective optics materials when required, and warranty.

Pavement markers will be measured in units of each for the type specified and will be paid for at the contract unit price per each. This price shall include surface preparation, furnishing, installing, prismatic retroreflectors, pavement cutting, adhesive, castings, quality control tests, and daily log.

Eradication of pavement markings will be measured and paid for in accordance with Section 512.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| (Type or class) Pavement line marking (width) | Linear Foot |
| (Type or Class) Contrast Pavement Line Marking (width) | Linear Foot |
| Pavement message marking (Message) | Each or Linear Foot |
| Pavement symbol marking (Symbol, Type or class material) | Each |
| (Type) Pavement marker (type pavement) | Each |