# SECTION 812 – ITS CONDUIT

## 812.01 – Description

This work shall consist of furnishing and installing conduit for ITS systems in accordance with these Specifications and as shown on the Plans or as directed by the Engineer.

## 812.02 – Materials

ITS Conduit shall be Polyethylene (PE) or High-density (heavy-wall) Polyethylene (HDPE) conforming to Section 238 and as amended below.

- Conduit

PE conduits shall be SDR 11 meeting ASTM D 3035. The Contractor shall provide one orange conduit for fiber optic or ITS communications cable installations, where shown in the Plans. Where additional conduits are required along the same path, subsequent conduits shall be color-striped or solid-colored to distinguish each one separately. Colors or conduit stripe colors shall be submitted to the Engineer for review and approval prior to use. The conduit shall include all required fittings and incidentals necessary to construct a complete installation.

- Conduit Transitions

Only conduit couplings and fittings specifically listed for transition between designated conduit types shall be used.

- Utility RFID Marker Balls and Pegs

Utility ball markers shall conform to APWA uniform color coding for marking underground utility lines and industry standard frequency for detection. Communications (orange) 101.4 kHz marker balls shall be able to be detected at a depth of 5 feet below grade and shall be between 4 to 4.5 inches in diameter. Peg markers shall be detectable at a depth of 2 feet below grade and 3 to 4 inches in length. Marker balls and pegs shall have a passive antenna circuit and; therefore, not need an internal power source to be functional. Markers and pegs shall be a polyethylene, weather resistant enclosure, or an Engineer approved equivalent. The Contractor shall preprogram markers and pegs, and the Department will verify the contents for detection capability at least 30 days prior to the installation.

## 812.03 – Procedures

- Fiber Conduit Installation Requirements

The Contractor shall install conduits in continuous unspliced runs between enclosures and junction boxes, unless otherwise approved by the Engineer. The installation of the fiber optic backbone conduit shall be placed in front of the tree line as far from the travel lane as possible (maintaining as straight an alignment as possible) and outside of any ditches or stormwater retention areas. A minimum distance of 10 feet shall be maintained from the edge of pavement unless otherwise approved by the Engineer. Install conduit a minimum of 30 inches below grade, except where bedrock is encountered in which case an installation depth of at least 18” may be permitted with the Engineer’s approval. All conduit installations shall be within the right of way.

The Contractor shall prevent water and debris from entering a conduit riser above ground during construction by sealing the conduit with tape or any other approved temporary protective measure.

The conduit system shall not exceed the fiber optic cable manufacturer’s allowable bending radius after installation. Fiber optic cable manufacturer’s installation instructions shall be provided to the Engineer upon request.

Fiber shall be installed in the orange conduit and mule tape, or an approved equal, shall be installed in all spare and empty conduits.

HDPE or PE conduit may be installed by trenching, boring, or plowing and/or as indicated on the Plans. Submit a conduit installation plan to the Engineer for approval prior to the start of construction that details where each method of installation is planned for use. Conduits shall be installed by directional boring for any locations where the conduit will cross existing pavement or other existing facilities unless otherwise shown on the plans or approved by the Engineer.

Conduit shall terminate horizontally into pull boxes at a maximum depth of 18 inches. All conduit terminations in pull boxes, junction boxes, cabinets, etc. shall be sealed using a method and materials approved by the Engineer.

- Conduit Transitions

Transitions between conduit types shall occur in junction boxes unless otherwise approved by the Engineer.

Where transitions are approved outside of a junction box, the Contractor shall install couplings specifically listed by the manufacturers for such purpose for the conduit materials being joined.

- RFID Marker Ball/Peg Installation Requirements

Locator tape shall be installed 6 to 8 inches below finished grade. The Contractor shall furnish and install non-detectable underground locator tape with the wording “WARNING – Fiber Optic Cable.”

RFID markers balls shall be placed in open trench operations below the locator tape, 12 inches below finished grade. RFID marker pegs shall be placed in pilot holes 18” below finished grade and above conduits that are installed by directional boring operations.

The Contractor shall use orange colored RFID markers to designate communication and telephone installations for ITS or fiber conduits.

RFID marker balls shall be placed every 100 feet in instances where minimum utility conflict is involved and where route deviation is minimal.

Standard placement of RFID marker balls is at 25 foot increments for open trench operations. RFID near surface markers are to be installed in pilot holes above the conduits at 20 foot increments or at every other rod length with directional boring applications. In areas where conduit routing significantly changes and where higher utility conflicts are potentially involved (such as at traffic intersections and interchanges), marker balls shall be placed every 25 feet unless otherwise directed by the Engineer.

RFID marker balls shall be placed at every change in direction of the conduit/cable and with 5 foot offsets to boxes and vaults. RFID near surface marker pegs may be substituted for RFID marker balls when permitted by the Engineer

The Contractor shall install a minimum of two marker balls per each utility trench.

The Contractor shall program the following data on the RFIDs: unique 10-digit serial number, utility owner designation, item description, item details, placement depth below grade, placement date, orientation, elevation, and information about item (for example, a pipe or cable lies below).

The data will be input into a standardized format as shown below:

| Label | Description |
| --- | --- |
| Company |  |
| Description |  |
| Depth |  |
| Date |  |
| Size |  |
| Cable # |  |

Similar technology may be used in lieu of marker balls or pegs only when preapproved by the Engineer.

- GPS Mapping

The Contractor shall use a handheld GPS device to provide a data base of electronic markers and matched GPS coordinates for each RFID marker with sub-foot accuracy. The handheld GPS devices shall have integrated software to facilitate mapping the marker template information in DGN format as an overlay to the MicroStation Plans, and KML using Google Earth/ Map as the base mapping, then copied as a PDF or other approved format as required by the Engineer. This electronic as-built information shall be provided to the Department within 10 days of successful completion of the installed ITS utility system. The handheld GPS device mapping shall include all the components as required for documenting a complete as-built installation.

- Testing

The Contractor shall perform a mandrel test on each conduit to ensure no conduit was damaged during installation. The mandrel test shall be performed after the installation of conduits and after completion of tamping and backfilling the installation. The Contractor shall furnish a non-metallic mandrel having a diameter of approximately 80% of the inside diameter of the conduit through which it is to be pulled. If damage has occurred to the conduit, the Contractor shall replace the entire length of conduit between the corresponding junction boxes or enclosures. Ensure pull tape, locator tape, and marker balls and pegs are re-installed with the replacement conduit installation.

## 812.04 – Measurement and Payment

Bored ITS Conduit (Size) will be measured in linear feet and will be paid for at the contract unit price per linear foot for the size specified. This price shall include directional bore installation of conduit, providing conduit, fittings, couplings conduit bodies, pull tapes, locator tape, RFID markers and pegs, mule tape, testing, and disposal of surplus and unusable material. This price shall also include electronic as-built information of conduit routes.

ITS Conduit (Size) will be measured in linear feet and will be paid for at the contract unit price per linear foot for the size specified. This price shall include providing and installing conduit, fittings, couplings, pull ropes, pull tapes, locator tape, RFID markers and pegs, testing, and electronic as-built information of conduit routes.

Trench excavation will be paid for separately in accordance with Section 700.05.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Bored ITS Conduit (Size) | Linear Foot |
| ITS Conduit (Size) | Linear Foot |