Guidelines: For use on all projects where permanent stormwater management facilities will be constructed or modified.

SP105-001000-01 [formerly SP105-100100-01]

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

CONSTRUCTION RECORD DOCUMENTATION OF PERMANENT STORMWATER MANAGEMENT FACILITIES

February 1, 2018

- Description

This specification covers the requirement for the Contractor to provide Construction Record Documents of permanent stormwater management facilities (SWMF).

- Definitions

  - Construction Record Documents (CRDs). Documents that record and detail the construction and final state of a SWMF, including, but not limited to, construction record surveys, shop drawings, and all certifications required in the Contract for the specific type of SWMF.

  - Licensed Professional. A Professional Engineer, Land Surveyor, or Certified Landscape Architect licensed to practice in the Commonwealth of Virginia.

- Requirements

The Contractor shall provide CRDs and other required information identified in Section IV for all permanent SWMF shown in the Plans. CRDs shall comply with Section 105.10(c) of the Specifications.  All survey work and drawings shall comply with the VDOT Survey Manual and CADD Manual.

CRDs shall document the items summarized in Section IV for each type or category of SWMF on the Project. The CRDs shall be signed and sealed by a Licensed Professional.

A digitally signed and sealed copy of the CRDs and other required information for permanent SWMF on the Project shall be provided to the Engineer prior to Final Acceptance.

Deviations from the Plans that result in a decrease in the water quality or quantity volumes, or any change to the shape, size, location or elevations of the facility or its associated structures shall be shown on the CRDs for the Engineer’s review. The Contractor shall be responsible for making any corrections to the SWMF required by the Engineer and updating the CRDs prior to Final Acceptance.

- CRDs for Permanent Stormwater Management Facilities

CRDs shall be provided for the following types of permanent SWMF’s shown in the Plans:

  - Constructed Wetlands, Wet Ponds, Extended Detention, and Dry Detention Basins. These facilities require a construction record survey which shall include:

    - Finished elevations, including pretreatment areas, basin floor elevations, bench elevations, pool elevations, and embankment contours and elevations.

    - Horizontal location of basin footprint, spillway, outfall structure and outlet protection.

    - Spillway dimensions and elevations.

      - Riser shape and elevations (crest and bottom).

      - Orifice shape, dimensions, and elevations.

      - Weir shape, dimensions, and elevations.

      - Barrel shape, dimensions, and elevations (inlet and outlet).

      - Emergency spillway shape, dimensions, and elevations.

    - Baffle location, shape and dimensions.

  - Infiltration, Bioretention, and Filtering Practices. These facilities require a construction record survey which shall include:

    - Finished elevations including pretreatment areas, filter bed surface, berm and earthen spillway.

    - Horizontal location of observation wells, cleanouts, spillways and outfall.

    - Types of outlet and overflow structures, shape and elevations (crest and bottom).

    - Pipe barrel shape, dimensions, and elevations (inlet and outlet).

    - Underdrain pipe shape, size and invert elevations.

    - Underground storage structure type, shape, dimensions, and elevations.

  - Manufactured Treatment Devices (MTDs) and Permeable Pavement. Manufacturer’s shop drawings shall be provided for all manufactured components of MTDs and Permeable Pavement. A statement for planting in conformance with the Plans shall be included. MTDs require a construction record survey which shall include:

    - Horizontal location of the facility and outfall.

    - Horizontal location of observation wells and cleanouts.

    - Rim and invert elevations of associated structures or access location.

- Measurement and Payment

Construction Record Documents for permanent SWMF will be paid for at the Contract lump sum price. This price shall include performing the work described herein on all SWMF’s shown on the Plans.

Payment will not be made until the Contractor provides the Engineer with CRDs, signed and sealed by a Licensed Professional, and they are accepted by the Engineer.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Construction Record Documents | Lump sum |