GUIDELINES — For projects where rideabilty evaluation of asphalt concrete pavement is required in writing by the Materials Engineer.  The proposal must contain the exact locations and limits of the rideabilty evaluation, including incentive only locations.  For plant mix schedule projects the exact locations and limits must be contained in schedule notes or in a project specific SPCN in the proposal.  For all other types of projects the exact locations and limits must be included in a project specific SPCN in the proposal. {2007-S315R01}

SP315-000710-01 [formerly SP315-070100-01]

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

RIDEABILITY

(For Asphalt Concrete Pavement)

August 3, 2017

- Description

For pavements designated in the Contract, the final ride quality acceptance will be based on the lowest average International Roughness Index (IRI) for each 0.01-mile section produced by a minimum of two test runs, using a South Dakota style road profiling device and reported for each travel lane.  The device shall measure both wheelpaths with laser height sensing instruments.  The Department will conduct the testing within 30 calendar days from Contractor’s written request for testing following the completion of the final surface course and final pavement striping over the designated section.  If temporary pavement marking is placed and the lanes are clearly delineated over the completed final surface course, the Contractor may request ride testing in writing and the Department will conduct testing within 30 calendar days from the request. The Department will conduct the testing as soon as possible upon receipt of the Contractor’s testing request, providing the Contractor can arrange unimpeded access to the paved surface for constant highway speed test runs.  Testing will be conducted according to VTM–106.

- Acceptance

An IRI number in inches per mile will be established for each 0.01-mile section for each designated lane.  The last 0.01-mile (52 feet) section before a bridge, the first 0.01-mile (52 feet) section after a bridge, and the beginning and end 0.01-mile (52 feet) sections of the final surface will not be subject to a pay adjustment.

Areas excluded from testing by the road profiling device will be tested using a 10-foot straightedge.  The variation of the surface from the testing edge of the straightedge between any two contacts with the surface shall not be more than 1/4 inch.  Humps and depressions exceeding the specified tolerance shall be subject to correction as directed by the Engineer, at no additional cost to the Department.

  - Incentive-disincentive projects

    - General

Tables A and B provide the acceptance quality of pavement based on the finished rideability for interstate and non-interstate roadways.

| TABLE A INTERSTATE SYSTEM |  |
| --- | --- |
| IRI After Completion (Inches Per Mile) | Pay Adjustment (Percent Pavement Unit Price) |
| 45.0 and Under | 115 |
| 45.1-55.0 | 110 |
| 55.1-70.0 | 100 |
| 70.1-80.0 | 90 |
| 80.1-90.0 | 80 |
| 90.1-100.0 | 70 |
| 100.1-120.0 | 60 or Subject To Corrective Action |
| 120.1-140.0 | 40 or Subject to Corrective Action |
| 140.1-160.0 | 20 or Subject to Correction Action |
| Over 160.0 | 0 or Subject to Corrective Action |

| TABLE B NON-INTERSTATE SYSTEM |  |
| --- | --- |
| IRI After Completion (Inches Per Mile) | Pay Adjustment (Percent Pavement Unit Price) |
| 55.0 and Under | 115 |
| 55.1-65.0 | 110 |
| 65.1-80.0 | 100 |
| 80.1-90.0 | 90 |
| 90.1-100.0 | 80 |
| 100.1-110.0 | 70 |
| 110.1-130.0 | 60 or Subject To Corrective Action |
| 130.1-150.0 | 40 or Subject to Corrective Action |
| 150.1-170.0 | 20 or Subject to Corrective Action |
| Over 170.0 | 0 or Subject to Corrective Action |

The Engineer reserves the right to require corrective action according to Tables A and B.  The method of correction shall be reviewed for approval by the Engineer and correction shall be performed at the Contractor’s expense.

Corrections to the riding surface of Asphalt Concrete Pavement, other than remove-and-replace, will not be permitted prior to the Department’s rideability testing. Reheating of asphalt concrete pavement will not be permitted. No incentives will be provided for sections on which corrective actions other than remove-and-replace have been performed before rideability testing.

The Engineer will be the sole determining authority on whether corrective measures submitted by the Contractor are acceptable. If the Contractor performs corrective action to the pavement without prior approval from the Engineer, no incentive payment will be made for the Asphalt Concrete Pavement within the limits where corrections were performed.

The Engineer may require correction of any or all adjoining traffic lanes or shoulders at the Contractor’s expense to ensure uniform cross section.

Where corrections are made after the initial Department rideability test, the pavement will be retested by the Department to verify that corrections have produced the acceptable ride surface. No incentives will be provided for sections on which corrective actions have been required by the Engineer.  Additional corrections may be required by the Engineer based on the retested IRI measurements at the Contractor’s expense.  In the event the corrective actions do not result in 100 percent payment, and not subject to further corrective action, the Contractor will be assessed the corresponding percent payment.

    - Single-Lift Construction

An AC layer is defined as a material lift equal to or greater than 2.5 times the maximum nominal aggregate size for the AC mixes specified in the Contract.  A material lift less than the specified application rate or less than 2.5 times the maximum nominal aggregate size for the AC mixes specified in the Contract is considered a “scratch course” and not an AC layer.

Where only one AC layer shall be placed, the Department will test pavement sites subject to this special provision prior to work by the Contractor.  Upon request by the Contractor, the Department will provide the IRI testing results.  If this IRI testing is conducted more than 180 calendar days prior to the scheduled beginning of the work, the Department  or Contractor may request new IRI testing.

If the completed surface has IRI test results which indicate a 30 percent or more improvement in the ride quality, based on the average IRI (original surface and completed overlay) for each 0.1-mile length of each travel lane subject to this Special Provision, no corrective action will be required.  This percent improvement is based on the 0.1-mile paved section average IRI and not the individual 0.01-mile increments.  When the percent improvement is achieved for a 0.1-mile section, the payments (incentives, disincentives and full payment) for the individual 0.01-mile increments will be summed.  The Contractor will then be paid the greater of the total adjusted payments or 100 percent for that 0.1-mile section.

This rideability specification does not relieve the Contractor from responsibility concerning workmanship according to the Specifications, other contract requirements or as defined by the Engineer.

  - Incentive Only Projects

For projects designated as “incentive only”, Table C will be applied for calculating pay adjustment.  A pay adjustment calculation will be made at each 0.01 mile segment and summed over each 0.1 mile.  Any penalties, calculated at each 0.1 mile, will be ignored for incentive only projects.  Only pay adjustment calculation producing an incentive for each 0.1 mile (if any) section will be summed to determine the total incentive over the project.  Therefore, no disincentive will be assessed over the entire project.  The contractor will be paid the greater of the total incentive or 100 percent payment for the project.  The standard exemptions will be applied to calculate the average IRI over the lane.

| TABLE C – INCENTIVE ONLY PROJECT |  |
| --- | --- |
| IRI After Completion (Inches Per Mile) | Pay Adjustment (Percent Pavement Unit Price) |
| 60.0 and Under | 115 |
| 60.1-70.0 | 110 |
| 70.1-85.0 | 100 |
| 85.1-95.0 | 90 |
| 95.1-105.0 | 80 |
| 105.1-115.0 | 70 |
| 115.1-135.0 | 60 |
| 135.1-155.0 | 40 |
| 155.1-175.0 | 20 |
| Over 175.1 | 0 |

Incentive only projects will not be subject to corrective action as a result of the rideability results.  Ride testing prior to paving by the Department is not required for incentive only projects.  Pay adjustments will be applied to the theoretical tonnage of the surface mix asphalt material for the lane width and section length tested.  This rideability specification does not relieve the Contractor from responsibility concerning workmanship according to the Specifications, other contract requirements or as defined by the Engineer.

- Measurement and Payment

Pay adjustments will be applied to the theoretical tonnage of the surface mix asphalt material for the lane width and section length tested (generally 12 feet wide and 52.8 feet long) based on testing prior to any corrective action directed by the Engineer.  For the sections where corrective action is required, pay adjustment will be based on the testing after the corrective action has been accomplished.