Guidelines: For use on all projects requiring installation of Safety Wedge.

SP315-DB0320-01

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

PAVEMENT SHOULDER WEDGE

March 19, 2020

- Description

This work shall consist of installing a consolidated pavement shoulder wedge shape at locations designated in the Plans or as directed by the Engineer. A pavement shoulder wedge is formed by a pavement shoulder wedge device to produce a consolidated wedge shaped pavement edge.

- APPLICABILITY

The Design-Builder shall install shoulder wedge on new construction, mill-and–resurface and overlay projects that mill or pave to the edge of pavement.  The Design-Builder shall install shoulder wedge on roadway segments where all of the following conditions are met:

- Open ditch sections (no curb and gutter).

- Paved Shoulder Widths 4 feet wide or less.

- Speed limits greater than 35 mph.

- Specified final asphalt surface lift thickness at least 1.25 inches.

The Design-Builder shall install shoulder wedge on other roadway segments directed by the Engineer.

The Design-Builder shall stop installation of the shoulder wedge in specific locations where any of the following field conditions are present, and resume installation after passing these locations:

- Driveways, intersections, interchanges, or bridges.

- Ditch slope begins within one foot from the edge of pavement or less than one foot of unpaved shoulder exists, if wedge is placed on unpaved shoulder as shown in Figures 3 and 4.

- Guardrail exists and the face of guardrail is within 3 feet from the existing edge of pavement.

- Equipment

The Design-Builder shall use a wedge forming and consolidating device to create a sloped edge profile onto the roadway shoulder. The device shall accommodate varying wedge thicknesses from 1-1/4 inches to 5 inches, compact the asphalt concrete, and provide a sloped wedge equal to 30° ±5° measured from the pavement surface cross slope extended. The device shall be adjustable to accommodate transitions to cross roads, driveways and obstructions. The Design-Builder shall not use a conventional single plate strike off.

The Engineer may require a test section constructed at the beginning of work to demonstrate the edge shape, after the compaction of the pavement surface, to the satisfaction of the Engineer

- PROCEDURES

The Design-Builder shall prepare the unpaved shoulder to accept the pavement shoulder wedge by removing soil build up and vegetation that exists within one foot from the existing edge of pavement.

Pavement shoulder wedge shall be placed either on the existing pavement (Figures 1 and 2) or on the unpaved shoulder (Figures 3 and 4) as directed in the Plans or by the Engineer.

Figure 1: Wedge over Existing Pavement – Low Shoulder

Figure 2: Wedge over Existing Pavement – High Shoulder

Figure 3: Wedge over Unpaved Shoulder – Low Shoulder

Figure 4: Wedge over Unpaved Shoulder – High Shoulder

The Design-Builder shall attach the shoulder wedge device to the screed of the paver that confines the material at the end gate and extrudes the asphalt material in such a way that results in a compacted wedge shape pavement edge equal to 30° ±5° measured from the pavement surface cross slope extended. Contact shall be maintained between the device and the road shoulder surface and allow for transitions to cross roads, driveways and obstructions. The Design-Builder shall use the device to consolidate the asphalt to increase the density of the extruded profile.

The Design-Builder may perform handwork such as transitions at driveways, intersections, interchanges, and bridges or other areas approved by the Engineer.

After paving is complete, shoulders, where specified, shall be constructed to smoothly tie the graded shoulder edge elevation to the adjoining elevation of the final pavement top suface edge or final paved or stabilized shoulder top surface edge.  The shoulder shall also be graded to obtain a uniform shoulder slope to the shoulder break that conforms to the Standard Drawings.

The Design-Builder shall furnish and place aggregate base material where specified.  The material shall be spread, graded, and compacted in accordance with Section 305.03(e) of the Specifications, except for the shaping of the subgrade which will not be required.

The Design-Builder shall follow the SWPPP and ESC Plan in the Contract.