# Guidelines: For use in contracts involving construction of new bridge substructures.

# SP401-DB0100-01

# VIRGINIA DEPARTMENT OF TRANSPORTATION

# SPECIAL PROVISION FOR

# CLEARING AND GRUBBING AT BRIDGE APPROACHES

# November 14, 2018

- DESCRIPTION:

This work shall consist of clearing and grubbing at the future sites of bridge approaches in accordance with Section 301 of the Specifications, except as modified by this Special Provision.  Clearing and Grubbing shall be performed prior to embankment construction in the vicinity of all bridge abutments, regardless of the height of the embankment fill and the height of the select backfill that will be placed behind the abutment.

- PROCEDURES:

The Design-Builder shall remove stumps, vegetation, trees, brush, roots, perishable material and nonperishable, manmade objects (e.g., fences) in the vicinity of all proposed bridge abutments.  Complete clearing and grubbing shall be conducted in advance of embankment fill placement in all areas where embankment fill will underlie the select backfill behind abutments.

All wet, loose, soft or disturbed soils that are present after clearing and grubbing operations are performed shall be removed or otherwise improved in accordance with Section 303 of the Specifications prior to embankment construction.

Clearing and grubbing shall be performed from the front toe of the embankment to the point behind the select backfill (see Longitudinal Limits in Figure 1).  The lateral (transverse) limits of the clearing and grubbing shall extend from right toe-of-slope to left toe-of-slope.

Figure 1

Limits of Clearing and Grubbing in the Vicinity of Bridge Abutments

Not to Scale