GUIDELINES — For projects requiring expansion joints cleaned and resealed only. {2007-S404D01}

SP404-000100-00

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

SEALING EXPANSION JOINTS

June 15, 2015; Reissued July 12, 2016

I.	DESCRIPTION

This work shall consist of cleaning and resealing expansion joints according to the Contract and as directed by the Engineer.

II.	MATERIALS

Expansion joint filler and sealer materials shall conform to Section 212 of the Specifications.

III.	PROCEDURES

Expansion joints shall be cleaned and shall be free of oil, grease, existing joint material or any other foreign material.  Loose material shall be removed from the joint with oil-free compressed air delivered with not less than 120 cubic feet of air per minute and a nozzle pressure of not less than 90 pounds per square inch and not more than 200 pounds per square inch.

The Contractor shall protect the edges of pavement adjacent to the joints to be cleaned.

The Contractor shall install joint filler and sealer materials according to the manufacturer’s written instructions.

Expansion joints shall be filled and sealed according to Section 404.05 of the Specifications.  Joints to be filled shall be completely dry and the ambient air temperature shall be at least 45 degrees F.  The applied sealer and finished joint shall be free of entrapped air.  Finished sealer shall conform to the lines and grades of existing pavement surfaces.

IV.	MEASUREMENT AND PAYMENT

Clean and reseal expansion joints will be measured in linear feet and will be paid for at the contract unit price per linear foot.  This price shall be full compensation for cleaning joints, furnishing and installing joint filler, joint sealer, removal and disposal of debris, and for all material, labor, tools, equipment and incidentals necessary to complete the work.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Clean and reseal expansion joint | Linear foot |