SP505-000200-00

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

GUARDRAIL TERMINAL SITE PREPARATION

October 2, 2020

I.	DESCRIPTION

This work shall consist of preparing run-on guardrail terminal sites in accordance with the Plans,  Specifications,  and Standard Drawings, and as directed by the Engineer.

- MATERIALS

Aggregate Material shall be No. 21A or No. 21B conforming to Section 208 of the Specifications.

III.	PROCEDURES

Guardrail terminal sites shall be prepared in accordance with Sections 301, 303, 305 and 505 of the Specifications and in accordance with the Standard Drawings. Site preparation requirements for the NCHRP-350 compliant GR-6, GR-7, or GR-9 terminals and the GR-MGS-2 terminal are included in the Standard Drawings for each terminal.

The type of site preparation required will be denoted on the Plans. The types are defined as follows:

- Guardrail terminal site preparation (minor) will be required for sites where minimal grading is required to provide the site preparation detailed in the Standard Drawing for the proposed terminal. The existing sites closely conform to the details in the Standard Drawings and require approximately 6 tons or less of aggregate base material (Type I or II, No. 21A or 21B) to prepare the terminal site.

- Guardrail terminal site preparation (4-ft moderate) will be required for sites where the Plans require GR-MGS2 terminals to be installed using the “limited use site preparation requirements” of Standard GR-MGS2, requiring a four-foot level bench area as measured from the face of the GR-MGS2 rail to the hinge point. These sites should require approximately between 6 and 16 tons of aggregate base material (Type I or II, No. 21A or 21B) to prepare the terminal site.

- Guardrail terminal site preparation (6-ft moderate) will be required for sites where the Plans require GR-MGS2 terminals to be installed using the “site preparation requirements” of Standard GR-MGS2, requiring a six-foot level bench area as measured from the face of the GR-MGS2 rail to the hinge point. These sites should require between 16 and 35 tons of aggregate base material (Type I or II, No. 21A or 21B) to prepare the terminal site as required on the Plans.

The quantity range of aggregate base material listed above is intended  to show scope and scale of the amount of grading work that could be needed to establish the site prep. In addition to, or in place of aggregate base material, borrow material may be used for grading sites, if approved by the Engineer. Guardrail terminal sites shall be fertilized and seeded in accordance with Section 603 of the Specifications with the seed mixes and fertilizer applied at the rates detailed in the Plans or as directed by the Engineer.

IV. MEASUREMENT AND PAYMENT

Guardrail terminal site preparation will be measured in units of each per site and will be paid for at the Contract each price. This price shall include: clearing and grubbing; excavating; backfilling holes; placing fill material; benching existing slopes; all required erosion and siltation control measures;  and restoring site including fertilizing and seeding. The cost of mobilizing equipment and material shall be included in the Guardrail terminal site preparation price.

Aggregate base material used for terminal site preparation, and for shoulder restoration for the guardrail runs downstream of the terminal will be measured in tons and paid for at the Contract ton price. This price shall include furnishing, delivering, placing, compacting, and grading the material; and removing and disposing of surplus, tracked, and spilled material resulting from the Contractor’s operations.

Borrow excavation used for terminal site preparation, and for shoulder restoration for the guardrail runs downstream of the terminal will be measured in cubic yards in accordance with Section 303.06(a)2 of the Specifications and will be paid for at the Contract cubic yard price. This price shall include furnishing, delivering, placing, compacting, and grading the material; and removing and disposing of surplus, tracked, and spilled material resulting from the Contractor’s operations.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Guardrail Terminal Site Preparation (Type) | Each |
| Aggregate Base Material (Type and No.) | Ton |
| Borrow excavation | Cubic yard |