Guidelines: For use on all projects that establish work zones on limited-access highways for longer than 6 months or when directed by the District Traffic Engineer.

SP512-DB0140-00

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

INSTALLATION OF “WORK ZONE NO HANDHELD COMMUNICATION DEVICES” SIGNS

March 18, 2020

- Description

This work shall consist of installing temporary signs stating “Work Zone No Handheld Communication Devices” (R16-V3) in all work areas on limited access highways and other locations shown on the Plans or directed by the Engineer.

- Materials

R16-V3 sign material shall conform to Section 512 of the Specifications.

- Procedure

R16-V3 Graphic Layout shall conform to Figure 1. The Design-Builder shall install the R16-V3 sign in all work areas on limited access highways, entrance ramps adjacent to these work areas or adjacent to a R16-V3 sign location, and other locations shown on the Plans or directed by the Engineer.

Figure 1: Work Zone No Handheld Communication Devices (R16-V3) Sign

The Design-Builder shall use the 60-inch x 48-inch R16-V3 sign on limited access highways and the 48-inch x 36-inch R16-V3 sign on all other roadways. The R16-V3 sign shall be the first sign seen by an approaching driver and be at least 500 feet in advance of the next temporary (construction) sign.