GUIDELINES — Use on projects where removal of asbestos from a bridge structure is required. If an asbestos report is available, a copied note should be created detailing the locations, neshap category, condition, and approximate quantities of asbestos.

# SP522-000240-00

# DEPARTMENT OF TRANSPORTATION

# SPECIAL PROVISION FOR

# REMOVAL OF ASBESTOS FROM BRIDGE STRUCTURES

September 12, 2017

- GENERAL

# 1.	Description of Work:

This Special Provision applies to the removal of asbestos containing materials (ACM) from bridge structures scheduled for demolition, renovation, reconstruction, or replacement. The Contractor is hereby advised that the bridge structures identified in the Plans may contain asbestos. The Contractor shall assume that any ACM contain asbestos in a quantity sufficient to be a health hazard when disturbed or found in a degraded state or friable condition.

Where asbestos inspection results indicate the presence of ACM, the Contractor shall employ a licensed asbestos abatement firm and a licensed project designer to conduct asbestos abatement and project design activities, respectively, on bridge structures, as appropriate. The firms shall furnish all labor, materials, supplies, and equipment necessary to legally remove and dispose of ACM identified in the asbestos inspection report and as-built construction plans as required by Federal and State regulations.  For renovation projects, only the ACM identified in the asbestos inspection report that will be removed or disturbed as part of the renovation shall be removed. All quantities are estimates. The Contractor shall be responsible for ascertaining the exact amount of material to be removed and removing it in accordance with the provisions herein.

Where no asbestos inspection report is available or where asbestos removal from any bridge appurtenance that will be re-occupied (e.g. bridge tender’s office) is required, the Contractor shall comply with requirements found elsewhere in the Contract.

# 2.	Contract Limitations

A.	An asbestos abatement firm is not eligible to perform work on projects if asbestos abatement activities were or will be performed by individuals or firms with an employer/employee relationship or financially affiliated with the asbestos inspection firm performing inspection activities.

B.	An asbestos abatement firm is not eligible to perform asbestos abatement activities if the asbestos sample analyses were performed by individuals or firms with an employer/employee relationship or financially affiliated with the laboratory performing sample analysis.

# 3.	Definitions and Abbreviations:

All definitions and abbreviations used in this Special Provision are consistent as defined under OSHA; the USEPA Asbestos National Emission Standards for Hazardous Air Pollutants (NESHAP); the Virginia Department of Labor and Industry (DOLI); the Virginia Department of Professional and Occupational Regulation (DPOR); and the Virginia Department of Environmental Quality (VDEQ).

- WORK PRACTICES

# 1.	Regulations

- The Contractor and its subcontractors shall comply with all applicable EPA, OSHA, VDEQ, DOLI, and DPOR regulations, and shall follow all applicable EPA and DOLI/OSHA workplace guidelines. EPA workplace guidelines include, but are not limited to: Parts I and II of EPA guideline document 560/5-85-024,“Guidance for Controlling Asbestos Containing Materials in Buildings”; EPA regulations 40 CFR 61 Subparts A and M; and “Demolition Practices Under the Asbestos NESHAP” (TRC Environmental Corporation Work Assignment No. IA2-19). OSHA workplace guidelines include, but are not limited to, any currently applicable OSHA compliance directives or instructions. In any instance of conflict between the DOLI and OSHA requirements, the DOLI requirements shall take precedence. Any “de minimus” quantity exemptions that are provided in the aforementioned references shall not apply to VDOT asbestos removal activities.

B.	The asbestos abatement firm is required to maintain at the job site copies of EPA, VOSH/OSHA, and applicable state and local government regulations regarding the handling of ACM.

C.	The firms and their employees shall be licensed to perform asbestos activities in accordance with DPOR requirements.

# 2.	Notifications

- The Contractor shall make all required notifications at least 20 days prior to beginning removal of ACM. Pursuant to 40 CFR 61.145(b), the Contractor shall also provide the requisite ten day demolition notification irrespective of minimum quantity or other exclusions. The Contractor shall also provide written notice to the Engineer prior to work being performed. Notifications shall be submitted to the DOLI and the EPA Region III, Land and Chemical Division according to Section 107.01 of the Specifications.

- The 20 day notification is only required for the removal of Regulated Asbestos Containing Materials (RACM). RACM are friable asbestos material; Category I non-friable ACM that has become friable; Category I non-friable ACM that will be or has been subjected to sanding, grinding, cutting, or abrading; and Category II non-friable ACM that has a high probability of becoming crumbled, pulverized, or reduced to powder by the forces expected to act on the material in the course of demolition or renovation. If any Category I or Category II non-friable ACM becomes friable during removal, the Contractor shall stop work and make all notifications.

# 3.	Competent Person:

- The asbestos abatement firm shall have a “competent person” present at all times while work on this contract is in progress. The competent person shall be thoroughly familiar and experienced with asbestos removal and related work; shall monitor and enforce the use of all safety procedures and equipment; and shall be knowledgeable of all National Institute for Occupational Safety and Health, EPA, OSHA, DPOR, DOLI, and VOSH requirements and guidelines.

B.	The competent person shall have a valid asbestos supervisor’s license issued by the DPOR in accordance with the provisions of  VA § 54.1-500 et seq.

4.	Additional Contractor Responsibilities

- The Contractor shall select a landfill approved for disposal of friable ACM and  for disposal of non-friable ACM by the USEPA and appropriate state.

- The abatement firm and its employees performing work under this Specification have been trained in the proper procedures to follow in case of an emergency.

- The abatement firm and its employees performing work under this Special Provision have an established medical surveillance program in compliance with 29 CFR 1926.1101 and a written respirator program in compliance with 29 CFR 1910.134.

- Copies of Material Safety Data Sheets are obtained for any chemical solvents that will be used.

5.	Materials, Work Areas, and Support Equipment

- Personnel of other trades not engaged in the removal of asbestos materials shall be excluded by the Contractor from areas where there is potential exposure to airborne concentrations of asbestos; the abatement firm shall erect appropriate signage and barricades.

All air handling equipment, if required, shall arrive at the job site in a clean (uncontaminated) condition and shall be compliant with ANSI 29.2.

- The abatement firm is responsible for providing all equipment necessary to access areas containing ACM.

- The abatement firm shall provide appropriate decontamination facilities and shall be responsible for providing any necessary electrical and water sources and disposal of all contaminated materials.

- Work areas shall be prepared in accordance with 40 CFR 61 Subpart M; other applicable EPA guidance; 29 CFR 1926.1101; and any currently applicable OSHA compliance instructions.

6.	ACM Management

  - All RACM shall be removed prior to demolition or renovation.

  - Any Category I or Category II non-friable ACM present during demolition, renovation, post-demolition, or post-renovation subject to sanding, grinding, cutting, crushing, breaking, or abrading shall be removed and disposed as RACM.

  - Category I and Category II non-friable ACM, that are not in poor condition and cannot be removed prior to demolition or renovation, shall be separated from the demolition debris. Alternatively, the entire debris mass shall be considered Category I or Category II non-friable ACM in good condition and shall be handled accordingly.

  - Category I non-friable ACM that is not in poor condition before demolition, but where the structure and the ACM can be expected to burn as a result of explosive demolition, shall be treated as RACM; and therefore, shall be removed from the structure before demolition.

  - All Category II ACM found in or on structures scheduled for explosion shall be removed before such demolition.

  - Asbestos Cement (A/C) pipe shall not be ground, broken, crushed, sawed, abraded, or handled in a manner which would cause asbestos material to become friable or airborne. Saw-cutting will only be allowed provided that specifically designed saws equipped with high efficiency particulate air filtered vacuums are used. Cuts shall be continuously sprayed with amended water during cutting. The water shall be collected and filtered or disposed of properly.  With the approval of the Engineer, A/C pipe that will be removed and that is encased in concrete, such as end walls, shall be cut flush with the concrete surface. With approval of the Engineer, the Contractor may pump grout into the buried lines that are no longer in service to maintain the structural bearing capacity of the area. No on-site burial of crushed A/C pipe shall be allowed.

7.	Cleanup

- All work areas shall be cleared of all construction debris and left in a neat and orderly condition.

- All visible accumulations of asbestos material and debris shall be removed, and all surfaces within the work area shall be wet cleaned.

- Sealed drums or equipment used in the work area shall be cleaned and subsequently removed from the work areas.

D.	If the results of the air or surface dust samples indicate that contamination has occurred as the result of Contractor negligence or poor work practices, the Contractor shall clean the site at no additional cost to the Department. In addition, the Contractor shall be liable for any damage claims or other legal actions brought against VDOT, its employees, or brought by VDOT or any persons exposed to such contamination.

E.	The Contractor shall be held responsible for the cost of re-inspections if the work is determined to not be substantially complete.

8.	Disposal of Asbestos Waste

The abatement contractor shall remove, transport, and dispose of the ACM from the job site in accordance with all federal and state regulations and this Special Provision. The Contractor shall be responsible for generating and maintaining a waste shipment record in accordance with applicable local, state, federal, and disposal facility requirements; and shall provide a copy to the Engineer for the Department’s records.

All asbestos waste, scrap, debris containers, asbestos contaminated clothing, and equipment that may produce airborne concentrations of asbestos fibers shall be collected and placed in sealed and properly labeled, 6 mil impermeable bags. Sealed impermeable bags of asbestos waste shall be temporarily stored in asbestos waste containers (drums, trailers, etc.). Uncontaminated containers may be recycled.

- SUBMITTALS

1.	Pre-Work Submittals

Before commencing work, the Contractor shall submit to the Department two copies of the required notifications submitted to the Asbestos Clerk, DOLI.

# 2.	Work/Post-Work Submittals

The Contractor shall submit to the Department:

A.	Copies of licenses for the firms and employees performing work under this Specification.

B.	Within 35 days of depositing a load of ACM waste from this project to the designated landfill, the Contractor shall submit a copy of the certificates of disposal from the landfill to the Department. All acceptable waste manifests and certificates of disposal shall be submitted to the Department before any payments are made to the Contractor for this work.

- SITE INSPECTION

The Department (or its representative) reserves the right to inspect all asbestos abatement operations at any time. If any aspect of the work is found inconsistent with this special provision, a stop work order will be issued and operations will be immediately suspended. Until the inconsistency is corrected, any standby time and costs for corrective actions shall be at the Contractor’s expense.

- MEASUREMENT AND PAYMENT

Where the Department has provided an asbestos inspection report, asbestos removal will be paid for at the Contract lump sum price per structure. This price shall include removal and disposal of all ACM, and submission of required documentation.

Where the Department has not provided an asbestos inspection report, payment for removal and disposal of all ACM will be made under Section 109.05 of the Specifications.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Asbestos Removal (Structure No.) | Lump sum |