SP522-DB0130-01

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

# TREE REMOVAL TIME OF YEAR RESTRICTION FOR ROOSTING BAT HABITAT

November 19, 2018

# Background

This Special Provision applies to projects that are in an environmentally sensitive area for bat species protected under the Endangered Species Act (16 USC 1531 et seq., hereinafter “the Act”) and the Virginia Endangered Species Act (29.1-563 et seq.). Removing trees greater than or equal to 3 inches diameter at breast height (dbh) is restricted as it may result in adverse impacts to bat species due to the removal of roosting habitat during summer months, and may be prohibited by the regulatory agencies at certain times of year and is prohibited during any applicable Time of Year Restriction period. Compensatory mitigation may be required for some impacts, depending upon location, and must be addressed prior to proceeding with work. Compensatory mitigation shall be at no additional cost to the Department.

Tree removal activities associated with this project shall conform to Section 107.01 Division I Amendments (Part 5), the Act, and this Special Provision.

# Requirements

  - The Design-Builder shall coordinate with the United States Fish and Wildlife Service (USFWS) and the Department of Game and Inland Fisheries (DGIF) regarding impacts to protected bat species and shall include the location and number of trees or acres of trees to be removed. Depending upon the location of the project and species involved, these agencies may require additional coordination or application of a Time of Year Restriction on tree removal.

  - If the project requires removing more trees than initially coordinated, the Design-Builder shall re-coordinate the proposed impacts with the agencies and satisfy requirements they may impose.

  - The Design-Builder shall notify the Engineer if additional restrictions are placed on work as the result of agency coordination. The Design-Builder shall provide the Engineer with documentation of all coordination efforts with USFWS and/or DGIF relative to protected bat species.

  - If the Design-Builder fails to comply with these requirements, the work may be suspended and administered in accordance with Section 108 of the Division I Amendments (Part 5) of the RFP.