# Guidelines: use on bridge maintenance projects where active nests have been previously identified and requested by the Project Manager. Federal structure number must be filled in.

# SP522-DB0170-02

# VIRGINIA DEPARTMENT OF TRANSPORTATION

# Special Provision for

# PROTECTION OF NESTING MIGRATORY BIRDS DURING BRIDGE MAINTENANCE

September 2, 2020

- DESCRIPTION

The Design-Builder shall take protective measures for nesting migratory birds as described herein for all structures included in this Contract.

- REQUIREMENTS

Active nests of migratory birds were identified on Structure No. XXXX (Federal Structure Number). A Time of Year Restriction (TOYR), from March 15 through August 15, applies to any activity that requires removal of any nests or contents on any structures included in this Contract. The Design-Builder may proceed with other work activities that do not result in removal of nests or contents during the TOYR.

Before beginning work on any structure where nests have not been previously identified, the Design-Builder shall visually scan each site for migratory bird nesting. If nests are observed on a structure, the Design-Builder shall immediately notify the Engineer and suspend all work in the immediate vicinity of the nests until authorized to continue. The “immediate vicinity” will be defined by the District Environmental Manager based on the specific situation.

With approval from the Engineer, the Design-Builder may take actions to prevent birds from accessing former nests or building new ones. Actions to preempt nesting activities include, but are not limited to, the placement of netting, planking, and tarpaulins.