GUIDELINES — Use on projects identified by the District Environmental Manager where compensatory mitigation site survey data is required.

SP522-DB0200-00

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

COMPENSATORY MITIGATION SITE SURVEY DATA

November 15, 2018

- GENERAL

The Design-Builder shall identify and document compensatory mitigation sites and planimetric features by survey control and conducting as-built surveys in accordance with Section 517 of the Specifications. Planimetric features shall include, but not be limited to: planting zones, site topography, structure locations, Compensation Area Boundaries, and problem areas. The Design-Builder shall follow the specific requirements for data formatting and submittal to the Engineer.

- SURVEY CONTROL

  - Four survey control points shall be established for every mitigation site, one of which should be a VDOT survey control point. If all survey control points are new, one of these shall be tied to an existing VDOT survey control point.

  - Easement and Compensation Area Boundaries shall be referenced to the property boundary, at least one corner of which shall have established horizontal and vertical coordinates.

- AS-BUILT SURVEY DATA

  - As-Built Surveys shall include:

    - Planimetric data necessary to define site character and limitations (e.g., site property boundary, existing and proposed easements, trees, fence lines, open water, buildings, or other existing structures). The survey traverse or baseline shall be clearly marked.

    - The limits of disturbance, at a minimum. If a Compensation Area Boundary is defined in the construction plans or mitigation narrative, this feature shall be used in lieu of the limits of disturbance.

    - Spot elevations and 0.5-foot contour intervals within the Compensation Area Boundary (excluding buffers) and 1.0-foot contour intervals elsewhere.

    - Spot shots at 50 foot on center in all areas where contour intervals exceed 50 foot spacing.

    - Sufficient points to define, in plan view, the channel boundary (i.e., the slope break between floodplain elevations and the channel), bankfull (if different from the channel boundary), and thalweg.

    - Inverts and sizes of all drainage structures.

    - Crest elevations at 30 foot on center of all berms and dams.

    - Inverts of all outlet spillways (10 foot on center with 5 shots minimum per spillway at each end and center).

    - Planting zones (including buffers), if deviating significantly from design planting plans.

    - Instream structure positions and elevations at three points (thalweg, mid-point on structure arms and bankfull benches). Structure survey points must be permanently marked by chiseling or painting to ensure collection of consistent locational data.

    - Stream cross section and longitudinal profile termini (designated by permanent concrete monuments).

    - Well, gage, and panoramic photo station locations, when present during as-built survey.

  - A coordinate table describing the geodetic coordinates and VDOT project coordinates of the Compensation Area Boundary, Easement Boundaries, all point feature locations (e.g., wells, photo stations, stream cross section and longitudinal profile termini), and all structures shall be included. Node points for boundary surveys shall be labeled on the plan sheet with northing and easting coordinates in decimal degrees.

  - Survey data shall be tied to a clearly identified VDOT control point, or to an on-site control point whose coordinates are horizontally and vertically defined. In most cases, control points will be set prior to design. Latitude and longitude are to be reported in decimal degrees using the North American Datum of 1983 (NAD83). Elevations shall be recorded and reported in the same units as the design plan, using the North American Vertical Datum of 1988 (NAVD 88).

  - As-Built surveys must be conducted by conventional survey or survey-grade GPS units.

  - Drainage Structures, instream structures, wells, gages, stream cross section, and longitudinal profile termini survey data shall include elevation, expressed in units consistent with the design plan to two decimal places.

- DATA FORMAT AND DELIVERABLES

  - All survey data shall be submitted in hard copy and digital format. The digital format shall be a file which can be opened by the version of Microstation currently in use by VDOT. Digital data shall be three dimensional, with the exception of planimetric features and wetland boundaries. Digital survey data shall have a VDOT project coordinates coordinate system.

  - Digital or hard copy data (copies of field books, data files or other information in conformance with VDOT Survey Manual standards) shall be submitted with surveys to demonstrate attainment of the horizontal and vertical survey specifications in 90 percent of the observations.