# SP522-DB0300-00

# VIRGINIA DEPARTMENT OF TRANSPORTATION

# SPECIAL PROVISION FOR

# ASBESTOS INSPECTION AND PROJECT MONITORING

September 9, 2020

- PURPOSE

This Special Provision establishes requirements for facility asbestos-containing material inspection in support of asbestos abatement and demolition to include: presence, location, NESHAP category, and estimated quantity; and observing and monitoring activities of an asbestos contractor to include ensuring compliance with the Contract and applicable environmental laws and regulations, collecting air samples before and during an asbestos project, and performing clearance air monitoring requirements in accordance with procedures outlined herein.

- DEFINITIONS

  - Abatement. The procedures to control fiber releases from asbestos containing building materials which includes securing the work area, removing the material, cleaning the work area, and disposing of the material.

  - Abatement contractor. The company or individual properly, licensed in the Commonwealth of Virginia by the DPOR, who conducts asbestos abatement activities such as, but not limited to, removal, encapsulation or enclosure of asbestos-containing materials.

  - AHERA. Asbestos Hazard Emergency Response Act - 40 CFR 763, Subpart E.

  - Asbestos. The asbestiform varieties of actinolite, amosite, anthophyllite, chrysotile, crocidolite, and tremolite, and any of these minerals that has been chemically treated or altered.

  - Asbestos analytical laboratory license. An authorization issued by the DPOR Board of Asbestos, Lead, and Home Inspectors to perform phase contrast, polarized light, or transmission electron microscopy on material known or suspected to contain asbestos.

  - Asbestos Board. The Virginia Board for Asbestos, Lead, and Home Inspectors.

  - Asbestos Containing Material (ACM). Any material or product which contains more than 1.0% asbestos or such other percentage as established by EPA final rule.

  - Asbestos Inspector. An individual who is licensed by the DPOR to perform onsite investigations to identify, classify, record, sample, test and prioritize by exposure potential all friable and non-friable asbestos-containing materials located with a structure.

  - Asbestos Management Planner. An individual who is licensed by the DPOR to develop, implement or alter an asbestos management plan.

  - Asbestos project or asbestos abatement project. An activity involving job set-up for containment, removal, encapsulation, enclosure, encasement, renovation, repair, construction or alteration of an asbestos-containing material. An asbestos project or asbestos abatement project shall not include non-friable asbestos-containing roofing, flooring and siding materials which when installed, encapsulated or removed do not become friable.

  - Asbestos Project Monitor. The individual licensed by the DPOR to observe and monitor the activities of the asbestos abatement contractor on asbestos projects to determine that proper work practices are used and that compliance with all federal, state, and local laws and regulations is maintained.

  - Asbestos Supervisor. Any person with a valid asbestos supervisor license issued by DPOR in accordance with the provisions of VA § 54.1-500 et seq.so designated by an Abatement contractor who provides on-site supervision and direction to the workers engaged in asbestos projects.

  - Building. A combination of any materials having a roof to form a structure for the use or occupancy by persons or property.

  - Category I non-friable ACM. Asbestos-containing packings, gaskets, resilient floor covering, and asphalt roofing products containing more than 1.0% asbestos.

  - Category II non-friable ACM. Any material, excluding Category I nonfriable ACM, containing more than 1.0% asbestos.

  - Clearance Air Sampling. The measurement of airborne asbestos fibers using sampling filters to determine the adequacy and completeness of the asbestos removal actions.

  - Demolition. The wrecking or taking out of any load-supporting structural member of a facility and any related razing, removing or stripping of asbestos products or solid barrier which is known to be containing or to be enclosing an asbestos-containing material.

  - DPOR. The Virginia Department of Professional and Occupational Regulation.

  - Encapsulation. The treatment of an ACM with a material that surrounds or embeds asbestos fibers in an adhesive matrix to prevent the release of fibers, as the encapsulate creates a membrane over the surface (bridging encapsulate) or penetrates the material and binds its components together (penetrating encapsulate).

  - Enclosure. The construction or installation over or around the asbestos-containing material of any solid or flexible coverings, which will not deteriorate or decompose for an extended period of time, so as to conceal the material, contain all asbestos fibers and render the asbestos-containing materials inaccessible.

  - Facility. Any institutional, commercial, public, industrial, or residential structure, installation, building (including any structure, installation, or building containing condominiums or individual dwelling units operated as a residential cooperative), ship, or active or inactive waste disposal site.

  - Friable. A material which is capable of being crumbled, pulverized or reduced to powder by hand pressure or which under normal use or maintenance emits or can be expected to emit asbestos fibers into the air.

  - HEPA. High-Efficiency Particulate Air

  - Homogeneous material. Any material that appears similar in terms of color, texture, pattern, date of material application and functional use.

  - NESHAP. National Emissions Standard for Hazardous Air Pollutants - 40 CFR 61, Subpart M.

  - NIOSH. The National Institute for Occupational Safety and Health.

  - PCM. Phase Contrast Microscopy.

  - Nonfriable ACM. Any material containing more than 1.0% asbestos, that when dry, cannot be crumbled, pulverized, or reduced to powder by hand pressure.

  - RACM. Regulated Asbestos Containing Materials and includes:

    - Friable ACM

    - Category I non friable ACM that has become friable

    - Category I non friable ACM that will be or has been subjected to sanding, grinding, cutting or abrading, and Category II non friable ACM that has a high probability of becoming crumbled, pulverized, or reduced to powder by the forces expected to act on the material in the course of demolition or renovation actions.

  - Removal. The physical extraction of ACM from a Facility and disposal thereof in accordance with all federal, state, and local laws and regulations.

  - Renovation. Altering a facility or one or more facility components in any way, including the stripping or removal of RACM from a facility component, such as the interior or exterior portions of walls, floors, or ceilings of a wing or other portion of a Structure; altering or replacing roofing materials or components and equipment of a Structure; re-roofing; or removing or disturbing any ACM.

  - Repair. The return of friable ACM to a condition where it is not friable or patch or fix a sampling location in such a manner as to prevent fiber release, deterioration of the material, or leakage.

  - Structure. Any bridge or building owned or to be acquired by the Department. The definition of “Structure” in Section 101 of the Division I Amendments (Part 5) will not apply in this Special Provision.

  - Varying visible appearance. Any visible difference in size, color, texture, degree of hardness, etc., which may indicate differing material. This term is synonymous with “visually distinct material”.

  - Work area. Designated rooms, spaces, or areas of the Project in which asbestos abatement actions are to be undertaken or which may become contaminated as a result of abatement actions. A contained work area is a work area that has been sealed, covered in plastic and equipped with a decontamination enclosure system. A non-contained work area is an isolated or controlled access work area that contains ACM that has not been covered in plastic nor equipped with a decontamination enclosure system.

- ASBESTOS INSPECTION PROCEDURES

The Design-Builder’s authorized representative should be contacted to determine if the facility is to be reoccupied or demolished. If the facility is to be reoccupied, then the additional procedures outlined in Section III-2C shall be followed.

  - Minimum Competency Requirements

Individuals conducting inspections of buildings for ACM shall meet the following minimum requirements:

    - Shall have a valid Asbestos Inspector’s License issued by the DPOR pursuant to § 54.1-500 et seq. of the Code of Virginia.

    - Pursuant to 18 VAC 15-20-453, Conflict of Interest, the Licensed Asbestos Inspector shall not have a financial interest in an Abatement contractor who has been contracted to perform asbestos abatement or demolition activities for the Design-Builder.

    - The Licensed Asbestos Inspector shall be familiar with the EPA’s NESHAP demolition practices and applicability to road construction projects (40 CFR 61).

  - Document Review and On-Site Survey

The Design-Builder shall provide a basic sketch of the representative floor plan showing any major detail including estimated or actual room dimensions to identify bulk sample locations and general ACM location.

    - Visual Inspection

All suspect asbestos-containing materials located in or on the structure must be identified. These areas may include but are not limited to:

      - Buildings

- Rooms, hallways, and offices

- Mechanical and electrical equipment rooms

- Pipe chases

- Attics

- The space above ceilings, between walls, and below floors

- Steam tunnels

- Stairwells

- Closets and storage areas

- All occupied and unoccupied spaces

- Crawl spaces, including soil as appropriate

- Floor covering and mastic

- Exterior covering and roofs

      - Bridges

- Asbestos-Cement Panels (typically corrugated)

- Asbestos-Cement Pipe or Conduit (“Transite”)

- Poured Joint Seal/Filler (excluding include obvious elastomeric (Neoprene) expansion joint/dam or 100% Silicone (Putty) Joint)

- Asphalt-Based Pipe Wrap/Tape

- Asphalt Saturated Felt

- Bearing Pads (excluding elastomeric i.e. neoprene and red-lead canvas)

- Fibrous or textured coating on concrete or wood

- Fibrous (Brushed/Rolled On) Silver Paint

- Mastics/Caulks

- Isolator/insulator Pads

- Concrete Rail Post Expansion Material

Areas which are not accessible for sampling shall be identified on the building sketches. A notation must be made in the inspection report as to why the areas could not be investigated. All materials in these areas are to be considered ACM.

    - Bulk sampling

All sampling areas shall be clearly marked on the drawings required by these procedures.

Representative samples of each distinct type of suspect asbestos-containing material shall be collected to confirm its asbestos content unless it is assumed to contain asbestos. For drywall and joint compound, a composite sample shall be collected when the joint compound is used to cover nail holes, cracks, sheet joints, wall corners, and other minor patchwork. The Licensed Asbestos Inspector must ensure that a skim coat is not present when collecting a sample from the wallboard system; however, if the joint compound is used as a surfacing material then multi-layered analysis shall be utilized. Distinction between types of material shall be based on at least the:

- Visual appearance, size

- Texture and hardness

- Functional use, including but not limited to insulation, ceilings, walls, boilers, tank, furnace, other mechanical equipment, ceiling pipes, pipe wrapping, elbow material, valve material, structural members, decks, beams, duct material, roofs, exterior coverings, fire doors, or stage curtains

- Information provided by documents, interviews, or any source as to prior renovation or patchwork;

- Patchwork

- Condition of material

For bridge structures, Department documentation demonstrates that structural concrete or red-lead canvas bedding material beneath bridge bearing plates are not suspect ACM and do not require sampling.

      - Number of samples

        - Sprayed or troweled material. Three random samples for each visually or functionally different material of known different application for up to 1,000 square feet, five random samples from 1,000 to 5,000 square feet, and seven random samples when over 5,000 square feet.

        - Pipe and duct insulation. At least of three samples for every 150 linear feet of material of varying size or visual appearance. Samples shall be taken where material is damaged or exposed where possible, to avoid breaching intact covering.

        - Valve or fitting muds. Three samples of valve material or elbow mud for each insulated line of varying size or visual appearance. Samples shall be taken were material is damaged or exposed wherever possible.

        - Boilers, tanks, and furnaces. Three samples per unit, if homogeneous. If not homogenous, three samples of each material.

        - Patchwork. One sample of each patch or repair.

        - Ceiling or acoustical tile. Three samples for each material of varying visible appearance.

        - Built-up roof systems (field of roof). Three samples per 10,000 square feet plus one additional sample per each additional 5000 square feet per homogeneous area.

        - Other roofing materials. Three samples per homogeneous area.

        - Other materials. Three samples per homogeneous materials.

      - Selection of sample location

        - For sprayed on or troweled on material, the EPA guidelines located in their “Asbestos-Containing Materials in School Buildings - Guidance for Analytical Programs” publication will be followed.

        - For other types of uses, visually distinct materials will be sampled.

        - Wallboard system samples shall be collected at corners and joints.

      - Bulk sample size

Samples shall be taken to penetrate all layers of the material. Samples should contain at least 5 cubic centimeters of materials, and shall be placed in a container and sealed at the time of collection, prior to submittal to laboratory with an asbestos analytical laboratory license.

      - Sampling precautions

The facility shall be vacated prior to taking samples for the time necessary to collect samples. Although destructive sampling is required, the Licensed Asbestos Inspector shall attempt to obtain samples from discrete locations and care shall be taken to minimize property damage. When samples are taken in areas where the material is in poor condition, care shall be taken to prevent further deterioration or fiber release. Proper procedures and equipment shall be used during sampling to minimize fiber generation.

    - Reoccupying structures

Care shall be taken to minimize fiber release; however, any visible debris or residue generated during the sampling shall be thoroughly removed by wet wiping the debris or HEPA vacuuming. An area at least four feet in each direction adjacent to the sample area shall also be cleaned using the above methods.

Locations from which samples are taken shall be patched immediately upon removal of the sample and may be treated by low pressure application of an approved encapsulate.

The sample location shall be adequately patched by the Licensed Asbestos Inspector to prevent fiber release, deterioration of the material, or leakage. Any damage created as a result of the improper or incomplete patching of a material after sampling will be the responsibility of the Licensed Asbestos Inspector.

    - Structures to be Demolished

Right-of-way must be acquired and property vacated prior to asbestos sampling activities.

For samples collected from structures to be demolished, the “positive stop” method of analysis should be used to ensure that no additional samples from a homogeneous area will need to be analyzed after one of the samples has been found to contain greater than 1.0% asbestos.

Sampling of multi-layered systems shall follow EPA’s “Clarification Regarding Analysis of Multi-layered Systems” (Federal Register: January 5, 1994 and December 15, 1995) and other appropriate guidelines, especially in regards to sampling of “wallboard systems”, or joint compound and tape used to cover nail holes, cracks, sheet joints, wall corners and other minor patch work over drywall.

The Licensed Asbestos Inspector should identify samples of drywall and joint compound (when not used as an Add-on Material) as “Wallboard System” on the chain of custody and in their reports. These samples must be analyzed as a composite. The Licensed Asbestos Inspector must also ensure that a skim coat is not present when collecting samples from wallboard systems.

- REPORTING

The Licensed Asbestos Inspector shall provide to the Design-Builder a narrative report in hard-copy and electronic format which includes:

- Name, address, and telephone number of the inspection firm

- Copies of the licenses for the firm and Licensed Asbestos Inspectors performing work under this procedure

- The Department’s project number

- Virginia and Federal identification for each bridge inspected

- The Department’s structure number and address of each building inspected

- A complete description of the building and building type

- The location and type of all building materials suspected of containing ACM

- The location, percent asbestos, type of asbestos, approximate quantities, general condition of the ACM discovered, and a list the of sample numbers collected in or on each structure

- A site map or sketch of the Structure indicating the location at which the samples were collected and the locations of the ACM identified for all Structures inspected

- Color photographs

- A table identifying the NESHAP Category of the materials and which RACM must be removed prior to demolition and which non-regulated Asbestos Containing Materials (non-RACM) that may remain during demolition. Non-RACM that may be present in the structures to be demolished includes Category I Non-friable ACM in “good condition” (e.g. resilient flooring, asphalt roofing products, mastics), and certain Category II Non-friable materials (e.g. floor tile mastics) that do not have a high probability of becoming or will not become crumbled, pulverized or reduced to powder by the forces expected to act on the material in the course of demolition

- A table identifying structures which require demolition with ACM in accordance with EPA guidance “Demolition Practices Under the Asbestos NESHAP”

- A cost estimate for those items requiring abatement, including Project Monitoring costs

- Proper “Chain-of-Custody” documentation

The Design-Builder shall provide a copy of the report to the Engineer.

- ASBESTOS PROJECT MONITORING PROCEDURES

Project monitoring activities will be performed on facilities owned by the Department when renovation or demolition activities are planned. Any NESHAP ACM threshold quantity exemptions related to notifications and procedures for asbestos emission controls  or reduced OSHA requirements for activities involving less than 25 linear feet or 10 square feet of ACM shall not apply to Department asbestos removal activities in structures to be demolished for road construction projects.

  - Minimum Competency Requirements

    - Individuals observing and monitoring asbestos abatement projects and conducting air sampling shall have a valid Asbestos Project Monitor License issued by DPOR

    - Pursuant to 18 VAC 15-20-453 the Licensed Asbestos Project Monitor shall not have a financial interest or employee/employer relationship with an asbestos abatement or demolition contractor who has been contracted to perform abatement or demolition activities for the Department.

    - Laboratories conducting asbestos analysis shall have a valid Asbestos Analytical Laboratory License issued by DPOR.

    - Pursuant to 18 VAC 15-20-453 the Asbestos Analytical Laboratory utilized for air sample analysis on asbestos projects shall not have a business relationship concerning the projects with the abatement or demolition contractors, nor have financial affiliation with the abatement or demolition contractors.

    - The Licensed Asbestos Project Monitor shall be familiar with the EPA’s NESHAP demolition practices and applicability to road construction projects (40 CFR 61).

    - The Licensed Asbestos Project Monitor shall be familiar with the “Asbestos Removal and NESHAP-Related Demolition Requirement for Structures” Special Provision.

  - Project Monitoring Procedures and Requirements

The Licensed Asbestos Project Monitor does not have the authority to waive compliance with or change the requirements of these procedures or any part of the Contract. The Licensed Asbestos Project Monitor shall maintain a daily log detailing the progress of the work, interactions with the Abatement contractor and his personnel, all arrival and departure times, and any other information or contacts which may impact on the project or its outcome.

The Licensed Asbestos Project Monitor may monitor the asbestos abatement activities at no more than two structures on one project at any time. The Licensed Asbestos Project Monitor shall observe all abatement activities during the course of the work covered by the asbestos abatement contract.

The Licensed Asbestos Project Monitor shall meet with the Abatement contractor daily to review work, progress, discuss problems and adjust methods as needed. The Licensed Asbestos Project Monitor shall ensure that the Abatement contractor has an Asbestos Supervisor present at all times during abatement activities.

The Licensed Asbestos Project Monitor shall have the authority to correct or stop any job activities if they are not being performed in accordance with all applicable regulations or guidelines or with the Contract. The Licensed Asbestos Project Monitor shall inform the Abatement contractor of any area samples outside the containment with results in excess of 0.01 fibers/cc. The Abatement contractor shall immediately discontinue operations until the violation is corrected.

All instances of improper performance under the abatement requirements will be reported to the Design-Builder verbally upon initial observation and followed up with a written description of the activity, description of why the performance is improper and recommended corrective actions within 24 hours. The Design-Builder shall provide copies of the written descriptions and proposed corrective actions to the Engineer within 24 hours of receiving them from the licensed Asbestos Project Monitor.

The Licensed Asbestos Project Monitor shall notify the Design-Builder’s authorized representative at the completion of the asbestos removal portion of each project in a manner designated by the Design-Builder.

    - Pre-Abatement Requirements

The Licensed Asbestos Project Monitor will review required submittals from the Abatement contractor and provide comments to the Design-Builder. Required submittals include, but are not limited to, a plan detailing the removal activities, notification documentation, laboratory information, qualification and training records of the asbestos contractor’s employees, and medical surveillance and respirator program.

The Licensed Asbestos Project Monitor will conduct a pre-work walk through with the Abatement contractor and the Design-Builder. All parties will agree to quantities and the Asbestos Project Monitor will put into writing the agreed to quantities. The Design-Builder will forward the agreed to quantities statement to Engineer before the start of the any abatement activities.

The Licensed Asbestos Project Monitor shall review and approve preparation of Abatement contractor’s work area. Removal activities may not proceed without Licensed Asbestos Project Monitor’s approval.

The Licensed Asbestos Project Monitor shall collect pre-abatement air samples per the Pre-Abatement and Environmental Air Monitoring Procedures.

The Licensed Asbestos Project Monitor shall inspect and sign off on barriers and decontamination enclosure systems.

    - Pre-Abatement and Environmental Air Monitoring Procedures

These procedures are to be utilized when collecting pre-abatement and environmental air samples associated with the removal of asbestos-containing materials. All samples shall be analyzed by PCM unless modified by the Engineer. No other changes may be made without prior approval from the Engineer.

      - Pre-Abatement Air Samples

Air samples shall be collected prior to the set-up and removal of asbestos-containing materials which are being removed inside of a facility. Pre-abatement air samples will not be collected for the removal of roofing or exterior materials. The pre-abatement air samples are to be analyzed by PCM analysis.

A calibrated flow rate of at least 8 L/min and at most 10 L/min shall be used to collect pre-abatement air samples. A total volume of at least 1,200 liters and at most 1,500 liters shall be collected.

The following shall be used to determine the number of pre-abatement air samples to be collected:

- One sample for areas less than 100 square feet or 25 linear feet of materials to be removed.

- Two samples for areas between 100 square feet and 500 square feet or 200 linear feet of material to be removed.

- Three samples for areas greater than 500 square feet or 200 linear feet of material to be removed.

      - Environmental Air Monitoring and Abatement Inspection

Air samples shall be collected continuously during the removal process. Air samples shall be collected in areas where the Licensed Asbestos Project Monitor considers the greatest likelihood of fiber release occurring if a breach of containment occurs. No more than 2 locations are to be sampled simultaneously.

A calibrated flow rate of at least 4 L/min and at most6 L/min shall be used to collect environmental air samples. A total of at least 800 liters and at most 1,800 liters shall be collected on each filter.

      - Laboratory Analysis

The PCM analysis shall be conducted using the NIOSH Method 7400 entitled “Fibers” published in the NIOSH Manual of Analytical Methods.

The following minimum information shall be provided to the Engineer:

- Concentration in fibers per cubic centimeter (PCM)

- Analytical sensitivity used for the analysis

- Area analyzed

- Volume of air samples

- Average grid size opening

- Number of grids analyzed

- Copy of the count sheet

- Type of asbestos

- Signatures of laboratory analysts

- Official laboratory identification

- Floor plan indication location where samples were obtained

    - Clearance Air Monitoring

Clearance air monitoring shall be conducted using the specified sampling techniques whenever an asbestos containment area is used to control release of airborne asbestos fibers. Clearance air sampling in structures to be demolished shall use static air sampling techniques (i.e., no aggressive air sampling). Clearance air sampling in structures to be re-occupied shall use aggressive air sampling methods.

Clearance air sampling techniques shall: (1) be used only in an asbestos containment area with effective negative air filtration; (2) be performed only by individuals with a valid Virginia Asbestos Project Monitor License; and (3) not be initiated until a visual inspection is conducted and visible ACM and asbestos debris have been removed.

      - Equipment Requirements

High volume air pumps with necessary peripheral equipment (hoses, connectors, etc.) calibrated to draw from 8 to 10 liters of air per minute through the filter shall be used for air sampling.

Filters shall be 25 millimeter mixed cellulose ester (MCE) filters with a 0.8-1.2 micrometer pore size if the filter is to be analyzed by PCM. Sampling cassettes shall have 50-millimeter extension cowls and shall not have been previously used.

      - Sampling Procedures

        - Set up

Ensure that the area inside the enclosure is visibly clean of all ACM, dust and debris; spray encapsulate utilized for lockdown purposes has dried; and the negative air system is and remains fully operational at a rate of four air changes per hour.

Place the air pumps and sampling cassettes such that each covers approximately the same square footage of floor area, and the exposed filter faces of the sampling cassettes are oriented approximately 45 degrees from the horizontal using tape and clips as necessary to position the sampling cassettes.

Start the pumps and sample for the required time; turning off the air pumps when sampling is completed.

        - Number of samples

The minimum required number of sample cassettes for each enclosure site is listed in the table below (each set of samples consists of inside air samples, field blanks, and a sealed blank).

| Square Feet of Enclosure | Sample Cassettes (PCM Analysis) | Field Blanks1 |
| --- | --- | --- |
| Less Than 100 | 2 | 1 |
| 100-500 | 3 | 1 |
| 500-1000 | 4 | 1 |
| 1000-10,000 | 5 | 1 |
| Greater than 10,000 | 5 + 1 sample per each 5,000 additional ft2 | 1 |
| 1The cap of each field blank cassette is to be removed for not more than 30 sec and replaced (before air sampling is initiated) near the entrance to each abatement area. |  |  |

        - Sampling times

The run time for each air pump shall be calculated by dividing the flow rate (1 L/min) of the pump into the minimum number of liters of air required to be collected (1200 liters for PCM analysis) to obtain the required number of minutes.

        - Recording

A floor plan indicating the locations of the collected sample cassettes, along with a data sheet showing the project name, project monitor, location of project, date samples were collected, calibrated flows for each air pump, start and stop times for each air pump, the preferred method of analyses specified, and calculated number of liters drawn for each cassette sample, shall be transmitted to the laboratory with the cassette samples to be analyzed. The inclusion of these documents is required when generating reports on final clearance air sampling for the project.

        - Shipping

Air sample cassettes shall be shipped in separate containers from bulk samples to prevent sample cassette contamination. Avoid using expanded polystyrene and particle-based packaging materials.

      - Laboratory Analysis

The PCM analysis shall be conducted using the NIOSH Method 7400 entitled “Fibers” published in the NIOSH Manual of Analytical Methods.

The following minimum information shall be provided to the Engineer:

- Concentration in fibers per cubic centimeter (PCM)

- Analytical sensitivity used for the analysis

- Area analyzed

- Volume of air samples

- Average grid size opening

- Number of grids analyzed

- Copy of the count sheet

- Type of asbestos

- Signatures of laboratory analysts

- Official laboratory identification

- Floor plan indication location where samples were obtained

      - Final Clearance

Final clearance shall be achieved when the concentration of fibers for each of the samples is shown to be less than or equal to 0.01 fiber per cubic centimeter of air.

If the results of the clearance samples are above the level specified herein, the abatement site must be re-cleaned, and new sets of sample cassettes collected and analyzed until the abatement area passes. This process shall continue until the abatement area complies with the above standard.

- REPORTING

The Licensed Asbestos Project Monitor shall submit a final report to the Design-Builder within 10 Days of the completion of the asbestos removal portion of the project. The report shall be in hard-copy and electronic format and include:

- Specific accomplishments achieved during contract period.

- Project completion report for each phase.

- Copies of all pre and post abatement submittals, air monitoring results, daily logs, lists and all other information required by the Specifications.

- Proper Chain-of-Custody documentation.