SP522-DB0310-00

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

ASBESTOS REMOVAL AND NESHAP-RELATED DEMOLITION REQUIREMENTS FOR STRUCTURES

September 9, 2020

- Description

This Special Provision applies to the removal of Regulated Asbestos Containing Materials (RACM) from structures scheduled for demolition, renovation, reconstruction, or replacement and to demolition or renovation of structures containing non-RACM. The Structures, as defined in Section II, identified in the Contract contain asbestos. The Design-Builder shall assume that any Asbestos Containing Material (ACM) contains asbestos in a quantity sufficient to be a health hazard when disturbed or found in a degraded state or friable condition.

- Definitions

  - Abatement. The procedures to control fiber releases from asbestos containing building materials which includes securing the work area, removing the material, cleaning the work area, and disposing of the material.

  - Abatement contractor. The company or individual properly licensed in the Commonwealth of Virginia by the DPOR, who conducts asbestos abatement activities such as, but not limited to, removal, encapsulation or enclosure of asbestos-containing materials.

  - Asbestos. The asbestiform varieties of actinolite, amosite, anthophyllite, chrysotile, crocidolite, and tremolite, and any of these minerals that has been chemically treated or altered.

  - Asbestos analytical laboratory license. An authorization issued by the DPOR Board of Asbestos, Lead, and Home Inspectors to perform phase contrast, polarized light, or transmission electron microscopy on material known or suspected to contain asbestos.

  - Asbestos Containing Material (ACM). Any material or product which contains more than 1.0% asbestos or such other percentage as established by EPA final rule.

  - Asbestos project or asbestos abatement project. An activity involving job set-up for containment, removal, encapsulation, enclosure, encasement, renovation, repair, construction or alteration of an asbestos-containing material. An asbestos project or asbestos abatement project shall not include non-friable asbestos-containing roofing, flooring and siding materials which when installed, encapsulated or removed do not become friable.

  - Asbestos Project Designer. The individual licensed by DPOR to prepare or update an asbestos abatement project design.

  - Asbestos Project Monitor. The individual licensed by the DPOR to observe and monitor the activities of the asbestos abatement contractor on asbestos projects to determine that proper work practices are used and that compliance with all federal, state, and local laws and regulations is maintained.

  - Asbestos Supervisor. Any person with a valid asbestos supervisor license issued by DPOR in accordance with § 54.1-500 et seq. of the Code of Virginia so designated by an Abatement contractor who serves as the “competent person” providing on-site supervision and direction to the workers engaged in asbestos projects.

  - Building. A combination of any materials having a roof to form a structure for the use or occupancy by persons or property.

  - Category I non-friable ACM. Any of these items that contain more than 1.0% asbestos: packings, gaskets, resilient floor covering, and asphalt roofing products.

  - Category II non-friable ACM. Any material, excluding Category I nonfriable ACM, containing more than 1.0% asbestos.

  - Clearance Air Sampling. The measurement of airborne asbestos fibers using sampling filters to determine the adequacy and completeness of the asbestos removal actions.

  - Demolition. The wrecking or taking out of any load-supporting structural member of a facility and any related razing, removing or stripping of asbestos products or solid barrier which is known to be containing or to be enclosing an asbestos-containing material.

  - DEQ. The Virginia Department of Environmental Quality

  - DOLI. The Virginia Department of Labor and Industry.

  - DPOR. The Virginia Department of Professional and Occupational Regulation.

  - Enclosure. The construction or installation over or around the asbestos-containing material of any solid or flexible coverings, which will not deteriorate or decompose for an extended period of time, so as to conceal the material, contain all asbestos fibers and render the asbestos-containing materials inaccessible.

  - Facility. Any institutional, commercial, public, industrial, or residential structure, installation, building (including any structure, installation, or building containing condominiums or individual dwelling units operated as a residential cooperative), ship, or active or inactive waste disposal site.

  - Friable. A material which is capable of being crumbled, pulverized or reduced to powder by hand pressure or which under normal use or maintenance emits or can be expected to emit asbestos fibers into the air.

  - HEPA. High-Efficiency Particulate Air

  - NESHAP. National Emissions Standard for Hazardous Air Pollutants - 40 CFR 61, Subpart M.

  - Nonfriable ACM. Any material containing more than 1.0% asbestos, that when dry, cannot be crumbled, pulverized, or reduced to powder by hand pressure.

  - RACM. Regulated Asbestos Containing Materials and includes:

    - Friable ACM

    - Category I non friable ACM that has become friable

    - Category I non friable ACM that will be or has been subjected to sanding, grinding, cutting or abrading, and Category II non friable ACM that has a high probability of becoming crumbled, pulverized, or reduced to powder by the forces expected to act on the material in the course of demolition or renovation actions.

  - Removal. The physical extraction of ACM from a Facility and disposal thereof in accordance with all federal, state, and local laws and regulations.

  - Renovation. Altering a facility or one or more facility components in any way, including the stripping or removal of RACM from a facility component, such as the interior or exterior portions of walls, floors, or ceilings of a wing or other portion of a Structure; altering or replacing roofing materials or components and equipment of a Structure; re-roofing; or removing or disturbing any ACM.

  - Structure. Any bridge or building owned or to be acquired by the Department. The definition of “Structure” in Section 101 of the Division I Amendments (Part 5) will not apply in this Special Provision.

  - Work area. Designated rooms, spaces, or areas of the Project in which asbestos abatement actions are to be undertaken or which may become contaminated as a result of abatement actions. A contained work area is a work area that has been sealed, covered in plastic and equipped with a decontamination enclosure system. A non-contained work area is an isolated or controlled access work area that contains ACM that has not been covered in plastic nor equipped with a decontamination enclosure system.

  - USDOT. The United States Department of Transportation.

- General Requirements

Where asbestos inspection results indicate the presence of RACM, or ACM that have a high probability of becoming RACM during demolition or renovation activities, the Design-Builder shall employ a licensed Asbestos Project Designer, a licensed asbestos abatement contractor, and a licensed Asbestos Project Monitor (APM) to conduct asbestos design activities, abatement, and project monitoring, respectively, on Structures, as appropriate. The firms shall furnish all labor, materials, supplies, and equipment necessary to legally remove and dispose of ACM identified in the asbestos inspection report and as-built construction plans as required by Federal and State regulations. For renovation projects, only the ACM identified in the asbestos inspection report that will be removed or disturbed as part of the renovation must be removed, except that, for structures to be re-occupied, ACM determined to be friable or in poor condition shall be removed before re-occupancy. Any quantities referenced in any ACM Inspection Reports provided by the Department are estimates. The Design-Builder shall ascertain the exact amount of material to be removed and remove it in accordance with the provisions herein. This work may require removing existing flashing and miscellaneous trim in order to remove ACM.

The Design-Builder shall perform demolition of structures in accordance with Section 516 of the Specifications and this special provision.

In accordance with 18 VAC 15-20-453, an asbestos abatement firm is not eligible to perform work on projects if asbestos abatement activities were or will be performed by individuals or firms with an employer or employee relationship or financially affiliated with the asbestos inspection firm performing inspection activities, the Asbestos Project Designer performing design activities, the APM performing monitoring activities, or the laboratory performing sample analysis.

- Work Practices

  - Regulations

The Design-Builder and its subcontractors shall comply with all applicable EPA, OSHA, USDOT, VDEQ, DOLI, and DPOR regulations, and shall follow all applicable EPA, DOLI, and OSHA workplace guidelines. EPA workplace guidelines include but are not limited to:

- Parts I and II of EPA guideline document 560/5-85-024, “Guidance for Controlling Asbestos Containing Materials in Buildings”

- EPA regulations 40 CFR 61 Subparts A and M

- “Demolition Practices Under the Asbestos NESHAP” (TRC Environmental Corporation Work Assignment No. IA2-19)

OSHA workplace guidelines include but are not limited to any currently applicable OSHA compliance directives or instructions. In any instance of conflict between the DOLI and OSHA requirements, the DOLI requirements shall take precedence. Any “de minimus” quantity exemptions that are provided in these references shall not apply to Department-required asbestos removal activities.

The asbestos abatement contractor shall maintain copies of EPA, VOSH, OSHA and applicable state and local government regulations regarding the handling of ACM at the job site.

The firms and their employees shall be licensed to perform the appropriate asbestos activities in accordance with DPOR requirements.

The APM shall conduct project and clearance air monitoring services in accordance with the Department’s Asbestos Project Monitoring and Clearance Air Monitoring Procedures.

  - Notifications

The Design-Builder shall make all regulatory-required notifications at least 20 days prior to beginning the removal of RACM. RACM are: friable asbestos material; Category I non-friable ACM that has become friable; Category I non-friable ACM that will be or has been subjected to sanding, grinding, cutting, or abrading; or Category II non-friable ACM that has a high probability of becoming crumbled, pulverized, or reduced to powder by the forces expected to act on the material in the course of demolition or renovation. If any Category I or Category II non-friable ACM becomes friable during removal, the Design-Builder shall stop work and make all notifications. The on-site APM shall determine friability.

The Design-Builder shall also provide the regulatory-required minimum 10 business day notification prior to starting demolition work for each structure irrespective of minimum quantity or other exclusions in accordance with 40 CFR 61.145(b) and Section 107 of the Division I Amendments (Part 5). The Design-Builder shall copy the Engineer on all correspondence with regulatory agencies.

  - Competent Person

The asbestos abatement contractor shall have a “competent person” present at all times while abatement work on this contract is in progress. The competent person shall be thoroughly familiar and experienced with asbestos removal and related work; shall monitor and enforce the use of all safety procedures and equipment; and shall be knowledgeable of all National Institute for Occupational Safety and Health, EPA, OSHA, DPOR, DOLI and VOSH requirements and guidelines.

The competent person shall have a valid asbestos supervisor’s license issued by the DPOR in accordance with the provisions of VA § 54.1-500 et seq.

  - Materials, Work Areas and Support Equipment

Personnel of other trades not engaged in the removal of asbestos materials shall be excluded by the Design-Builder from areas where there is potential exposure to airborne concentrations of asbestos; the abatement contractor shall erect appropriate signage and barricades.

The abatement contractor shall provide all equipment necessary to access areas containing ACM. All air handling equipment, if required, shall arrive at the job site in a clean (uncontaminated) condition and will be compliant with ANSI 29.2.

The abatement contractor shall provide appropriate decontamination facilities and shall provide any necessary electrical and water sources and disposal of all contaminated materials in accordance with the regulations.

Work areas shall be prepared in accordance with 40 CFR 61 Subpart M, applicable EPA guidance, 29 CFR 1926.1101, and any currently applicable OSHA compliance instructions. The abatement contractor shall perform daily personal air monitoring for asbestos exposure and shall cooperate with the APM for all testing and sampling activities.

In instances where the APM informs the abatement contractor of any area samples outside the containment with results in excess of 0.01 fibers per cubic centimeter, the abatement contractor shall immediately discontinue operations until the violation is corrected.

All instances of improper performance under the abatement requirements will be reported to the Design-Builder verbally upon initial observation and followed up with a written description of the activity and description of why the performance is improper and recommended corrective actions within 24 hours. The Design-Builder shall provide copies of the written descriptions and proposed corrective actions to the Engineer within 24 hours of receiving them from the licensed Asbestos Project Monitor.

  - ACM Management

All RACM shall be removed before demolition or renovation. Any Category I or Category II non-friable ACM present during demolition or renovation or post demolition or renovation subject to sanding, grinding, cutting, crushing, breaking, or abrading shall be treated as RACM.

Category I and Category II non-friable ACM that are not in poor condition and cannot be removed before demolition or renovation shall be separated from the demolition debris. Alternatively the entire debris mass shall be considered Category I or Category II non-friable ACM in good condition and shall be handled accordingly.

Category I non-friable ACM that is not in poor condition prior to demolition, but where the structure and the ACM can be expected to burn as a result of explosive demolition, shall be treated as RACM and, therefore, shall be removed from the structure before demolition. All Category II ACM found in or on structures scheduled for explosive demolition shall be removed before conducting demolition.

Intact removal and connections to Asbestos Cement (A/C) pipe shall be in accordance with the Special Provision for Removal and Connection of Asbestos Cement Pipe. A/C pipe shall not be ground, broken, crushed, sawed, abraded or handled in a manner which would cause asbestos material to become friable or airborne. Saw-cutting will only be allowed provided that specifically designed saws equipped with high efficiency particulate air filtered vacuums are used. Cuts shall be continuously sprayed with amended water during cutting and the water shall be collected and properly filtered or disposed. If the Design-Builder proposes to cut A/C pipe encased in concrete, such as end walls, flush with the concrete surface, the Design-Builder shall only do so with the approval of the Engineer. If the Engineer approves a line to be abandoned, the Design-Builder shall pump grout into the buried lines that are no longer in service to maintain the structural bearing capacity of the area and abandon the lines. No on-site burial of crushed A/C pipe shall be allowed.

Removal of contaminated soils may be required if ACM falls onto unprotected soil. Contamination shall be determined when fragments of ACM are visible as debris or when bulk sample analysis using Polarized Light Microscopy shows an asbestos fiber level in soil greater than 1%. The abatement contractor shall remove any contaminated debris and soil to a depth of 4 inches below the original surface or to hardpan and until all visible debris has been removed. All remaining ACM in the area in conjunction with the contaminated soil shall be removed in a manner to prevent re-contamination of the cleaned areas.

  - Cleanup

All work areas shall be cleared of all construction debris and left in a neat and orderly condition.

For internal work areas, the Design-Builder shall remove visible accumulations of asbestos material and debris and wet clean all surfaces within the work area and clean any other contaminated areas with water or High Efficiency Particulate Air (HEPA)-filtered vacuum equipment. All visible accumulations of asbestos material and debris shall be removed and all surfaces within the work area shall be wet cleaned. Sealed drums and equipment used in the work area shall be cleaned and subsequently removed from the work areas.

The APM shall inspect and approve completeness of the removal and subsequent cleanup actions in the asbestos control area before the abatement contractor removes any barricades. The abatement contractor shall keep the windows and doors sealed and the decontamination system operational until final clearance is certified. Air filtration/negative pressure systems and decontamination enclosure systems shall remain in operation until the time of the initial inspection and in an operational condition until final clearance is certified.

If the results of the air or surface dust samples indicate that contamination has occurred as the result of Design-Builder work practices, the Design-Builder shall clean the site at no additional cost to the Department. In addition, the Design-Builder shall be liable for any damage claims or other legal actions brought against the Department or its employees, or brought by the Department or any persons exposed to such contamination.

The Design-Builder shall be responsible for the time and cost of all re-inspections if the work is determined to not be substantially complete.

  - Disposal of Waste

The asbestos abatement contractor shall remove, transport, and dispose of ACM from the job site in accordance with all federal, state, and local regulations, and this Special Provision. All ACM waste shall be disposed at an EPA or state approved landfill permitted to accept ACM waste. The Design-Builder shall generate and maintain a waste shipment record, regardless of whether the ACM-containing debris is or is not RACM, in accordance with applicable local, state, federal, and disposal facility requirements and shall provide a copy to the Engineer and Quality Assurance Manager (QAM.

All asbestos abatement waste, scrap, debris containers, and asbestos contaminated clothing and equipment that may produce airborne concentrations of asbestos fibers shall be collected and placed in sealed and properly labeled, 6 mil impermeable bags. Sealed impermeable bags of asbestos waste shall be temporarily stored in asbestos waste containers (drums, trailers, etc.). Uncontaminated containers may be recycled.

For non-friable or non-regulated demolition waste, the Design-Builder shall provide notification to the landfill that the debris contains non-friable/non-regulated ACM and shall only dispose of debris containing non-friable ACM in a permitted landfill that provides daily soil cover. The debris shall be transported in covered USDOT-approved containers.

- Submittals

  - Pre-Work Submittals

Before commencing work, the Design-Builder shall submit documentation that the required notifications were submitted to DOLI and EPA to the Engineer and QAM.

  - Work/Post-Work Submittals

The Design-Builder shall submit to the Engineer and QAM:

- Copies of licenses for the firms and employees performing work under this specification.

- A copy of the certificate of disposal from the landfill within 35 days of the deposit of a load of ACM waste from this project at the designated landfill. The Department must receive all acceptable waste manifests and certificates of disposal before any payments to the Design-Builder will be issued.

- Site Inspection

The Engineer may inspect any asbestos operation at any time. If any aspect of the work is found inconsistent with this Special Provision, the Engineer will issue a work suspension order and operations shall be immediately suspended. Any standby time and corrective action costs shall be borne by the Design-Builder.