SP522-DB0320-00

DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

REMOVAL OR CONNECTION OF ASBESTOS CEMENT PIPE

May 15, 2020

- GENERAL

The Design-Builder shall assume any existing pipe designated in the Contract Documents as asbestos cement (A/C) pipe and scheduled for modification, removal or for connection contains asbestos in a quantity sufficient to be a health hazard if found in a friable condition or made friable during removal or connection. A/C pipe is a “facility component” as defined in 40 CFR 61.141. The Environmental Protection Agency (EPA) and the Virginia Department of Labor and Industry (DOLI) consider A/C pipe to be Category II non-friable asbestos-containing materials. Disposal of A/C is regulated by the Virginia Department of Environmental Quality (VDEQ).

II.	CONSTRUCTION METHODS

Modifications of, connections to, or removal of A/C pipe that involve breaking, crushing, saw-cutting or abrading shall comply with the Special Provision for Asbestos Removal and NESHAP-Related Demolition Requirements for Structures.

This Special Provision applies to all removal modifications to A/C pipe where the A/C pipe is removed intact by disconnecting at the slip (bell) joint (with no breakage) and where any subsequent connections are made without disturbing the integrity of the existing pipe. If at any time the Design-Builder determines that the pipe cannot be removed without breakage, abrading, cutting or crushing, the Design-Builder shall cease work and resume operations in accordance with the Special Provision for Asbestos Removal and NESHAP-Related Demolition Requirements for Structures on Design-Build Projects.

The Design-Builder shall spray and saturate pipe joints with amended water prior to disturbing any pipe.

No “T”-type connections shall be made to existing pipe by internally piercing or breaking existing potable water pipe without pre- and post-connection monitoring for asbestos fibers in water downstream of the connection. Any results that exceed 7 million fibers per liter (7MFL) shall be reported immediately to the Engineer and Quality Assurance Manager (QAM).

Removal, connection, hauling, and disposal shall be performed in accordance with applicable local, state, federal, and disposal facility requirements. The Design-Builder shall ensure that the intact A/C pipe sections remain intact during loading and hauling of the material to the licensed disposal facility. The Design-Builder shall double bag or wrap A/C pipe in plastic and seal and mark the materials. The Design-Builder shall dispose of the material in a permitted landfill that provides daily soil cover and only after the Design-Builder has provided notification to the landfill that the material is non-friable/non-regulated ACM. Within 35 days of the deposit of the waste in the landfill, the Design-Builder shall submit a copy of the certificate of disposal from the landfill to the Engineer and QAM. The Engineer must receive all acceptable waste manifests/certificates of disposal prior to making payment to the Design-Builder.

With approval of the Engineer, abandoned portions of A/C pipe may be left in place of origin and backfilled if the pipe is not crushed; however, pipe that is scheduled to be abandoned may not be removed and re-deposited. If the Engineer approves a line to be abandoned, the Design-Builder shall pump grout into buried lines that are no longer in service to maintain the structural weight bearing capacity of the area. No on-site burial of crushed A/C pipe will be allowed.