SP522-DB0330-00

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

NATURALLY-OCCURRING ASBESTOS-CONTAINING SOIL

July 14, 2020

- SITE PREPARATION

Soil within the project limits may contain naturally-occurring asbestos. The Design-Builder shall plan and prosecute excavation, management, and filling activities in such a manner as to be in full compliance with the applicable requirements of all federal, state and local agencies, including but not limited to the Virginia Department of Labor and Industry (DOLI), Virginia Department of Professional and Occupational Regulation (DPOR), EPA, and Virginia Department of Environmental Quality (VDEQ) for the safe handling of this material.

Excavated or exposed naturally-occurring asbestos soil shall be adequately wetted at all times. “Adequately wetted” means sufficiently mixed or coated with water or other aqueous solution to prevent dust emissions. In-situ naturally-occurring asbestos soil shall be left undisturbed as far as practicable. Where excavation is required, handling shall be minimized.

The Design-Builder shall provide the services of a qualified asbestos professional, licensed by DPOR, to perform both perimeter and personal air monitoring for asbestos and to ensure the availability and use of proper personal protective equipment for the Design-Builder personnel. Perimeter monitoring shall be performed on a daily basis. The asbestos professional shall certify that the perimeter monitoring test results are representative of environmental conditions during operational periods; these results and certification shall be provided to the Engineer. If the results of perimeter monitoring indicate levels that exceed the National Emission Standards for Hazardous Air Pollutants (NESHAP) standards at 40 CFR Part 61 Subpart M, the Design-Builder shall cease operations until additional preventative measures can be implemented to reduce airborne asbestos to below acceptable NESHAP levels.

The Design-Builder shall ensure the removal and containment of naturally-occurring asbestos soil from all equipment and personnel, immediately before departure from locations on the site where naturally-occurring asbestos soil exists.

- DISPOSAL

The Design-Builder shall ensure that naturally-occurring asbestos soil, including any surplus naturally-occurring asbestos soil, are utilized in areas containing similar naturally-occurring asbestos soil or disposed in areas containing similar naturally-occurring asbestos soil.