GUIDELINES — For projects with signs, signals or lighting that must conform to AASHTO’s Standard Specifications for structural support for these items.

SP700-000180-03		Sepember 29, 2020

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

MODIFICATIONS TO AASHTO’S SIGN STRUCTURE SPECIFICATION

I.	GENERAL REQUIREMENTS

Lighting (conventional and high mast), signal (overhead, mast arm and span wire), pedestal poles, overhead (span, cantilever and butterfly) sign structures, and ITS structures (camera poles, dynamic message signs (DMS), etc.) shall conform to the requirements of the AASHTO Standard Specifications for Structural Supports for Highway Signs, Luminaires, and Traffic Signals, 6th Edition (LTS-6), 2013 with 2015 and 2019 interims as modified by this Special Provision. Any AASHTO Specification optional design parameter noted as “may be used at the discretion of the owner” that are not addressed in this document shall not be used for design.

Square tube sign post (STP-1 and STP-2), wood post, SSP-VA and SSP-VIA structures shall be provided in accordance with the requirements as shown in the Standard Drawings.

II.	WIND LOADING (LTS-6 Article 3.8 and Appendix C)

1.	The alternate method for wind pressures provided in AASHTO Appendix C shall be used.  Linear interpolation between wind contours is not permitted. The next higher contour shall be used for design. Reduced forces shall not be used for free swinging traffic signal and free swinging sign wind loadings.

2.	LTS-6 Article C.2 is supplemented with the following: Wind speeds using 50-year mean recurrence shall be used for all conventional light poles, high mast light poles, ITS device support poles, and overhead sign structures (span, cantilever and butterfly).

3.	Mast arm signal poles, mast arms, and strain poles shall be designed using the following wind speeds:

| District | Design Wind Speed for strain poles, mast arms, and mast arm poles |
| --- | --- |
| Bristol, Salem, Lynchburg, Staunton and Culpeper | 70 mph |
| Richmond, Northern Virginia and Fredericksburg | 80 mph |
| Hampton Roads | 90 mph |

Ancillary structures procured under regional signal contracts that encompass multiple districts shall be designed for the District with the greatest wind speed within that Region.

Mast arm signal pole and strain pole foundations shall be designed for wind speeds at the foundation location using the 25-year mean recurrence.

4.	For special wind regions in Bristol District shown in Figure 3.8.3-2 of LTS-6, the selection of the design wind speed shall consider localized effects. The minimum design wind speed for 50 year mean in these areas is 90 MPH, 25 year mean in these areas is 80 MPH and 10 year mean in these areas is 70 MPH.

5.	For structures elevated above the surrounding terrain (e.g. bridge mounted light pole, overhead sign, and other structures), the height factor shall be increased to account for the increased wind effects.

III.	STEEL DESIGN

1.	Laminated Structures (LTS-6 Article C5.1): Laminated or multi-ply structures shall only be used in tapered sections.

2.	Holes and Cutouts, Unreinforced and Reinforced (LTS-6 Article 5.14.5): The location and size of hand holes and cutouts shall be in accordance with the details shown in the Standard Drawings. For high mast light poles, the width of unreinforced and reinforced holes and cutouts in the cross-sectional plane of the tube shall not be greater than 50 percent of the tube diameter at that section.

3.	Welding: A connection detail using a full penetration groove weld with a backing ring may be considered for all traffic structures. For tubes 18” diameter and greater, the backing ring shall be attached at the top and bottom face of the ring using a continuous fillet weld. For tubes less than 18” diameter, the backing ring shall be attached at the bottom face using a continuous fillet weld and the top shall be caulked to provide a thick durable continuous seal. The caulk shall be a durable material approved by the Engineer which is formulated for this type of Industrial application.

4.	Diameter: Mast arm signal pole structures shall have the following maximum column and arm outside diameters, unless otherwise approved by the Engineer.

| Configuration | Arm Length | Design Loading | Max. column diameter at base of column | Max. arm diameter at base of arm |
| --- | --- | --- | --- | --- |
| Dual arm | Length of one arm exceeds 70 feet or total length of both arms exceeds 130 feet | Varies (Project specific loads will be provided on the Plans) | 22 inches | 20 inches |
|  | All other dual-arm structures | Design loading does not exceed Standard Drawing MP-3 | 20 inches | 18 inches |
| Single arm | > 75 feet | Varies (Project specific loads will be provided on the Plans) | 22 inches | 20 inches |
|  | ≤ 75 feet | “Case 2” loading as per Standard Drawing MP-3 | 22 inches | 20 inches |
|  |  | “Case 1” loading as per Standard Drawing MP-3 | 20 inches | 18 inches |

IV.	FATIGUE DESIGN

1.	Fatigue Importance Categories (LTS-6 Article 11.6): The following fatigue importance categories shall apply to structures:

| Fatigue Importance Categories |  |  |
| --- | --- | --- |
| Structure Type | Span Length1, ft. | Fatigue Category |
| All structures supporting dynamic message signs or partial dynamic message signs3 | All span lengths | Category I |
| Overhead sign span structure | > 150 | Category I |
|  | ≤ 150 | Category II |
| Overhead sign cantilever structure | > 50 | Category I |
|  | ≤ 50 | Category II |
| Overhead sign butterfly structure | All span lengths | Category II |
| Signal mast arm structure2 | > 75 | Category I and an approved mitigation device |
|  | 50 to ≤ 75 | Category II |
|  | < 50 | No fatigue design required |
| Overhead signal structure | > 190 | Category I |
|  | ≤ 190 | Category II |
| High mast light poles | All lengths | Category I |
| Signal span wires, conventional lights poles and ITS device support poles (excluding DMS) |  | No fatigue design required |
| 1Span length is defined as center-to-center of column(s) for span structure and face-of-column to tip of arm for cantilever and signal structures. 2For twin mast arms, the pole, arms and connections shall be designed for the applicable fatigue category for the longest arm attached. 3For signs that are a combination of primarily static sign panels and thin dynamic message elements, if less than 40% of the sign consist of thin dynamic message elements, the sign may be treated as a static sign for the purposes of determining appropriate fatigue category as long as the thickness of the partial dynamic sign does not exceed 14 inches. A special design is required for the attachment of these structures; the weight and thickness of the thin dynamic message sign element shall be included in the structural analysis. |  |  |

2.	Mitigation Devices (LTS-6 Article 11.6 and 11.7.1): Mitigation devices shall not be used in lieu of designing for fatigue. Approved mitigation devices shall be used for Signal Mast Arm Structures greater than 75 feet in addition to Fatigue Category I design.

3.	Aluminum light poles (LTS-6 Article 11.6 and 11.7.1): Internal first and second mode vibration dampeners shall be provided and installed according to the manufacturer’s instructions in all cases. External dampeners may be used if approved by the Engineer.

4.	Galloping Loads (LTS-6 Article 11.7.1): Galloping loads shall not be considered in the design of overhead sign cantilevered structures with four chord trusses, signal mast arm structures, and multi-chord overhead signal structures.

5.	Truck-Induced Gust Loads (LTS-6 Article 11.7.1.3): Truck induced gust loads shall not be considered in the design of signal mast arm and overhead signal structures.

6.	Vertical Deflection (LTS-6 Article 11.8): The vertical deflection of the free end of the arm for overhead sign cantilevered structures due to the wind load effects of galloping or truck-induced gusts shall not exceed 8”.

V.	FOUNDATION DESIGN

The AASHTO Standard Specifications for Highway Bridges, 1996, and the 1997 and 1998 Interim Specifications, as referenced in the AASHTO Standard Specifications for Structural Supports for Highway Signs, Luminaires, and Traffic Signals, are modified as follows:

1.	Geotechnical Design: The factor of safety shall be as follows:

| MINIMUM FACTORS OF SAFETY1 |  |  |  |
| --- | --- | --- | --- |
|  | Drilled Shaft |  | Spread Footing |
|  | Overhead Sign Structures and all other types of ancillary structures except for Mast arm traffic Signals | Mast arm traffic Signals |  |
| Axial resistance/ Bearing pressure | 1.75 | 1.75 | 2.0 |
| Torsion/Sliding/Skin Friction | 1.752 | 1.32 | 1.23 |
| Overturning (Broms Method) | See horiztonal deflection limits | See horizontal deflection limits | 1.5 |
| 1The factors of safety shown above already account for the 1.33/1.40 group overload/overstress factor. No reduction shall be applied to the design loading used in the analysis. 2Torsion Resistance may be evaluated using the rational method as presented in FHWA-NHI-10-016 Drilled Shafts for Construction Procedures and LRFD design methods. A value of 1.0 shall be used in lieu of the resistance factors. 3Passive resistance shall be reduced by 50% to limit foundation movement. |  |  |  |

In capacity calculations for the foundation design of a drilled shaft, the soil resistance of the top 2.0 feet shall be neglected in the analysis for torsion/skin friction/overturning. Soil resistance from the shaft bottom shall not be included in either torsional or axial resistance of the shaft.

2.	Horizontal Deflection Limits: In lieu of Broms method, COM624P or other commercially available software may be used to evaluate the overturning of shafts and to estimate shaft deflections. For mast arm signals and span wire signals, the total horizontal deflection shall be limited to 0.75 inches at the ground level and the tip of the pile deflection shall not exceed -0.25 inches. For other structures, the total horizontal deflection shall be limited to 0.50 inches at the ground level and the tip of the pile deflection shall not exceed -0.15 inches. The loading used in the analysis shall not be reduced by the allowable overload/overstress factor. The shafts shall be modeled such that the nonlinear flexural rigidity (non-linear EI, or “cracked” section) is accounted for when the horizontal deflections are calculated.

- Reinforcement: Where tremie placement of concrete is anticipated, a minimum spacing of 5 inches or 10 times the size of the largest coarse aggregate whichever is greater shall be provided in both horizontal and vertical direction. For dry shafts, a smaller space of 5 times the size of the largest coarse aggregate may be considered. A dry shaft is when the amount of standing water in the base of the shaft prior to concreting is less than or equal to 3 inches and water is entering the shaft at a rate of less than 12 inches/hour.

- Drilled Shafts: For mast arm traffic signals with an arm 60 feet or greater, the minimum length of drilled shaft shall be 10 feet of embedment unless a spread footing is proposed.