SP703-000130-00

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

ADVANCED TRANSPORTATION CONTROLLER (ATC) CABINETS

July 15, 2021

# DESCRIPTION

This work shall consist of furnishing and installing Advanced Transportation Controller (ATC) cabinets in accordance with the Plans, Standard Drawings, and Specifications, or as directed by the Engineer.

# EQUIPMENT

The ATC cabinets shall be certified by the manufacturer as conforming to the Institute of Transportation Engineers (ITE) Standards: the ATC Cabinet Standard Version 02, ATC 5301 v02.02 dated March 18, 2019 for ATC components, Intelligent Transportation System (ITS) Standard Specifications for Roadside Cabinets, and the ITS Cabinet Standard v01.02.17b dated November 16, 2006 for cabinet components. The ATC cabinets shall conform to any exceptions and additions stated herein unless otherwise specified.

## ATC Cabinet Configurations

### ATC Cabinet - Type I (Standard Configuration)

The Type I cabinet shall consist of an ITS Housing #1 type cabinet with one front and one rear door, one mounting cage, and be mounted on a standard size base adapter. Furnish the Type I cabinet, ready for operation, conforming to Standard Drawing ATCC-1 with the following composition:

Service Assembly

DC Power Supply

SB1/SB2 and DC Power Bus

AC Clean Power Bus

Advanced Detection Assembly

Auxiliary Input / Output Assembly

Input Assembly

Input Test Panel Assembly

Field Input Termination Assembly

Output Assembly

Field Output Termination Assembly

Drawer Shelf Unit

Shelf

### ATC Cabinet – Type II (4 Door Configuration)

The Type II cabinet shall consist of an ITS Housing #3 type cabinet with two front doors and two rear doors, two mounting cages, and be mounted on an oversized base adapter. Facing the front, the left rack cage, Segment A, shall include modular and interchangeable ATC assemblies interconnected through an advanced serial data bus. The right rack cage, Segment B, shall include shelving for the backup power system and a power distribution unit for powering communications devices. Furnish the Type II cabinet, ready for operation, conforming to the Standard Drawing ATCC-2 with the following composition:

Segment A shall consist of:

Service Assembly

DC Power Supply

SB1/SB2 and DC Power Bus

AC Clean Power Bus

Advanced Detection Assembly

Auxiliary Input / Output Assembly

Input Assembly

Input Test Panel Assembly

Field Input Termination Assembly

Output Assembly

Field Output Termination Assembly

Drawer Shelf Unit

Segment B shall consist of:

Power Distribution Unit

(3) Shelf

## General Requirements

The Contractor shall clearly and permanently label all assemblies, switches, terminal blocks, and connectors in the ATC Cabinets. Ensure all fuses, circuit breakers, switches (except police panel switches) and indicators are readily visible and accessible when the ATC Cabinet front door is open. All circuit breakers located on the rack shall include covers to prevent accidental tripping.

Make the marker strips out of material that can be easily and legibly written on using a pencil or ballpoint pen. Install marker strips, where clearly visible, immediately below the item they are to identify.

Provide guides (top and bottom) for assembly plug-in units. The guides shall begin 0.5 inch from the assembly front panel face.

The Contractor shall not expose high-voltage components (over 50 V) per the NEC.

When servicing the ATC Cabinet, the Contractor shall replace the Input Assembly, Output Assembly, and Controller in the flash mode condition, without putting the intersection in a dark condition.

Only install modular assemblies with pluggable cabling. Ensure all cabling is of sufficient length to allow assemblies to be mounted in any position within the rack. Do not hardwire assemblies. Integrate wire raceways as part of the ATC Cabinet, allowing for neat internal and field wiring. Wires and cables shall not block, hinder, or inhibit the installation of standard rack mounted equipment for the full height and width of the rack chassis.

Locate a momentary push button switch labeled “24 VDC BYPASS” on the front of the Output Assembly that, when pressed, energizes 24 VDC to the High-Density Switch Packs during flash mode for troubleshooting purposes.

### Environmental and Electrical

Ensure all components properly operate within the following limits:

Applied Line Voltage: 90 to 135 VAC

Frequency: 60 (±3.0) Hertz

Humidity: 5% to 95%

Ambient Temperature: -30°F to 165°F

Shock - Test per Specification MIL-STD-810G Method 516.6

Vibration - per Specification MIL-STD-810G Method 514

### Assemblies and Files

Mount all assemblies and files on the cage mounting rails per the cabinet model detail. Provide a clear area for the ATC controller unit. The area shall extend 1.5 inches in front of the front EIA mounting angles and 16 inches behind the front EIA mounting angles.

### Stainless Steel

Use stainless steel bolts, nuts, washers, screws (size 8 or larger), hinges, and hinge pins.

### Protection

Protect all conductors, terminals, and parts which could be hazardous to maintenance personnel with suitable insulating material.

## ATC Cabinet Housing

The ATC Cabinet Housing shall conform to the ITS Cabinet Standard v01.02.17b dated November 16, 2006, Section 6.2 Housings, except as specified below.

### General

Base the Type I cabinet Housing on the ITS Housing #1 type enclosure. Base the Type II Housing on the ITS Housing #3 type enclosure. Both cabinets shall include, but not be limited to, the following:

Enclosure & Doors

Gasketing

Lifting Eyes & External Bolt Heads

Door Latches & Locks

Housing Ventilation

Cage Supports and Mounting

Door Hinges & Catches

Police Panel

Aluminum Surfaces

### Material

The enclosure, doors, lifting eyes, gasket channels, police panel, and all supports welded to the enclosure and doors shall be fabricated of 0.125 inch minimum thickness aluminum sheet. Bolted on supports shall be either the same material and thickness as the enclosure or 0.105 inch minimum thickness steel. The Side Panels and filter shell shall be fabricated of 0.105 inch minimum thickness aluminum sheet.

### Housing Dimensions

The Type I cabinet shall measure 66.375 inches (± 0.5 inch) high by 24.25 inches wide (± 0.5 inch) by 30.25 inches deep (± 0.5 inch). The front-to-back cabinet dimensions shall not exceed 36 inches at its widest point, including the door handles, louvers, and roof overhang.

The Type II cabinet shall measure 66.375 inches high (± 0.5 inch) by 44.5 inches wide (± 0.5 inch) by 26 inches deep (± 0.5 inch). The front-to-back cabinet dimensions shall not exceed 32 inches at its widest point, including the door handles, louvers, and roof overhang.

### Aluminum Surface Protection – Exterior Surfaces

Apply an anti-graffiti coating as an aluminum surface protection to the exterior surface of the cabinet housing. The surface protection shall be ANTI-GRAFFITI Paint.

### Anti-Graffiti Paint

The anti-graffiti paint shall conform to the ITS Cabinet Standard v01.02.17b dated November 16, 2006, Section 6.2.2.3.1 for Anti-Graffiti Paint.

### Powder Coating – Interior Surfaces

Powder coat the interior of the cabinet housing and doors white. The powder coat finish shall consist of a Urethane or Triglycidyl Isocyanurate (TGIC) Polyester Powder. Clean and prepare the interior aluminum surface per the powder coating manufacturer’s recommendations. Electrostatically apply the powder coating and then cure per the manufacturer’s recommendations. The thermosetting powder resin shall provide both intercoat as well as substrate fusion adhesion that meets 5A or 5B classifications as per ASTM D3359.

### Lifting Eyes and Exterior Bolt Heads

The Type I cabinet housing shall include two lifting eyes for placing the cabinet on its foundation. Mount the lifting eyes above the front and rear cabinet doors, centered, and extend above the top of the cabinet to provide unobstructed access to the eyes.

The Type II cabinet housing shall include two lifting eyes for placing the cabinet on its foundation. Mount lifting eyes on the sides of the cabinet, centered, and extend above the top of the cabinet to provide unobstructed access to the eyes.

Each lifting eye on the Type I and Type II cabinet shall have an opening measuring at least 0.75 inches in diameter and supports the weight load of 1,000 lbs. All bolt heads shall be tamperproof type.

### Doors, Door Latches, and Locks

The Type I cabinet shall have two vertically hinged doors mounted on the cabinet for interior access, with one door on the front face and one door on the rear face of the cabinet.

The Type II cabinet shall have four vertically hinged doors mounted on the cabinet for interior access, with two doors on the front face and two doors on the rear face of the cabinet.

#### Enclosure Door Frames and Door Seals

Each door shall measure 58.375 inches high (± 0.5 inch) by 24 inches wide (± 0.5 inch). Each of the door openings shall not measure less than 56.375 inches high (± 0.5 inch) by 22 inches wide (± 0.5 inch).

#### Latching Handles

The latching handles shall have a provision for padlocking in the closed position. Each handle shall consist of stainless steel and have a minimum diameter of 0.75 inch with a minimum 0.5 inch shank. Place the padlocking attachment 4 inches from the handle shank center. Provide an additional 4 inch minimum gripping length.

Door handles shall be vertical in the close position. All door handles shall rotate in a direction towards the door hinges to open the cabinet door.

#### Latch and Lock Mechanism

Ensure the latching mechanism is a three-point draw roller type. The pushrods shall be turned edgewise at the outward supports and have a cross section of 0.25 inches thick by 0.75 inches wide minimum. When the door is closed and latched, the lock mechanism shall lock. Ensure the lock and lock support are rigidly mounted on the door. Mount the lock in the upper quadrant, above the handle when in its fully open position. In the locked position, the bolt throw shall extend a minimum of 0.25 inch (+/-0.03125 inches) into the latch cam area. Provide a seal to prevent dust or water entry through the lock opening.

#### Lock and Keys

Ensure the cabinet door locks are mounted to the cabinet with four bolts and the tumbler is keyed to a 2 type. Supply one key with each lock. Keys shall only remove from the lock when the mechanism is in the locked position. Place a swing away cover over the key entrance to protect the lock mechanism.

### Housing Ventilation

Housing ventilation shall include intake, exhaust, filtration, and thermostat-controlled fans.

#### Door Ventilation

The ATC cabinet doors shall have louvered vents installed in the cabinet doors. The louvered vent shall have a maximum of 0.25 inch depth. House a removable and reusable air filter behind the door vents. The filter filtration area shall cover the vent opening area. A filter shell, provided for mechanical support for the filter, shall fit over the filter. This shell shall direct the incoming air downward with louvers. The shell shall house the filter when the sides and top are bent over a minimum of 0.25 inch. The shell shall hold the filter firmly in place with a bottom trough and spring-loaded upper clamp. No incoming air shall bypass the filter. The bottom filter bracket shall form a waterproof sump with drain holes to the outside housing.

The Type I cabinet front door shall have louvered vents.

The Type II cabinet front facing left door and the rear facing left door shall have louvered vents.

#### Intake and Filter

The intake (including filter with shell) and exhaust areas shall pass a minimum of 60 cu. ft. of air per minute.

#### Fans

Each electric fan shall be equipped with ball or roller bearings and shall have a minimum capacity of 100 cu. ft. of free air delivery per minute. The fan shall be fastened to the cabinet with two thumb screws which shall not be fastened in a manner that requires any tools for removal or installation

The Type I cabinet shall have two fans mounted within the housing and protected with a finger guard.

The Type II cabinet shall have four fans mounted within the housing and protected with a finger guard.

#### Temperature Controlling Panel

Temperature controlling panels shall control fans and include one thermostat, an on/off switch, fan test button, and fuses. Fans shall be thermostatically controlled and shall be manually adjustable. The thermostat shall include a control dial, perminently marked with operating temperature between 32°F and 140°F and graded in 20ºF increment scale. The thermostat control dial, when set to a specific temperature, will operate the fans with a differential of 20 degrees between automatic turn on and off. The fan circuit shall be protected at 125% of the fan motor ampacity. Install temperature controlling panels in the same section of the cabinet as the fans which they control.

The Type I cabinet shall have two temperature controlling panels, one controlling each fan.

The Type II cabinet shall have two temperature controlling panels, one controlling each pair of fans. Mount one panel in Segment A and the other in Segment B.

### Hinges and Door Catches

#### Hinges

Stainless steel hinges (two bolts per leaf) shall be provided to bolt the enclosure to the doors. Provide four hinges on each door. Each hinge shall measure 3.5 inches minimum length and have a fixed pin. Weld the pin ends to the hinge and ground smooth. The pins and bolts shall be covered by the door edge and when the door is closed. Install a ground strap between the door and the main cabinet housing when 120 VAC components are mounted on the door.

#### Door Catches

Provide front and rear doors with catches to hold the door open at both 90 degrees (±10) degrees and 165 degrees (±10) degrees. The catches shall consist of 0.375 inch minimum diameter aluminum rods. The catches must have the potential to hold the door open at 90 degrees in a 60 mph wind acting at an angle perpendicular to the plane of the door.

### Police Panel

#### Manual Advance Cable

Each cabinet shall include an interval advance cord, six feet long, with a weatherproof manual police push button.

#### Toggle Power Switches

The Police Panel shall include three double-pole, double-throw (DPDT) switches and labelled as follows: “SIGNAL OFF/ON”, "FLASH/AUTO", and “MANUAL CONTROL ENABLE OFF/ON”. Provide a receptacle for the MANUAL ADVANCE cord.

All switches shall be “ON” or “AUTO” when positioned to the right.

Individually wire the Police Panel switches through factory molded connectors which allow the switches to be disconnected and not usable, as desired by the Department. Wire these connectors in such a way to provide full cabinet operation when any or all Police Panel switches are connected or disconnected.

### Cabinet Lights

Provide LED lighting fixtures that contain a minimum of 10 LEDs, the enclosed unit measures approximately 1.2 inches wide x 12 inches long x 0.37 inches height, operates on 24 VDC power, mounts using metallic clips, and provides 600 lumens per foot.

The Type I cabinet shall include two LED lighting fixtures. Mount each lighting fixture inside and at the top of each cabinet door opening.

The Type II cabinet shall include four LED lighting fixtures. Mount each lighting fixture inside and at the top of each cabinet door opening.

### Door Switches

Install door actuated switches to turn on the cabinet lights when either the front or rear door is opened. Install manual switches to activate and deactivate the lights.

Include additional switches of the same type to actuate when either the front or rear door is opened. Wire all door open switches as inputs to the ATC controller through SB1 or SB2. These switches will be used to provide the “Door Open” alarm when any cabinet door is opened.

### Drawer Shelf Unit

The ATC cabinets shall provide a telescopic slide out drawer to store documents. Mount the Drawer Shelf Unit across the front EIA rails and extend the shelf mounting brackets to the rear EIA rails. Provide the Drawer Shelf Unit with a non-conductive top, locking provision when fully extended, and lip or handle for pulling. The drawer storage area shall have no openings when fully closed to limit rodent intrusion and minimum dimensions shall be 16 inches wide by 14 inches long by 1.5 inches deep.

### Backup Power Confirmation Lights

The ATC Cabinets shall include one red external LED confirmation light indicating when the traffic control system is not operating on primary service power. Provide a LED confirmation light approximately one inch in diameter, located above the rear door in the Type I cabinet or above the left rear door in the Type II cabinet, 4 to 8 inches from the left cabinet edge and half way between the top of the door and the top of the cabinet. The mounting location of the LED confirmation light shall not impede or obstruct cabinet doors or lift plates. The LED confirmation light shall have a durable gasket to ensure weatherproofing. Neatly wire the LED configuration light to a confirmation light relay panel mounted above the Service Assembly for ease of connecting to the backup power system. Permanently label this panel and include terminal blocks for wiring. Configure the light such that when the external LED confirmation light is on, the traffic control system is operating on backup power. When the external LED confirmation light is off, the traffic control system shall operate on primary service power.

### AC- Copper Terminal Bus

The AC- Copper Terminal Bus shall not be grounded to the cabinet or connected to logic ground. Use nylon screws with a minimum diameter of 0.25 inch for securing the bus to the service panel. Provide the AC- Copper Terminal Bus with a minimum number of wiring lugs for each output channel. Include a minimum of 16 available wiring lugs with the 16 channel Output Assembly.

## ATC Cabinet Rack Cage:

Install standard rack cages inside the housings for mounting of the ATC Controller unit and cabinet assemblies. The EIA rack portion of the cage shall consist of four continuous, adjustable equipment mounting angles. The mounting angle shall have a nominal thickness of 11-gauge-plated steel. Tap the mounting angles with 10-32 threads with EIA universal spacing. The mounting angle shall comply with standard EIA-310-B and supported at the top and bottom by either welded or bolted support angles to form a cage. The mounting angles shall provide holes to mount the side panels.

### Clearance between Rails

The rails for mounting assemblies shall have a clearance of 17.75 inches.

### Cage Connection

Bolt the cage to the cabinet at four points via the housing cage supports and four points via associated spacer brackets (top and bottom).

### Cage Location

Center the cages within the cabinet door openings.

## ATC Cabinet Assemblies

The cabinet shall contain the following equipment capable of being completely removed from the cabinet, without removing any other equipment, by using only a slotted or Phillips screwdriver:

Service Assembly

DC Power Supply

AC Clean Power Bus

SB1/SB2 and DC Power Bus

Advanced Detection Assembly

Auxiliary I/O Assembly

Input Assembly (IA)

Input Test Panel Assembly

Field Input Termination Assembly

Output Assembly (OA)

Field Output Termination Assembly

### Service Assembly

The Service Assembly shall be modular, utilize no more than three EIA rack units (e.g. 3U), and mounted on the left or right side of the EIA rack when viewed from the front. Provide the Service Assembly with the following:

Wire a terminal block with 5 screw-type wire connectors for AC+, AC-, Ground, BBS IN, and BBS OUT. Wire AC+ to BBS IN. Install a jumper between BBS IN and BBS OUT that is removable for interconnection with the BBS System. BBS Out shall provide power to the ATC cabinet.

One 30 amp main breaker

Four 15 amp breakers for the Output Assembly, High-Density Flasher Units, raw power and GFCI receptacles, and the Clean Power Bus

Raw power duplex receptacle

GFCI duplex receptacle

Two High-Density Flasher Units with Serial Bus 3 interface

Four fuses for each of the four flasher outputs

A terminal block with 4 screw-type wire connectors for connection to raw power.

### Cabinet Surge Protection Device

Install the cabinet surge protection device in the Service Assembly, pluggable and mated to a 12 pin receptacle for ease of replacement. The surge protector shall filter and absorb power line noise and switching transients. The device shall incorporate LEDs for failure indication. The device shall meet the following minimum specifications:

Peak Surge Current (Single pulse, 8x20µs)	20,000A

Occurrences (8x20µs waveform) 		10 minimum @ 20,000A

Maximum Clamp Voltage			395VAC

Operating Current				15 amps

Response Time					< 5 nanoseconds

### DC Power Supply

Supply the DC Power Supply with the ATC cabinet as a standalone sub-assembly or as an integrated part of another assembly. The DC Power Supply shall be a modular, 19 inch EIA rack mounted device, and provide DC voltages necessary to operate the ATC Cabinet.

The DC Power Supply shall be powered from an AC Line provided by the AC Line cord with NEMA Type 5-15 plug. The input voltage range shall consist of 80 to 270 VAC, 45 to 65 Hz. The DC Power Supply shall provide power factor corrected features and ensure a full load power factor of 0.95 or better, reducing peak AC Line input current and associated stress on wiring. The DC Power Supply shall use modern switching technology, provide full output regulation of 24 and 48 VDC across changes in AC Line voltage, and output load over the full operating temperature range of -30°F to +165°F without the need for a fan. Electrically isolate the DC Output from the AC Mains, Earth Ground, and other DC outputs.

Provide separate clear LED indicators to display green for AC input status, DC output status, and associated fuse integrity. The DC Power Supply shall have fused outputs for over-current protection and shall protect against voltage transients by a 1500 Watt suppressor.

For High Voltage (HV), 120VAC configuration, supply a DC Power Supply rated at 168 Watts, 48 VDC at 1 Amp, and 24 VDC at 5 Amps.

### SB1/SB2 and DC Power Bus

Supply the SB1/SB2 and DC Power Bus with the ATC cabinet as a standalone sub-assembly or as an integrated part of another assembly.

The SB1/SB2 and DC Power Bus, as a standalone unit, shall include a minimum of eight DB25 connectors to interconnect the SB1/SB2 communication ports of the assemblies and the controller. It shall include a termination circuit at the end of the connections (S8) to prevent radio frequency signal reflection and exposure to environmental elements (e.g. rain). SB1/SB2 and DC Power Bus shall include one factory molded plug block to bring DC power to the SB1/SB2 and DC Power Bus; eight factory molded receptacle blocks shall distribute this power to the cabinet assemblies. The copper conductors for the DC voltages shall support a minimum of 10 Amps. Mount the SB1/SB2 and DC Power Bus in the EIA rails, and it shall swing out to provide access to the back of the assemblies mounted on the opposite side.

### AC Clean Power Bus

Supply the AC Clean Power Bus with the ATC Cabinet as a standalone sub-assembly or as part of another assembly. The standalone sub-assembly shall swing out to provide access to the back of the assemblies mounted on the opposite side.

The AC Clean Power Bus shall include a minimum of seven single NEMA 5-15 receptacles, to provide AC clean power to the ATC controller and auxiliary devices.

### Advanced Detection Assembly

Supply the Advanced Detection Assembly with the ATC Cabinet as a standalone sub-assembly or as an integrated part of another assembly. Provide all necessary serial bus and power cabling to connect the assembly to the cabinet busses.

The Advanced Detection Assembly shall include a minimum of three NEMA TS2 Port-1 (SDLC) 15 Pin connectors and used to interface with NEMA TS2 compatible detection systems and hardware. It shall utilize SB2 to interface with the controller and to make it configurable in the controller.

### Auxiliary Input / Output Assembly

Supply the Auxiliary Input/Output (I/O) Assembly with the ATC Cabinet as a standalone sub-assembly or as an integrated part of another assembly. The Auxiliary I/O Assembly shall provide functionality similar to a NEMA TS1 D Connector Panel allowing for 24 VDC inputs and outputs to be available to and from the controller. Use the Auxiliary I/O Assembly to send special function outputs from the controller to cabinet devices or to receive inputs from cabinet devices to the controller (e.g. Uninterruptable Power Supply (UPS) relay alarms, blank-out sign relay panels, preemption panels, or detection inputs). Provide all necessary serial bus and power cabling to connect the assembly to the cabinet busses.

The Auxiliary I/O Assembly shall include a minimum of eight inputs, eight outputs, and a connection to Logic Ground (-24VDC). Map each input and output to the controller through SB2 and make them configurable.  Terminate each of the I/O points and the logic ground in factory molded plug blocks for ease of connection.

### Input Assembly (24-Channel)

The ATC Cabinet shall include two 24-Channel Input Assemblies. The two 24-Channel Input Assemblies shall consist of the upper Input Assembly referred to as the “I” Assembly and the lower Input Assembly referred to as the “J” Assembly.

The Input Assemblies shall consist of a 19 inch EIA rack mounted assembly providing twelve slots of 22/44 pin PCB sockets. Provide a Serial Interface Unit with each assembly, in its location mated to a DIN 96-pin connector. The Serial Interface Unit shall provide interface and control between the ATC Controller and the input assemblies via system SB1/SB2. Each slot shall have the ability of providing two inputs into the controller. Configure Slot 11 and 12 of the J Assembly to support emergency vehicle preemption equipment. Configure these two slots in such a way as to provide sufficient power to the preemption detector modules. Input Assemblies shall have the capability of supporting pedestrian push button detections through any slot, with the use of a DC Isolator.

Provide each Input Assembly with an Opto Input Card. The Opto Input Card shall have four LED indicators and four toggle switches. Toggling any of the four switches shall insert an input to the controller.

### Input Test Panel Assembly

The ATC Cabinet shall include an Input Test Panel Assembly used to place vehicle, pedestrian, or preemption calls to the ATC controller through system SB1/SB2. Provide the Input Test Panel Assembly as a standalone 19 inch EIA rack mounted unit or integrated into another sub-assembly. The Input Test Panel Assembly shall include 16 toggle switches and a LED function indicator light for each switch. Switches shall consist of the following three positions: ON (place call), OFF (normal detector operation), and Momentary ON (place momentary call and return to normal detector operation after the switch is released). Permanently label each switch 1 through 16. Map the 16 toggle switches to an internal Serial Interface Unit and address to Serial Interface Unit 10-13 by a separate 4 position selector switch on the front panel.

### Field Input Termination Assembly

The ATC Cabinet shall include two 24-Channel Field Input Termination Assemblies. Couple the 24-Channel Field Input Termination Assembly with the corresponding 24-Channel Input Assembly. Provide the Field Input Termination Assembly with twelve 10-position terminal block connector plugs with screw-type terminations. Configure each connector plug to provide four Two-wire inputs and one One-wire input for their associated earth ground. Label each terminal block connector receptacle with the number of its associated input channel (1-24) and earth ground.

The Field Input Termination Assembly shall have positions for 12 Detection Module Suppressors. Supply the Detection Module Suppressors with the cabinet. Mount the Field Input Termination Assembly across the EIA rails. The Field Input Termination shall swing down to provide access to the back of the assemblies mounted in the opposite side.

### Output Assembly (16- or 32-Channel)

The ATC Cabinet with 16 output channels shall include one 16-Channel Output Assembly. The Output Assembly shall be a 19 inch EIA rack mounted device, house eight High Density Switch Packs, and shall provide forty-eight load circuits. One Serial Interface Unit shall provide interface and control between the Output Assembly and the controller. The 16-Channel Output Assembly shall house the Cabinet Monitor Unit, Main Contactor, Stop Time Switch, Flash / Auto Switch, four Circuit Breakers and Momentary 24 VDC Bypass Switch.

The ATC Cabinet with 32 output channels shall include one 32-Channel Output Assembly. The Output Assembly shall be a 19 inch EIA rack mounted device, house sixteen High Density Switch Packs, and provide ninety-six load circuits. Two Serial Interface Units shall provide interface and control between the Output Assembly and the controller. The 32-Channel Output Assembly shall house the Cabinet Monitor Unit, Main Contactor, Stop Time Switch, Flash / Auto Switch, eight Circuit Breakers and Momentary 24 VDC Bypass Switch.

### Field Output Termination Assembly

Interconnect the 16-Channel Field Output Termination Assembly with the 16-Channel Output Assembly, and it shall house eight High Density Flash Transfer Relays (HDFTR) and 16 Flash Program Blocks (FPB). Provide the HDFTRs and FPBs to control and select the color (red, yellow, or dark) during ATC Cabinet flash mode. Include pluggable and replaceable transient protectors at the field terminals for the protection of the High-Density Switch Packs. Provide a visual method to indicate when the transient protector has failed. Label each HDFTR position with the number of its associated High-Density Switch Pack (1-16). Label each FPB position with the number of its associated channel (1-16).

The Field Output Termination Assembly shall include sixteen 6-position terminal block connector plugs. The connection method for each terminal position shall be push-in spring connections. Each plug shall have two positions for each red, yellow, and green output channel. Label each terminal block connector receptacle with the number of its associated channel (1-16). Include additional labels to clearly indicate which terminals correspond to the red, yellow, and green switch pack outputs. Match the color of these labels to the color of their associated output (red, yellow, or green).

Mount the 16-Channel Field Output Termination Assembly across the 19 inch EIA rack directly behind the Output Assembly. The 16-Channel Field Output Termination Assembly shall swing down to provide access to the HDSP transient protectors.

Provide one Field Output Termination Assembly with each 16-Channel Output Assembly.

Provide two Field Output Termination Assemblies with a 32-Channel Output Assembly.

## ATC Cabinet Components:

### Cabinet Monitor Unit

Include a Cabinet Monitor Unit (CMUip) for cabinet monitoring, to query various cabinet conditions, and, if the application requires action, transfer control from the ATC to a flashing control mode. Develop the CMUip specifically for the ATC Cabinet, pluggable, interconnected with the Output Assembly, and include communication circuitry to interface SB1 and SB3. The CMUip shall include a microprocessor, memory devices including non-volatile memory, and front panel indicators. The CMUip shall have the capability to fully monitor 32 output channels and utilize direct SB3 communication to each High-Density Switch Pack – Flasher Unit for field voltage and load current status. Program the CMUip with an interchangeable data key and include an Ethernet port for diagnostics and remote management. The CMUip shall include a built-in diagnostic wizard that: analyzes the ATC controller output commands and High Density Switch Pack – Flasher Unit field input status; isolates whether the cabinet fault was caused by an ATC malfunction, failure in the load bay, or field wiring; identifies the faulty channels and output directly; and provides guidance on how the technician should isolate the cause of the malfunction.

Provide the High Voltage (HV) rated CMUip for 120 VAC operation.

### CMUip Data Key Programming Tool

Provide a CMUip data key programming tool with the capability to read and write data from the CMUip data key device. The CMUip data key programming tool software shall have the same manufacturer and full compatibility with the provided CMUip.

### Cabinet Monitor Unit – Auxiliary Display Unit

Provide a Cabinet Monitor Unit – Auxiliary Display Unit (ADU) of the same manufacturer and full compatibility in function and operation with the CMUip. The ADU device shall mount on a 19 inch rack, utilize one rack space, and receive power from the cabinet’s 48 VDC supply. All indicators shall include clear LEDs and shall not depend on reflectors or diffusion as part of the design. Clear LEDs shall not appear to be on when exposed to ambient light. The ADU shall provide 32 columns of LED indicators corresponding to channels 1 through 32. Each column shall include a Red, Yellow, and Green status plus a Blue LED for fault status. The ADU shall provide an enhanced user interface for the ATC Cabinet Monitor Unit system and provide the ability to view status, configuration settings, voltages, and event logs through an LCD menu driven display. The LCD display shall provide detailed status information from the CMUip and displays from the built-in diagnostic program. The diagnostic program provides views of the signal states involved in a fault, pinpoints faulty signal inputs, and guidance on how the technician should isolate the cause of a malfunction.

### High-Density Switch Pack - Flasher Unit

Include compact, pluggable, modular PCB-based High-Density Switch Pack – Flasher Units (HDSP-FU) with the ATC Cabinet. The HDSP-FU shall have compatibility with ultra-low power (less than 2 watts) LED signal heads and have a current monitoring feature for each output of each channel. The HDSP-FU shall use real-time standardized high speed SB3 communications to send a complete set of RMS voltage and load current measurements to the Cabinet Monitor Unit. The HDSP-FU shall measure 4.5 inches high by 6.5 inches deep. Equip the HDSP-FU with a handle, reset push button switch, six RYG LED indictors, four flasher LED indicators, one power LED indicator, and two Rx/Tx LED indicators. The HDSP-FU shall function as either a switch pack (HDSP) or as a flasher unit (HDFU). When installed in the Output Assembly, the HDSP-FU shall function as a switch pack with two RYG channels of operation (six outputs). When the HDSP-FU is installed in the Service Assembly, it shall function as a four-output flasher unit.

Provide the High Voltage (HV) rated HDSP-FU for 120 VAC operation.

### Serial Interface Unit

The Serial Interface Unit (SIU) shall function as the cabinet communications and control unit. Provide a compact and pluggable SIU modular PCB-based device with a half-width faceplate. The SIU shall use real-time standardized 614.4 Kbps communications with the ATC to transfer command and response data on SB1 and SB2. Each SIU shall include 54 programmable input/output pins, four optically isolated input pins, one line sync reference input pin, and 4 address select input pins. The optically isolated inputs shall consist of either 120 VAC or 24 VDC. Provide SIU outputs rated for 150 mA continuous sink current, having a 500 mA typical current limit on each output, having a rating to 50 V, and utilizing a voltage clamp for inductive transient protection. The SIU shall include a front panel LED indicator that can report the current SIU assembly address assignment for cabinet configuration verification. The SIU shall possess a front panel serial port used to provide diagnostics using monitoring software.

### Flash Transfer Relays

The High-Density Flash Transfer Relay (HDFTR) shall have a hermetically sealed cover to ensure it is moisture proof. The HDFTR shall contain dry nitrogen to protect contacts from corrosion and to prevent condensation. The HDFTR shall include a shock and impact resistant metal can cover with solid and bend proof pins. Provide HDFTR’s rated for 5 Amps at 120 VAC switching, 10 Amps surge. Use coil voltage of 48 VDC. The HDFTR shall have an LED indicator to display contact transfer position.

### Cabinet Surge Protection Device

Provide a modular Cabinet Surge Protection Device utilizing a pluggable 12-position Beau 5412 connector; rated for 120 VAC, single phase operation; and integrate as part of the Service Assembly. The Cabinet Surge Protection Device shall incorporate warning and failure indicators with a dry relay contact remote sensing circuit. Provide a Cabinet Surge Protection Device rated for continuous service current of 15 Amp, maximum clamp voltage of 340 VAC, and filter noise and spike from 10 KHz to 25 MHz with a peak surge current of 45.5 kA/total.

### High-Density Switch Pack Transient Protector

Provide a modular and pluggable High-Density Switch Pack Transient Protector designed specifically for traffic controller cabinet operation. Provide the unit epoxy encapsulated and equipped with 9-position 5.08 mm factory molded connector. The unit shall protect up to six circuits. The High-Density Switch Pack Transient Protector shall have an operating voltage of 120 VAC, clamping voltage of 340 VAC, and peak surge current of 39 kA. Ensure the unit dimensions are no greater than 2 inches high by 0.7 inches wide by 2 inches long.

### Detection Module Suppressor

Ensure the Detection Module Suppressor is modular and pluggable. Provide the unit epoxy encapsulated and equipped with 6-position 5.08 mm factory molded connector. The unit shall provide differential and common mode protection for up to six circuits. Provide device operating voltage of 75 VDC and clamping voltage of 130 VDC. Ensure the unit dimensions are no greater than 2 inches high by 0.7 inches wide by 1.2 inches long.

### Main Contactor

Provide mercury-free Main Contactors (MC) rated at 120 VAC at 60 Amps. Include MC coils rated at 48 VDC and equipped with an input indicator and SPST – N.O. contacts.

### Two-Channel DC Isolator

Conform to the ITS Cabinet Standard v01.02.17b dated November 16, 2006 for Two-Channel DC Isolator.

### Two-Channel AC Isolator

Conform to the ITS Cabinet Standard v01.02.17b dated November 16, 2006 for Two-Channel AC Isolator.

### Shelf

Provide a shelf that mounts across all four 19 inch EIA rack rails and provides a surface that extends from the front to the back rails. Ensure the shelf is of the same material as the cage rails and have a minimum weight capacity of 200 pounds to support up to two backup power system batteries. Vent the shelf to allow heat to pass through the shelf surface.

## Auxiliary Communications Cabinet

The Auxiliary Communications Cabinet (ACC) shall be furnished conforming to Standard Detail ACC-1.

### Auxiliary Cabinet

Fabricate the ACC of the same material and finish as the ATC cabinet, and mounted on the right side, below the police panel, when facing the ATC cabinet front.

### Cabinet Dimensions

The ACC cabinet dimensions shall measure approximately 24 inches wide (± 0.5 inch) by 24 inches tall (± 0.5 inch) by 8 inches deep (± 0.5 inch).

### Cabinet Attachment

The ATC cabinet side wall shall form the back wall of the ACC and the ACC shall be continuously welded to provide a complete weather tight seal.

### Knockouts

Provide two 1.375-inch knockouts on the bottom of the ACC. Locate knockouts on the lower left and lower right side of the cabinet, positioned approximately 2 inches from their respective side wall and 3-inches from the back of the ACC.

### Panel Board, Through Holes, and Power Receptacles

Install a 0.75 inch plywood panel board inside the ACC, fastened to the back wall. Center the panel board in the ACC, have a minimum of 0.5 inch gap between the panel board outer edge and the side wall of the communications cabinet.

Install four through holes in the plywood panel board and continuing through the ATC Cabinet wall. The panel board holes will be 2 inches in diameter and the ATC cabinet wall holes will be 1.25 inches in diameter and protected with rubber grommets. Install two through holes on the left side of the panel board. The top hole shall be 6 inches OC from the top ACC wall and 4 inches OC from left ACC wall. The bottom hole shall be 8 inches OC below the top hole and 4 inches OC from the left ACC wall. Install two through holes on the right side of the panel board. The top hole shall be 6 inches OC from the top ACC wall and 4 inches OC from the right ACC wall. The bottom hole shall be 8 inches OC below the top hole and 4 inches OC from the right ACC wall.

Install one double duplex GFCI receptacle inside the ACC and mounted to the plywood panel board. Mount the receptacle 4 inches OC from right and 6 inches OC from bottom of the ACC cabinet. Provide a through hole, protected with a grommet, behind the receptacle enclosure allowing the power cord to pass through the panel board and ATC cabinet wall. Install a 60 inch SJOOW 15 Amp rubber cord with NEMA 5-15P plug, inside the ATC Cabinet connected to raw AC power.

### Lock

The ACC door shall include a lock. Provide a communications lock that is similar in mount and functionality as the cabinet lock, but deliver a TYPE IB tumbler. Key the lock for a master communication key to operation with the TYPE IB lock and tumbler.  Furnish two keys with each door lock. Each communication key shall have a shaft a minimum of 1.75 inches in length.

## Cabinet Base Adapters

### General Requirements

Fabricate the Base Adapters of the same material and finish as the cabinet housing.

### Base Adapter Dimensions

The standard base adapter for the Type I cabinet shall measure a minimum of 12 inches high and 24 inches wide (± 0.5 inch) by 30 inches long (± 0.5 inch). The top and bottom of the base adapter shall have an opening that matches the opening in the bottom of the ATC Cabinet, approximately 15 inches wide by 21 inches long.

The oversized base adapter for the Type II cabinet shall measure a minimum of 12 inches high and 45 inches wide (± 0.5 inch) by 26 inches long (± 0.5 inch). The top and bottom of the base adapter shall have an opening that matches the opening in the bottom of the ATC Cabinet, approximately 15 inches wide by 21 inches long.

### Hardware

Furnish four bolt sets with each Cabinet Base Adapter. A bolt set shall consist of one bolt, two flat washers, and one nut. The stainless steel bolt shall measure 0.75 inch in diameter and 1.5 inches long. Flat washers and nut shall consist of the same material as the bolt.

### Anchor Bolt Holes

Install anchor bolt holes on the top and bottom of the Cabinet Base Adapter. Provide bolts, nuts, washers, and lock washers to bolt the ATC cabinet to the base adapter through these holes.

For the Standard Base Adapter, install four, 1 inch by 2 inch oblong, punched holes on the top of the adapter and match the anchor bolt holes of the ITS Housing #1 type cabinet. Install four, 1 inch by 2 inch oblong, punched holes on the bottom of the adapter that also match the anchor bolt holes of the ITS Housing #1 type cabinet.

For the Oversized Base Adapter, install four, 1 inch by 2 inch oblong, punched holes on the top of the adapter and match the anchor bolt holes of the ITS Housing #3 type cabinet. Install four, 1 inch by 2 inch oblong, punched holes shall be installed on the bottom of the adapter that also match the anchor bolt holes of the ITS Housing #3 type cabinet.

### Structural Requirements

Construct the base adapter so that it does not sag under the weight of the fully loaded cabinet. Any internal members shall not obstruct cables going from the cabinet to the conduits below. Continuously weld all seams and grind smooth.

## Documentation

Provide one electronic version (PDF format) and two full size 24 inch by 36 inch prints of the cabinet circuit diagram. Produce clear and legible prints from the original drawing. Place both copies of the circuit diagram inside the sliding drawer in a readily accessible waterproof enclosure.

## Testing

The Contractor shall arrange for and provide all necessary field tests to demonstrate the traffic signal installation is in proper working order and in accordance with the applicable requirements of the Contract. The Contractor shall supply all tests and test equipment unless specifically noted herein. Final Acceptance of the project requires successful completion of all tests for each intersection.

Test controllers and traffic signal equipment cabinets before installation. Assemble and configure each controller and traffic signal equipment cabinet according to the proposed signal design. Test each controller and traffic signal equipment cabinet for proper color sequence, flashing operation, phase timings, coordination and signal monitor or malfunction management unit programming. Ensure that simultaneous conflicting phase outputs will cause the traffic signal equipment cabinet to revert to flashing operation. Test the traffic signal equipment cabinet and controller for at least eight hours. The test shall take place on a weekday during normal Department business hours at a location approved by the Engineer. The Contractor shall notify the Engineer at least 5 business days before commencing controller testing. The Contractor shall submit written documentation to the Engineer that the controller has successfully completed the Controller Testing.

## Warranty

Provide a manufacturer’s warranty, in writing, for each traffic signal equipment cabinet and all cabinet components. The manufacturer’s warranties shall be fully transferable from the Contractor to the Department. The Contractor shall ensure that these warranties require the manufacturer to furnish replacements for any part or equipment found to be defective during the warranty period at no cost to the Department within 10 calendar days of notification by the Department.

Warranty periods shall begin on the date of final acceptance by the Department and remain in effect for no less than 3 years.

# CONFIGURATIONS

## ATC Cabinet Type I - Configuration

Load the ATC Cabinet Type I with the following components:

### General:

CMUip Data Key Programming Tool

One Shelf

Standard Base Adapter

### Service Assembly:

One Cabinet Surge Protection Device

One High Density Flasher Unit (HV)

### Input Assembly - 24-Channel (Two Units):

Two Serial Interface Units

Three DC Isolators

Two AC Isolators

Two Opto Input Cards

### Field Input Termination Assembly - 24-Channel (Two Units):

Twelve Detection Module Suppressors

### Output Assembly - 16-Channel Option:

One Serial Interface Unit

Eight High Density Switch Pack (HV)

One Cabinet Monitor Unit (HV)

### Output Assembly - 32-Channel Option:

Two Serial Interface Unit

Sixteen High Density Switch Pack (HV)

One Cabinet Monitor Unit (HV)

### Field Output Termination Assembly - 16-Channel Option:

Eight High Density Flash Transfer Relays

Eight High Density Switch Pack Transient Protectors

Sixteen Red Flash Program Blocks

Four Yellow Flash Program Blocks

Four White Flash Program Blocks

### Field Output Termination Assembly - 32-Channel Option:

Sixteen High Density Flash Transfer Relays

Sixteen High Density Switch Pack Transient Protectors

Thirty-Two Red Flash Program Blocks

Eight Yellow Flash Program Blocks

Eight White Flash Program Blocks

## ATC Cabinet Type II - Configuration

Load the ATC Cabinet Type I with the following components:

### General:

CMUip Data Key Programming Tool

Four Shelves

Oversized Base Adapter

### Service Assembly:

One Cabinet Surge Protection Device

One High Density Flasher Unit (HV)

### Input Assembly - 24-Channel (Two Units):

Two Serial Interface Units

Three DC Isolators

Two AC Isolators

Two Opto Input Cards

### Field Input Termination Assembly - 24-Channel (Two Units):

Twelve Detection Module Suppressors

### Output Assembly - 16-Channel Option:

One Serial Interface Unit

Eight High Density Switch Pack (HV)

One Cabinet Monitor Unit (HV)

### Output Assembly - 32-Channel Option:

Two Serial Interface Unit

Sixteen High Density Switch Pack (HV)

One Cabinet Monitor Unit (HV)

### Field Output Termination Assembly - 16-Channel Option:

Eight High Density Flash Transfer Relays

Eight High Density Switch Pack Transient Protectors

Sixteen Red Flash Program Blocks

Four Yellow Flash Program Blocks

Four White Flash Program Blocks

### Field Output Termination Assembly - 32-Channel Option:

Sixteen High Density Flash Transfer Relays

Sixteen High Density Switch Pack Transient Protectors

Thirty-Two Red Flash Program Blocks

Eight Yellow Flash Program Blocks

Eight White Flash Program Blocks

# PROCEDURES

## Installing Ground-Mounted Traffic Signal Cabinets

Install ground mounted traffic signal equipment cabinets on a concrete foundation in conformance with Section 700 of these Specifications and the Standard Drawings. Foundations will be measured and paid for separately.

## Installing the Base Adapter

Prior to bolting the base adapter to the foundation, apply silicone sealant to the mating surface of the adapter to prevent water from seeping between the adapter and foundation. Likewise, prior to bolting the cabinet to the base adapter, apply silicone sealant to the mating surface of the cabinet to prevent water entry. Ensure that the cabinet is plumb, using shims if necessary, and ensure that it is properly aligned with the front edge of the base adapter.

# MEASUREMENT AND PAYMENT

ATC Cabinet (Type)(Outputs 16 or 32) will be measured in units of each and will be paid for at the Contract each price for the Type and number of outputs specified. This price shall include furnishing, installing, and testing of all equipment and materials; shop drawings, documentation, testing, warranty, and incidentals necessary to complete the work.

Auxiliary communications cabinet will be measured in units of each and will be paid for at the Contract each price. This price shall include furnishing, installing, and testing.

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| ATC Cabinet (Type) (No. of outputs) | Each |
| Auxiliary Communications Cabinet | Each |