GUIDELINES — For projects where signal heads or signal head backplates are being replaced or installed at existing signal locations.

SP703-DB0120-00

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

SIGNAL HEAD MODULE AND SIGNAL HEAD BACKPLATE REPLACEMENT

August 20, 2019

- DESCRIPTION

This work shall consist of replacing LED signal head modules, replacing backplates on existing signal heads, and installing backplates furnished by the Department on existing signal heads as shown on the Plans, in accordance with these Specifications and as directed by the Engineer.

- MATERIALS

Signal head sections shall be in accordance with Sections 238 and 703 of the Specifications.

High Visibility Signal Backplates (HVSBs) shall conform to Sections 238 and 703 of the Specifications. Retroreflective fluorescent yellow sheeting shall be in accordance with Section 247 of the Specifications.

Existing plastic or aluminum composite backplates shall be replaced with aluminum composite HVSBs. Existing aluminum backplates may be replaced with aluminum or aluminum composite HVSBs, unless otherwise shown on the Plans.

When no backplate exists on an existing operational traffic signal head, the use and type of backplates shall be in accordance with the Contract unless otherwise directed by the Engineer.

- REPLACE OR INSTALL HVSB BACKPLATE

When replacing backplates on existing traffic signal heads in operation with new HVSB backplates, the new backplate shall be neatly cut vertically near one of the lower corners to allow installation without disassembling the existing traffic signal head from the hanger equipment or disconnecting any existing conductor cables from the traffic signal head. The cut shall be a minimum of 1 inch from either traffic signal head attachment and be nearly inconspicuous and smooth without any gaps in the front surface of the backplate.

If the backplate is precut by the manufacturer, the cut shall be reconnected in accordance with the manufacturer’s instructions. If the backplate is not precut, the reconnection panel shall be connected with 4 rivets to the backplate or as approved by the Engineer. The reconnection panel shall be a minimum 4 inches x 4 inches and shall be of the same material and color as the backplate it is connecting.

The Design-Builder shall visually assess the traffic signal heads and hanger assembly surfaces and connections before performing any modification. If during this visual assessment, the Design-Builder observes a damaged hanger assembly or other signal head equipment that needs to be repaired, replaced, or in the opinion of the Design-Builder poses risk to the Department or travelling public, the Design-Builder shall notify the Engineer. The Design-Builder shall not make any modification to the existing signal head, backplate, or hanger assembly at that location unless authorized by the Engineer.

The Design-Builder shall remove the existing backplates and associated hardware from the traffic signal head and install the new HVSB in a single work day, reconnecting the cut area if a cut has been made. Removing and installing backplates shall be performed without disconnecting the traffic signal head mounting hardware and conductor cables. Any signal head sections, backplates, or components damaged or lost at the fault of the Design-Builder shall be repaired or replaced at no additional cost to the Department.

The Design-Builder shall perform the work such that each intersection approach shall have consistent backplates on all traffic signal heads (either with the fluorescent yellow border or without) at the end of each work week unless otherwise directed by the Engineer.