SQ302-DB0100-00

VIRGINIA DEPARTMENT OF TRANSPORTATION

SPECIAL PROVISION FOR

PIPE REPLACEMENT

December 17, 2018

I.	DESCRIPTION

This work shall consist of the replacement of existing storm water, surface water pipe culverts and/or sanitary sewer lines by the method or methods specified at the designated locations described in the Contract using various standard and non-standard methodologies/technologies according to this provision and as directed by the Engineer.

II.	MATERIALS

Cement grout shall conform to Section 218 of the Specifications except the grout shall be a non-shrink mix design.

Pipe used in replacement operations shall conform to Section 232 of the Specifications.

III.	GENERAL PROCEDURES

All work shall be contained within the existing right-of-way. In the event a temporary construction easement is deemed necessary due to the selected method or product chosen by the Design-Builder, the Design-Builder shall obtain such additional easement for his convenience at no additional cost to the Department.

The Design-Builder shall submit a plan, sequenced with his plan of operations and Maintenance of Traffic Plan, to the Engineer for his review prior to commencing work. The plan shall be designed and implemented according to the Virginia Work Area Protection Manual, the MUTCD and the project-specific temporary traffic control plan or requirements provided in the Contract.When temporary detours are required the Design-Builder shall design and construct temporary detours according to Sections 105.14 and 512 of the Specifications and Standard GS-10 of the Road and Bridge Standards respectively.

The Design-Builder shall contact Miss Utility prior to commencing any work that may conflict with existing utilities, and shall coordinate with the utility company(ies) and the Engineer for any adjustments deemed necessary to complete the work. The Design-Builder shall notify the Engineer at least 48 hours prior to initiating replacement operations at the site.

In the event the selected method of replacement requires disturbing existing surfaces, these surfaces shall be restored in kind to original pre-construction conditions after replacement operations have been completed.

The Design-Builder shall obtain all required OSHA confined space entry permits where these are required by the Design-Builder’s operations and the scope of work in the Contract.

The Department has performed an inspection of the existing pipes where so identified in the Contract to determine the extent and nature of each pipe’s deterioration or damage in each location so as to designate the allowable methods of replacement of the pipe at that specific site. Using the information collected from this inspection the Department has determined through its engineering analysis the best practical method(s) of replacement for each respective site (location) taking into account site specific conditions such as installation working space, hydraulic capacity before and after replacement, height of cover, soil density, and loading conditions, among other criteria. The Design-Builder shall select, according to the method or options of methods designated by the Department for each specified location or site indicated in the Contract, how he/she chooses to perform the work. Regardless of the final method selected, the Design-Builder shall provide the Engineer documentation of the procedures, materials, equipment, incidentals and resources he/she shall employ to ensure successful replacement of the existing culvert(s) and/or sewers to assist the Quality Assurance Manager in monitoring the Design-Builder’s operations. The Design-Builder will not be permitted to substitute a different method(s), if designated, than that or those specified for the location described in the Contract.

Replacement procedures shall be performed according to the method specified according to Section 302.03 of the Specifications at the location shown or described in the Contract. Pipes shall be installed to the line and grade shown or derived from invert elevations specified in the plans. Unless otherwise specified, pipes abandoned in place shall be grouted to fill the entire inside void with flowable backfill conforming Section 509 of the Specifications.

The following methods of pipe replacement are approved by the Department; however, not all methods may be appropriate for each individual location. The Design-Builder shall consult the Contract to determine the method or methods that are permitted per the location cited. Individual methods shall conform to the criteria specified.

Method A – Jacked pipe installation shall be the jack and bore method according to Section 302.03(a)1 of the Specifications.

Method B - Open trench pipe installation shall be the open trench method according to Section 302.03(a)2 of the Specifications.

Post installation inspection shall be performed according to Section 302.03(d) of the Specifications.