SS305-002020-01	June 2, 2020

VIRGINIA DEPARTMENT OF TRANSPORTATION

2020 ROAD AND BRIDGE SUPPLEMENTAL SPECIFICATIONS

SECTION 305 – SUBGRADE AND SHOULDERS

SECTION 305 – SUBGRADE AND SHOULDERS of the Specifications is amended as follows:

Section 305.02 – Materials is replaced with the following:

(a)	Materials may consist of material in place, treated material in place, or imported material. Imported material may be borrow material, select material, or other material as shown on the plans or specified in the Contract.

Materials other than regular excavation or borrow material that are shown on the Plans or specified in the Contract shall conform to the applicable requirements of these Specifications.

(b)	Geotextile materials used for subgrade stabilization or separation shall conform to Section 245.03(d).

Section 305.03(d) – Geotextile (Subgrade Stabilization) is renamed Geosynthetics and replaced with the following:

Geosynthetics includes Geotextile used for subgrade separation or stabilization, and geogrid.

  - Subsurface preparation: Before placing the geotextile, geogrid, or combination of both, prepare the subgrade in accordance with Sections 304 and 305. Separation and stabilization geotextiles shall not be placed when weather conditions, in the judgement of the Engineer, are not suitable to allow placement of geotextiles or cover materials. These include wet or snowy conditions, rainfall, temperatures below freezing, frost, or excessively windy conditions.

  - Geotextile or geogrid placement. Place geogrid on top of geotextile when both are shown at the same elevation in the Plans. Place the geosynthetic in the direction of traffic. Geosynthetic shall be smooth and free of wrinkles and folds. Placement by dragging the geosynthetic across the finished surface will not be allowed. On curves, the geotextile may be folded or cut to conform to the curve. The fold or overlap shall be in the direction of traffic and held in place by pins, staples or piles of aggregate subbase or base materials. Overlap in the direction of construction. Overlap at least 24 inches at the ends and sides of adjoining sheets or sew the joints according to the Manufacturer’s recommendations. Do not place longitudinal overlaps below anticipated wheel loads or joints. Hold the geosynthetic in place with pins, staples, or piles of aggregate subbase or base materials.

Replace or repair geosynthetic that is torn or punctured. Remove the damaged area and place a patch of the same type of geosynthetic overlapping 36 inches beyond the damaged area or sew a seam around the entire perimeter of the damaged area.

  - Initial layer placement and compaction: Place initial layer in accordance with Sections 308 and 309.

If during placement of the geosynthetic, the equipment causes subgrade rutting in excess of 2 inches, end dump the backfill material onto the geotextile or geogrid from the edge of the geosynthetic or from previously placed cover material. Do not operate equipment directly on the geosynthetic. Spread the end-dumped pile of cover material maintaining the minimum specified lift thickness over the geosynthetic. Avoid sudden stops, starts, or turns of the construction equipment. Fill ruts from construction equipment with additional cover material. Do not blade material down to remove ruts. If rutting continues to exceed 2 inches during placement, decrease the construction equipment size, decrease the equipment weight, or increase the first lift thickness as directed by the Engineer.

Compact in accordance with Sections 308 and 309. Do not use sheepsfoot or studded compaction equipment. Compact the cover material with pneumatic-tire or non-vibratory smooth drum rollers.

Tracked equipment shall not be operated directly on top of geosynthetic. Rubber-tire equipment may pass over the geosynthetic if the geosynthetic is not damaged by the equipment (causing excessive rutting, 2 inches or greater); the Contractor shall specifically avoid sudden braking or sharp turning, and shall maintain low speed.

  - Subsequent layer placement and compaction. Place and compact subsequent layers in accordance with Sections 308 and 309.

Section 305.04 – Measurement and Payment is amended by replacing the tenth paragraph with the following:

Geotextile will be measured in square yards, complete-in-place. Overlaps and seams will not be measured for separate payment. The accepted quantity of geotextile will be paid for at the contract square yard price. This price shall include furnishing, placing, lapping, and seaming material.

Section 305.04 – Measurement and Payment is amended by revising the Pay Item Table as follows:

The following pay items are removed:

| Pay Item | Pay Unit |
| --- | --- |
| Geotextile (Subgrade stabilization) | Square yard |

The following pay items are inserted:

| Pay Item | Pay Unit |
| --- | --- |
| Geotextile (type) | Square yard |