SS401-002020-01	May 1, 2020

VIRGINIA DEPARTMENT OF TRANSPORTATION

2020 ROAD AND BRIDGE SUPPLEMENTAL SPECIFICATIONS

SECTION 401—STRUCTURE EXCAVATION

SECTION 401 – STRUCTURE EXCAVATION of the Specifications is amended as follows:

Section 401.03(i) – Backfilling is amended by replacing the second and third paragraphs with the following:

The Contractor shall use select backfill material behind all abutments. The Department will include a detail indicating the limits (zone) of the select backfill in the Plans. The Contractor shall compact the material in accordance with Sections 305 and 303. The top surface of the backfill material shall be neatly graded.

The earthen fill around the perimeter of the select material zone in abutments, wingwalls, and retaining walls shall be placed in horizontal layers not more than 6 inches in loose thickness and then compacted at ±20% of optimum moisture content to a density of at least 95% as compared to the Theoretical Maximum Density. The Department will perform tests in accordance with VTM 10 to verify compliance with density requirements determined in accordance with VTM 1 or VTM 12. The Contractor shall place and compact the backfill in front of units in horizontal layers to the same elevation as the layers behind units until the final elevation in front is reached as the work progresses. Backfill in front of units shall be placed and compacted in horizontal layers to the same elevation as the layers behind units until the final elevation in front is reached. Backfill shall be placed in a manner to prevent wedging action against the concrete. The Contractor shall modify slopes bounding excavation for abutments, wingwalls, or retaining walls to lock in adjacent backfill material by stepping or serrating the existing soils. The Engineer will not permit jetting of the fill behind abutments, wingwalls, or retaining walls.