SS520-002020-01

VIRGINIA DEPARTMENT OF TRANSPORTATION

2020 ROAD AND BRIDGE SUPPLEMENTAL SPECIFICATIONS

SECTION 520 – WATER AND SANITARY SEWER FACILITIES

April 14, 2020

Section 520 – Water and Sanitary Sewer Facilities is amended as follows:

Section 520.02(r) – Flowable backfill is inserted as follows:

Flowable backfill shall conform to Section 249 and 509.

Section 520.03 Procedures is amended to replace the first paragraph with the following:

The Contractor shall be responsible for anticipating and locating underground utilities and obstructions in accordance with Section 105.08. In areas where the utility owner’s specifications conflict with the Contract, the utility owners’ specifications shall govern

Section 520.03 – Procedures is amended to replace the fourth paragraph with the following:

The Contractor shall abandon existing water and sewer lines and appurtenances and manholes not required in the completed system as directed by the Engineer. Abandoned materials shall become the property of the Contractor, unless otherwise noted on the plans, upon satisfactory replacement with the new installation. The Contractor shall clean abandoned pipe that is not removed of debris and plug it with Class A3 concrete at open ends if the utility is less than 8 inches inside diameter. If the abandoned pipe is 8 inches inside diameter or greater, the Contractor shall clean the pipe of debris and fill it entirely with Class A3 concrete or flowable backfill.

Section 520.06 – Measurement and Payment is amended by inserting the following after the fifteenth paragraph:

Concrete will be measured in cubic yards and will be paid for at the Contract cubic yard price. This price shall include furnishing and placing of concrete not included in other pay items, and installing plugs.

Flowable Backfill will be measured and paid for in accordance with Section 509.

Section 520.06 – Measurement and Payment is amended by revising the Pay Item Table as follows:

The following pay item is removed:

| Pay Item | Pay Unit |
| --- | --- |
| Flowable Backfill | Cubic Yard |