SS701-002020-01	October 20, 2020

VIRGINIA DEPARTMENT OF TRANSPORTATION

2020 ROAD AND BRIDGE SUPPLEMENTAL SPECIFICATIONS

SECTION 701 – TRAFFIC SIGNS

SECTION 701 – TRAFFIC SIGNS of the Specifications is amended as follows:

Section 701.02 Materials is amended as follows:

Section 701.02(a)1 – Overhead Permanent Signs is replaced with the following:

Overhead Permanent Signs (signs attached to sign structures which overhang travel lanes) shall use ASTM D4956 Type XI sheeting.

Table VII-1A – Reflective Sheeting is replaced with the following:

| TABLE VII-1A Reflective Sheeting |  |
| --- | --- |
| Background Color of Sign | Sheeting Type1 |
| White | ASTM D4956 Type XI |
| Red | ASTM D4956 Type XI |
| Fluorescent Yellow | ASTM D4956 Type XI2 |
| Fluorescent Yellow-Green | ASTM D4956 Type XI3 |
| Green | ASTM D4956 Type XI |
| Black | ASTM D4956 Type XI |
| Purple | ASTM D4956 Type XI |
| Brown | ASTM D4956 Types IV, IX, or XI |
| Blue | ASTM D4956 Types IV, IX, or XI |
| 1The following signs may use ASTM D4956 Types IV, IX, or XI, regardless of color: Pushbutton education signs (R10-series signs mounted adjacent to pedestrian pushbuttons), Signs erected on bikeways physically separated from adjacent roads, R7- or R8-series parking restriction signs located on non-limited-access highways, D10-series Reference Location Signs (mile markers) and Intermediate Reference Location Signs, and Post-mounted street name signs. 2The yellow portions of all signs shall be fluorescent yellow. 3All temporary and permanent warning signs related to school zones, pedestrians, or bicyclists (including associated supplemental plaques) shall use fluorescent yellow-green sheeting where required by the VA Supplement to the MUTCD unless otherwise specified on the Plans. |  |

Section 701.02(b) – Sign panel rivets is replaced with the following:

Sign panel rivets shall be fabricated in accordance with Standard Drawing SPD-1.

Section 701.03 Procedures is amended as follows:

Section 701.03 (a) 3 Letters, numerals, arrows, symbols, borders, and other features of the sign message is amended to include the following:

- Type L5 - Digital Printing: Features of the sign message shall be created with transparent and opaque durable inks, using the materials, equipment, and fabrication processes recommended by the sheeting manufacturer. Digitally applied colors shall present a smooth surface, free from foreign material, and all messages and borders shall be clear and sharp. Digitally printed signs shall have at least 70% of the retroreflective minimum values shown in Section 247for their type and color. Digitally printed signs shall meet the daytime color, nighttime color, and luminance, requirements of ASTM D4956. No variations in color or overlapping of colors will be allowed.

All digitally printed signs shall be comprised of an integrated engineered match component system from the sheeting manufacturer. The integrated engineered match component system shall consist of retroreflective sheeting, durable inks, and clear overlay film all from the same manufacturer applied to the substrate.

The digital sign fabricator shall be certified within the last 12 months by the sheeting manufacturer to produce an engineered-match component traffic control sign product. The Contractor shall  provide documentation of this certification before starting digital printing sign fabrication. The submittal shall also include documentation that the manufacturer will warrant digitally printed signs fabricated by the sign fabricator in accordance with Section 247.03.

After printing, all digitally printed permanent and temporary traffic signs shall have an integrated engineered match component clear protective overlay recommended by the sheeting manufacturer applied to the entire face of the sign.

Section 701.03(f)2 – Inventory Sheet is replaced with the following:

Inventory Sheet. The Contractor shall provide an .xlsx formatted file to the Engineer, using a sign inventory template provided by the Engineer. The file shall include the information required above for the label, as well as the following:

    - Route no.

    - Project UPC no. (if applicable).

    - Station or milepost information.

    - Lane designation.

    - MUTCD, if applicable and if denoted on the plans.

    - Sign message.

    - Sign width.

    - Sign height.

    - Sheeting Manufacturer

    - Sheeting Type

    - Sign fabricator company name

The cost of preparing and submitting the .xlsx formatted file shall be included with the cost of the sign panel pay items.