GUIDELINES — For projects requiring exposed aggregate finish for soundwalls, curb cut ramps, curbs and/or parapets. {2007-c504c00}

cn404-DB0100-00	EXPOSED AGGREGATE FINISH shall be performed by wirebrushing, blasting or surface retarder unless another method is approved by the Quality Assurance Manager.

Concrete for exposed aggregate finish shall conform to Section 217 of the Specifications for the class specified, except gravel shall be tan or light brown in color.

The Design-Builder shall provide a sample of the exposed aggregate finish for approval by the Engineer prior to beginning work.  The sample shall be at least 12 inches by 12 inches and approximately 2 inches in depth.  The approved sample shall be kept at the work site for comparison to completed work.

11-14-18 (SPCN)