IMPACT ATTENUATOR — This work shall consist of replacing damaged impact attenuators, or installing new, by furnishing and installing impact attenuators in accordance with Section 505 of the Specifications and as directed by the Engineer. Replacement impact attenuators shall be a Type I (Re-Directive Low-Maintenance) as designated on VDOT’s MASH Provisionally Approved Products List. Impact attenuators shall be installed in accordance with the manufacturer's specifications and instructions, Standard Drawing IA-LM, and Section 505 of the Specifications.

If replacing an existing impact attenuator, the Design-Builder shall inspect the condition of the existing impact attenuator foundation pad for size, thickness, adequacy of reinforcement, slope, cracking, surface wear, shifting, undermining, settling, or other signs of age or deterioration which may be unsuitable for continued use. If any of these conditions are present, the Design-Builder shall replace the existing foundation pad.

If the existing foundation pad is suitable for continued use and existing bolt patterns are present, the Design-Builder shall confirm that installing new bolts will not negatively impact performance of the unit.

Anchor bolts shall be set into holes drilled with rotary impact drills of the sizes recommended by the manufacturer of the attenuator, and as approved by the Engineer. Bolt holes shall be cleaned with a bottle brush before installing the anchor bolts.

The Type I (Re-Directive Low-Maintenance) Impact Attenuator shall be bolted in place according to the attenuator manufacturer’s instructions, but not before the fresh concrete foundation, if applicable, has reached 28-day strength, determined in accordance with Section 217 of the Specifications.

If a transition is required, the appropriate manufacturer’s standard transition shall be used.

The work site shall be protected according to Section 512 of the Specifications and the VWAPM.

Installers and repair personnel shall be certified by the manufacturer. The Design-Builder shall submit two copies of the manufacturers’ installation instructions to the Engineer prior to beginning work.

The Design-Builder shall provide one training session to VDOT personnel at a location selected by the Engineer within the VDOT District where the impact attenuators were installed. The training sessions shall be conducted by a manufacturer’s representative and cover impact attenuator maintenance and repair. The session shall be structured such that the personnel trained will be able to perform maintenance and repair beginning within one year of the training.

The Design-Builder shall provide documentation tracing the steel in the finished product back to raw material in accordance with Buy America requirements.

7-26-19 (SPCN)