Guidelines: For use on all projects that will require establishment of a work zone on a concrete wearing surface.

cn512-DB0130-00	TEMPORARY CONTRAST MARKINGS – Type D, Class III contrast pavement markings shall be used for all longitudinal temporary pavement markings on bridge decks and hydraulic cement concrete riding surfaces if all of the following are met:

- The road has a speed limit of 45 MPH or greater.

- The hydraulic cement concrete riding surface in question is at least 300 feet in length.

- The temporary markings are planned for at least 15 days of use.

Type D, Class III contrast markings are not required for any markings that are parallel to and within two feet of existing guardrail or other longitudinal barrier.

2-26-19 (SPCN)