cq312-000110-00

GUIDELINES – For Richmond district projects with emulsified asphalt slurry seal. {2007-cu312000a}

VACUUMING EMULSIFIED ASPHALT SLURRY SEALS

The Special Provision for EMULSIFIED ASPHALT SLURRY SEAL (SP312-000100-00) is amended as follows:

Section IV. C.  Application is amended to include the following as the third paragraph:

All routes receiving an Emulsified Asphalt Slurry Seal or a Cape Seal (Modified Single Seal or Modified Double Seal with Slurry Seal) shall be lightly broomed to remove loose aggregate prior to opening the pavement surface to traffic.  All excess aggregate material on the pavement surface travel lane(s) and accumulated along curb and gutter sections, the edge of pavement/lawns, and hard-surfaced driveway entrances shall be removed by mobile vacuum unit after either final or interim riding surface has been opened to traffic as directed by the Engineer up to three weeks after pavement surface has been opened to traffic.

9-4-14; Reissued 7-12-16_(SPCN)