cq510-000100-01

Guidelines: Asphalt resurfacing (2020 latex only) projects only when there is no separate pay item for such work.

LOCATING, REMOVING AND DISPOSING OF RECESSED PAVEMENT MARKERS AND RAISED SNOW-PLOWABLE MARKERS —  The Contractor shall locate, remove and dispose of existing recessed pavement markers and raised snow-plowable markers prior to resurfacing.  The cavity left by the removal of the existing recessed pavement markers shall be cleaned of debris, filled with an approved mix design for resurfacing or material found on the Department’s Approved List 78, and compacted.  Locating, removing and disposing of recessed pavement markers and raised snow-plowable markers; cleaning and filling the cavity, and compacting the material placed in the cleaned cavity will not be measured for payment.  The cost for performing this work shall be included in the price bid for other appropriate items of work.

12-5-19 (SPCN)