Guidelines: For use on all projects requiring shifting of travel lanes within the work zone.

REMOVAL OF SNOW-PLOWABLE RAISED PAVEMENT MARKERS DURING TRAVEL LANE SHIFTS – The Contractor shall locate, remove, and dispose of all existing asphalt-embedded Snowplowable Raised Pavement Marker (SRPM) castings which lie within the travel path when traffic lanes are shifted during construction for three months or longer. The cavity left by the removal of the existing marker shall be cleaned of debris, filled with an approved mix design for resurfacing or material found on the Department’s Approved List 78, and compacted before shifting traffic. The cost for performing this work shall be included in the price bid for other appropriate items of work.

3-3-20 (SPCN)